Code No:220553

OR

II-B.Tech. II-Semester Examinations, April/May, 2003

SWITCHING AND LOGIC THEORY THROUGH DIGITAL IC’S

(Common to Computer Science and Engineering and Computer Science and Information Technology)

Time: 3 hours							        Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Draw the circuit of ECL NOR gate and explain its operation.

b)	Compare the performance of various digital logic families.

2.a)	Simplify the following Boolean expressions and implement by using only NAND gates.

Y=A(A+B)(AB+B)

Z=(A+B)(A+C)(B+C)

b)	Explain about Quirie McCluskey’s Tabular method of minimization with an example.

3.a)	Draw the circuit of four bit R-ZR Ladder type digital to analog converter and obtain the expression for its analog output voltage.

b)	Implement the following switching function using a Four input multiplexer

f(A, B, C, D) =  ( 0, 1, 2, 4, 6, 9, 10, 13, 14)

4.a)	What is Racing condition in JK Flip flop? Explain how this problem can be overcome by the Master slave JF Flip Flop with the help of circuit diagram.

b)	Explain the operation of the Johnson counter. Modify this circuit for self starting.

5.a)	List the advantages of synchronous counters over ripple counters.

b)	Using a shift register and a combinational logic circuit, design a sequence generator which will generate the binary sequence:   01001011101

6.	Write short notes on any Three:

- SOP and POS forms.

- PLA and PAL.

- RAM, ROM, EPROM and E2PROM.

- Multiplexer and decoder.

7.	Write short note on any Three:

- Analog to digital converter.

- MOS and CMOS logic families.

- Melay and Moore machines.

- Design of Modulo counter.

(Contd…2)

Code No:220553				-2-			OR

8.a)	Define state diagram. Explain the procedure to reduce the state diagram.

b)	For the state diagram shown, obtain the state table and design the circuit using minimum number of JK Flip Flops.

***     ***     ***