Code No.:222101

Set No.

1

II-B.Tech. Regular II Semester Examinations April/May, 2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours.							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Why are viscosities called as dynamic and kinematic?

b)	Calculate the capillary rise in a 2mm diameter glass tube when immersed partially

a) in water	ii)	Glycerin	iii)	Mercury. The surface tension for water, glycerin and mercury is 0.075 N/m, 0.065 N/m and 0.52 N/m respectively. The specific gravity of glycerin is 1.6 and that of mercury is 13.6.

2.a)	How is the pressure related to the shape of the container?

b)	Show that for a plane surface immersed in a liquid vertically, the centre of pressure is always below the centre of gravity.

3.a)	Define and explain the concept of one, two and three dimensional flow.

b)	Potential function for a fluid flow is

=xy33+x2−x3y3−y2+6

Determine the velocity and acceleration at a point (1,2).

4.a)	Why is the streamlining of automobiles of less practical importance than that of subsonic airplanes?

b)	A thin plate 90cm wide and 2.1m long is dragged lengthwise through water at a velocity of 1.5m/s. If the kinematic viscosity of water is 0.315 stokes, calculate the total drag on the plate.

5.a)	Explain the advantages and disadvantages of each of the following:

i)	Venturimeter	ii)	Nozzle meter	iii)	Orifice meter.

What factors account for a higher coefficient of discharge in a venturimeter as compared to the other two meter?

b)	A 10cm orifice in the side of a 1.8m diameter tank draws the surface down from 2.25 to 1.2m above the orifice in 8.25 sec. Calculate the discharge coefficient.

Contd…2

Code No.: NR-222101			-2-				     Set No.1

6.a)	Derive the following relations stating the assumptions used:

i)	ho = h+1/2 c2		ii)	Po= P+1/2Pc2 		c)	ao2=(r-1)ho

iii)	ToT= 1+γ−12 m2	iv)	TT= 2γ+1+ γ−1γ+1M2

b)	Derive the following relations foe flow through a normal shock:

My2 = 2(x−1)+Mx22γ(γ−1)Mx2−1

7.a)	Derive the following relations for one dimensional isentropic flow:

PP = 2(γ+1)+γ−1γ+1M2γ(γ−1)

- Explain how does a shock wave develop in the diverging section of a supersonic nozzle? How does this wave move towards the exit?

8.	Write short notes on any FOUR of the following:

i)	Steady and unsteady flows.

ii)	Rotameter.

- Shock wave formation.

- Measurement of Turbulence.

- Vortex flows.

^^^

Set No.

2

Code No.:222101

II-B.Tech. Regular II Semester Examinations April/May, 2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours.							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is vapour pressure? What is saturation vapour pressure?

b)	A plate is moving relative to the other with a velocity of 400mm/s. If the distance between the two plates is 10mm and some liquid is filled between the plates, find the velocity gradient.

2.a)	State and prove Pascal’s law.

b)	Derive an expression for the total pressure on a plane surface immersed vertically in a liquid.

3.a)	Derive equation of continuity for three dimensional flow.

b)	Find the equation streamlines for the following case

U = x + y, v = x – y, [x2 -2xy –y2 = const]

4.a)	How does the drag coefficient change with i)Surface roughness ii) turbulence level? Explain why golf balls are designed to have dented surface.

b)	An automobile perfectly streamlined has as projected area of 2.4m2 and travels in air (200C) at a maximum speed of 120km/hr. What is the power required to overcome the air frictional resistance? Take cp = 0.30units kJ/kg K and  for air=1.235 kg/m3.

5.a)	Under what circumstances will an orifice function as a large orifice?

b)	Select the size of orifice that permits a tank of horizontal cross-section 1.5m2 to have the liquid surface drawn at the rate of 18cm/sec for 3m head on the orifice. Take cd=0.63.

6.a)	What is the effect of Mach number on the compressibility? Prove for =1.4

Po−P12Pc2= 1 + 14M2+140M4+−−−

b)	Starting from the energy equation for flow through a normal shock obtain the following relations:

CxCy=a*2

Mx My=1

Contd…2

Code No.: NR-222101			-2-				     Set No. 2

7.a)	Derive the following relations for one dimensional isentropic flow.

AA=1M2γ+1+γ−11+1M2(γ+1)2(γ−1)

b)	The entry conditions of air for a nozzle are P1=1.392 bar, T1=333K, M1=0.40. If the exit pressure is 1.039 bar determine the exit Mach number and the maximum specific mass flow m/A*.

8.	Write short notes on FOUR of the following:

i)	Center of pressure

ii)	Stream function

- Wedge probes

- Mach cone

- Pitot tube

^^^

Code No.:222101

Set No.

3

II-B.Tech. Regular II Semester Examinations April/May, 2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours.							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Why does the saturation vapour pressure of a liquid increase with the increase in temperature?

b)	A liquid of viscosity  is filled between two plates moving relative to each other with a velocity of 100mm/s under the action of a force of 100N. What will be the velocity when the force is doubled?

2.a)	Derive an expression for the intensity of pressure at a point.

b)	Distinguish between total pressure and resultant pressure on a plane surface.

3.a)	Derive Euler’s equation of motion along a streamline.

b)	Find the equation streamlines for the following case

U=x, V=-y,  [xy=const]

4.a)	Comment on the shape of the body front and its rear for the minimum drag in

i)	Subsonic flow	ii)	Supersonic flow?

b)	The sail of a rotor ship consist of a pair of vertical cylinders 3m in diameter and 20m long rotating at a speed of 300r.p.m. Determine the force on cylinders for a wind blowing crosswire at 50km/hr relative to the cylinders. Take specific weight of air as 11.95 N/m3.

5.a)	What is a standard orifice? Under what conditions will an opening made in a thick wall behave as a sharp crested orifice?

b)	An orifice of area 30cm2 located in a vertical plate has a head of 1.1 meter of oil of specific gravity 0.92. It discharges 6023 kg of oil in 74.8 sec. Trajectory measurements yield x=2.15m, y=1.24m. Determine Cc, Cv, and Cd.

6.a)	The conditions of an air stream at entry to a duct are P1=1.0 bar, T1=300K, M1=1.3

If the Mach number at exit of the duct is 0.6 determine for adiabatic flow the temperature and velocity of air at the duct exit.

b)	Write down the four basic equations which satisfy the state points before and after a normal shock wave.

Contd…2

Code No.: NR-222101			-2-				       Set No. 3

7.a)	Derive the following relations for one dimensional isentropic flow:

FρoA	= 2γ+1(γ+1)2(γ−1)1+γM2M1+γ−12M112

b)	Show isentropic and adiabatic expansion and compression processes in nozzles and diffusers on enthalpy-entropy coordinates, Indicate the initial and final values of static and stagnation pressures and temperatures.

8.	Write short notes on any FOUR of the following:

i)	Ideal and Real fluids.

ii)	Free and forced Vortex flows.

- Flow over an air foil.

- Compressibility factor.

v)	Oblique shock waves.

^^^

Code No.:222101

Set No.

4

II-B.Tech. Regular II Semester Examinations April/May, 2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours.							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define dynamic and kinematic viscosities of fluids.

b)	Calculate the capillary rise of water in a soil of particle size 0.05 mm. The pore size may be assumed to be of size one fourth of particle size. The surface tension of water may be taken to be 0.075 N/m.

2.a)	Define the terms absolute pressure, gauge pressure and vacuum pressure.

b)	Explain the graphical method to determine the total pressure on a rectangular surface immersed in a liquid.

3.a)	Differentiate between:

i)	Steady and unsteady flow

ii)	Uniform and non-uniform flow.

- Stream function for a fluid flow is

 = x4+y412+2xy−x2y2+10

Determine the velocity and acceleration at a point (-2, 3).

4.a)	Distinguish between the friction drag, the pressure drag and the profile drag.

b)	Estimate the maximum load which may be carried by a 5m diameter parachute without attaining a velocity of descent of more than 6m/s. Take Cp=1.33 for a hemisphere hollow upstream. The specific gravity of air is 12.15 N/m3.

5.a)	Distinguish between a i) pitot tube ii) pitot-static tube. What is the advantage of using prandtl tube over the other types?

b)	Determine the dimensions of a tank such that the surface velocity varies inversely as the distance from the centerline of an orifice draining the tank. When the head is 0.30m, the velocity of fall of the surface is 2m/minute and the orifice diameter is 1.5cm. Take Cd = 0.65.

6.a)	Define the following terms; write down their formulas and units where applicable.

Stagnation pressure, Stagnation temperature, stagnation velocity of sound, Mach number, critical and maximum velocities, transonic and hypersonic flows.

- Derive the following relations for flow through a normal shock:

PyPx=2γγ+1Mx2−γ−1γ+1

Contd…2

Code No.: NR-222101			-2-				       Set No. 4

7.a)	Derive the following relations for one dimensional isentropic flow:

dAA=dPPc2(1−m2)

b)	The Mach number and pressure at the entry of a subsonic diffuser are 0.9 and 4.165 bar. Determine the area ratio required and the pressure rise if the Mach number at the exit of diffuser is 0.20. Assume isentropic diffusion of air.

8.	Write short notes on any FOUR of the following:

i)	Hydrodynamic boundary layer.

ii)	Newtonian fluids.

- Laplace equation.

- Mach number.

- Flow Visualization Techniques.

^^^