Code No.220403

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

EM WAVES AND TRANSMISSION LINES

(Common to Electronics and Communication Engineering and Electronics and Telematics)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Explain the following terms:

- Homogeneous and isotropic medium and

- Line, surface and volume charge distributions.

b) 	A circular ring of radius ‘a’ carries uniform charge L C/m and is in xy-plane.     Find the Electric Field at Point (0, 0, 2) along its axis.

2.a) 	Define and explain the Biot-Savart’s Law.  Hence obtain the field due to a straight current carrying filamentary conductor of finite length.

b) 	In a  medium of     ε  =  5 εo    ,  µ  =  2.5 µo   ,  σ   = 0.2  m ʊ/m.,   E  =  20 µV/m., find the conduction current density.  If this current density exists in a cylindrical rod of 2 cm. diameter, evaluate the current that can flow through the rod.

3.a)	Define uniform plane waves.  Solve the wave equations for uniform plane waves in a medium of conductivity  and hence establish the relations for propogation constant, attenuation and phase constants in terms of .

b)	Explain the characteristics of the propogating waves in a good conducting medium.

4.a)	What is meant by the polarization of wave?  When is the wave linearly polarized and circularly polarized?

b)	A traveling wave has two linearly polarized components Ex = 2 Cos wt and Ey = 3 Cos (wt + /2)

(i)	What is the axial ratio?

(ii)	What is the tilt angle of the major axis of the polarization ellipse?

- What is the sense of rotation?

5.a)	Explain the significances of Poynting theorem and poynting Vector.  By integrating the Poynting Vector over the cross-section of co-axial cable, show  that the total power carried by the cable is VI, where v is the voltage and I is the current.

b)	Obtain an expression for the Power loss in a Plane conductor in terms of the surface resistances Rs.

(Contd…2)

Code No.220403				-2-				Set No.1

6.a) 	A rectangular wave guide has cross section dimensions   a= 7 cm ,  b= 4 cm

determine all the modes which will propagate at a frequency of (i) 3000MHz

(ii) 5000MHz

b)	With above data, find Vp, Zo and guide wavelength for possible modes.

7.a)	Explain the meaning of the terms characteristic impedance and propagation constant of a uniform transmission line and obtain the expressions for them in terms of parameters of line?

b)	A telephone wire of 20 km long has the following constants per loop km resistance 90 , capacitance 0.062 F, Inductance 0.001H and leakage                   = 1.5 x 10-6mhos. The line is terminated in its characteristic impedance and a potential difference of 2.1 V having a frequency of 1000 Hz is applied at the sending end.  Calculate:

(i)	The characteristic impedance.

(ii)	Wavelength.

(iii)	The velocity of propagation.

8.a)	Explain the principle of Impedance matching with Quarter wave Transformer?

b)	A 100  lossless line connects a signal of 100 KHz to a load of 140 .  The load power is 100 mW.  Calculate

(i) Voltage Reflection coefficient, (ii) VSWR, (iii) Position of VMax, IMax Vmin  and Imin.

***     ***     ***

Code No.220403

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

EM WAVES AND TRANSMISSION LINES

(Common to Electronics and Communication Engineering and Electronics and Telematics)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Define and distinguish between the terms Electric field, Electric displacement and Electric flux density with necessary mathematical relations.

b) 	A point charge of 300C is located at point (1, 2, 1)m.  Find the force on another point charge of 30C at the point (3, 0, 0)m and also calculate the Electric field at      (3, 0, 0)m.

2.a)	Using the relation for H  of a finite straight wire, obtain the expressions for the fields due to a semi infinite wire and infinite wire located on z-axis, carrying a current  I.

b) 	In a conducting medium   H   =   y2   z   X +  2 ( x + 1) y z   Y – ( x + 1) z2  Z  A/m. Find the current density at (1,0,-3)  and  calculate the current  passing through  Y  = 1 Plane, 0 ≤ x ≤ 1,  0 ≤ z ≤ 1.

3.a)	In free space D = Dm Sin (Wt +Z)ax.  Determine B and displacement current density.

b)	Region 1, for which r1 = 3 is defined by x <	 0 and region 2, x < 0 has r2 = 5 given H1 = 4 ax + 3ay – 6 az (A/m).  Determine H2 for x> 0 and the angles that H1 and H2 make with the interface.

4.a)	Explain the terms “Linear Polarization”, “Elliptical polarization” and “Circular polarization”

b)	In a Medium Ē = 16 e-x/20 Sin (2.108t –2x) ZV/m.  Find the direction of propogation, the propogation constant, wavelength, speed of the wave and skin depth.

5. 	Starting from Maxwell’s equations, derive the expressions for the E and H field components for TE waves in a parallel plane wave guide.

(Contd…2)

Code No.220403				-2-				Set No.2

6. 	A lossless air dielectric wave guide for an S-band radar has inside dimensions  a=7.214cm and  b=3.404cm. For the TM11 mode propagating at an operating frequency that is  1.1 times the cutoff frequency of the mode , calculate

  - Critical wave number  (b) cutoff frequency  	 (c) operating frequency

(d)  Cutoff wave length     (e) operating wave length (f) guide wave length

(g)  Phase velocity 	       (h) wave impedance 	 (i) group velocity.

7.a)	Starting from the equivalent circuit, derive  the transmission line equations for V and I, in terms of the source parameters.

b)	Give the T and  equivalent network representations for a transmission Line.

8.a)	Explain clearly why the short circuited stubs are preferred over to a open circuited stubs?

b)	Derive the expression for the input impedance of a loss-less line Hence evaluate Zsc and Zoc and sketch their variation with line length.

***     ***     ***

Code No.220403

Set No.

3

II-B.Tech. II-Semester Examination April/May 2003

EM WAVES AND TRANSMISSION LINES

(Common to Electronics and Communication Engineering and Electronics and Telematics)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	State the Coulomb’s law in SI units and indicate the parameters used in the equations with the aid of a diagram.						4

b) 	Point charges Q1 and Q2 are respectively located at (4, 0, -3) and (2, 0, 1).  If Q2 = 4 nC, find Q1 such that.

- The E at (5, 0, 6) has no Z-component.

The force on a test charge at (5, 0, 6) has no X-component.

2.a) 	Obtain  expressions for the energy density and energy stored in a static magnetic field.

b) 	Find  H   and   B  at  (0, 0, 5)  due to side  OA  and   side OB  of the triangular current loop shown in Fig. 2.

Y

B

1                               10mA

2

0    		1		      x

A

3.a)	Define uniform plane waves. Solve the wave equations for uniform plane waves in a medium of conductivity  and hence establish the relations for propagation constant, attenuation and phase constants in terms of .

b)	Explain the characteristics of the propagating waves in a good conducting medium.

4.a)	A uniform plane wave is normally incident from air on a perfect conductor.  Determine the resulting E and H fields.  Sketch their variations.

b)	An EM wave is propagated through a material having r= 5 and r = 10.  Determine the

- Velocity of propagation.

- Interinsic impedance of free space and of materials

- Wavelength in free space and in material, if the frequency is 1 GHZ.

5. 	Derive the expressions for the E and H field components for TM waves in a Parallel plane wave guide, using Maxwell’s equations approach.

(Contd…2)

Code No.220403				-2-				Set No.3

6. 	An air dielectric L-band rectangular wave guide has  a/b  = 2  and a dominant mode cutoff frequency of 0.908GHz. If the measured guide wave length is  40cm, find the operating frequency , the guide dimensions , and wave number .For the above problem  find the lowest frequency  at which a  TE21 mode would propagate.  Derive the relations used.

7.a)	For any uniform transmission line for which R, L, C and G per unit length are independent of position along the line.  Show that the variation along the line of V and I can always be represented by the exponential law.

b)	Derive an expression for the inductance and capacitance per unit length of coaxial transmission line.

8.a)	Explain the significance and Utility of /8, /4, and /2 Lines.

b)	A low loss transmission line of 100  characteristic impedance is connected to a load of 200 .  Calculate the reflection coefficient and standing wave ratio. Derive the relationships used.

***     ***     ***

Code No.220403

Set No.

4

II-B.Tech. II-Semester Examination April/May 2003

EM WAVES AND TRANSMISSION LINES

(Common to Electronics and Communication Engineering and Electronics and Telematics)

Time:3 hours								Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	What are equipotential surfaces? Give two examples of these.

b) 	A line charge ρL = 400pC/m lies along the x-axis. The surface of zero potential passes through the point (0,5,12)m. Find the potential at point (2,3,-4)m.

2.a) 	A circular loop of  3 units radius is centred at the origin in z = 0 plane, and	carries a dc current of 10 m A. along  φdirection.  Find the magnetic flux density at

(0, 0, ± 4).

b) 	For a  N turn  toroid of circular cross section, carrying a current I, determine 	the magnetic field inside and outside the toroid.

3.a)	In a perfect dielectric medium, the EMwave has maximum value for E of 10 V/m with r = 1 and r = 4.  Find the velocity of the wave, peak poynting vector, Average poynting vector, impedance of the medium and peak value of the magnetic field.

- What is the inconsistency in Ampere’s Law?  How it is rectified by Maxwell?

Show that the total displacement current between the condenser plates connected to an alternating voltage sources is exactly the same as the value of charging current (Conduction current).

4.	A Y-polarized uniform plane wave with fields (Ei , Hi) and a frequency of 100 MHz  propogates in air in the + X direction and impinges normally on a perfectly conducting plane at x = 0, assuming the amplitude of Ei to be 6 mv/m, write the phasor and instantaneous expressions for.

a)	Ei and Hi of the incident wave

b)	Er and Hr of the reflected wave

c)	ET and HT of the total wave in air

d)	Determine the location nearest to the conducting plane where ET and HT are Zero.

5.a)	Describe the mechanism of propogation of waves in a Parallel plane wave guide, accounting for different wave lengths and velocities.

b)	Derive an expression for the attenuation factor ‘’ in any general parallel plane guide, using transmission line approach.

(Contd…2)

Code No.220403				-2-				Set No.4

6.  	An X band rectangular wave guide of 1 inch x1/2 inch has a signal propagating at 10GHz. Determine all the propagation characteristics for the dominant mode, if the guide is

  - Air filled.

  - Filled with a dielectric of r = 2.25.

What are your observations?

7.a)	A 100 Line of 1km. is terminated by a 200 load and is fed by a generator of 10V  and internal impedance of 50.  Find the load voltage and load power at 3 x 105 rad/sec.

b)	A distortion less Line of 60 has  = 20 milli Nep/m, and a velocity of 60% of Light velocity.  Find the primary constants at 100 MHz.

8.a)	What is the significance of  standing wave ratio in a transmission line? Calculate the reflection coefficient & VSWR for a 50 line, terminated with (i) matched load (ii) short circuit (iii) +j50 Loads	(iv) -j50 load.

b)	A 50  transmission line is terminated by an unknown impedance.  The VSWR is 4 and the first minima is formed at 2 cm from the load end.  The frequency of operation is 1 GHz.  Design a single stub line matching for the above conditions.

***     ***     ***