Code No.220453

OR

II-B.Tech. II-Semester Examination April/May 2003

EM WAVES AND TRANSMISSION LINES

(Common to Electronics and Communication Engineering and Electronics and Telematics)

Time:3 hours								Max.Marks: 70

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Define and distinguish between the terms Electric field, Electric displacement and Electric flux density with necessary mathematical relations.

b) 	A point charge of 300C is located at point (1, 2, 1)m.  Find the force on another point charge of 30C at the point (3, 0, 0)m and also calculate the Electric field at      (3, 0, 0)m.

2.a)	Using the relation for H  of a finite straight wire, obtain the expressions for the fields due to a semi infinite wire and infinite wire located on z-axis, carrying a current  I.

b) 	In a conducting medium   H   =   y2   z   X +  2 ( x + 1) y z   Y – ( x + 1) z2  Z  A/m. Find the current density at (1,0,-3)  and  calculate the current  passing through  Y  = 1 Plane, 0 ≤ x ≤ 1,  0 ≤ z ≤ 1.

3.a)	In free space D = Dm Sin (Wt +Z)ax.  Determine B and displacement current density.

b)	Region 1, for which r1 = 3 is defined by x <	 0 and region 2, x < 0 has r2 = 5 given H1 = 4 ax + 3ay – 6 az (A/m).  Determine H2 for x> 0 and the angles that H1 and H2 make with the interface.

4.a)	Explain the terms “Linear Polarization”, “Elliptical polarization” and “Circular polarization”

b)	In a Medium Ē = 16 e-x/20 Sin (2.108t –2x) ZV/m.  Find the direction of propogation, the propogation constant, wavelength, speed of the wave and skin depth.

5. 	Starting from Maxwell’s equations, derive the expressions for the E and H field components for TE waves in a parallel plane wave guide.

(Contd…2)

Code No.220453				-2-				OR

6. 	A lossless air dielectric wave guide for an S-band radar has inside dimensions  a=7.214cm and  b=3.404cm. For the TM11 mode propagating at an operating frequency that is  1.1 times the cutoff frequency of the mode , calculate

  - Critical wave number  (b) cutoff frequency  	 (c) operating frequency

(d)  Cutoff wave length     (e) operating wave length (f) guide wave length

(g)  Phase velocity 	       (h) wave impedance 	 (i) group velocity.

7.a)	Starting from the equivalent circuit, derive  the transmission line equations for V and I, in terms of the source parameters.

b)	Give the T and  equivalent network representations for a transmission Line.

8.a)	Explain clearly why the short circuited stubs are preferred over to a open circuited stubs?

b)	Derive the expression for the input impedance of a loss-less line Hence evaluate Zsc and Zoc and sketch their variation with line length.

***     ***     ***