Code No. 221001

Set No.

1

II-B.Tech. II-Semester Examinations April/May, 2003

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Discuss in detail the working of series type voltmeter with relevant diagram and derive the necessary expressions.

2.a)	Explain the method of AC measurement using rectifier type instruments.

b) 	A basic d’Arsonval movement with internal resistance Rm = 150 and full scale

Ifsd = 1.5mA is to be converted into a multirange DC voltmeter with voltage ranges of

(0-10v), (0-50v), (0-250v) & (0-500v),   the following circuit arrangement is used. Determine the values of potential divider network components.

V

100V

Rs=100K

Rx

S=100/V

(0-50V) Scale

3. a)    Give the circuit of a basic DC voltmeter with F.E.T input and explain its working  in brief.

b)	Give a circuit diagram of amplified voltage and current meter capable of measuring milli voltages and currents.

4. a)	How do you measure R, L and C using electronic Instruments?

b)	How do you perform all-electronic capacitance measurements where the measurement is not performed by a comparison? Explain one method in detail.

5. a)	With neat block diagram explain the working function of each block of general purpose oscilloscope.

b)	Mention the advantages of general purpose oscilloscope.

Contd…2

Code No. 221001				.2.				Set No. 1

6. a)	Explain the working operation of a storage CRT with multiple targets and two electron guns with secondary emission curves.

b)	With neat figure, explain schematic view of a bitable storage tube

7. a)	Explain how  you generate sine waves from function generator.  Draw the circuit and describe its working.

b)	What is the principle employed in the operation of a function generator.  Mention the applications of Function Generator.

8. a)	Explain with the help of a block diagram how the period can be               measured?

b)	What is meant by time base error and explain a calibration method to improve the accuracy of it.

###

Code No. 221001

Set No.

2

II-B.Tech. II-Semester Examinations April/May, 2003

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Derive the expression for Rh in shunt type ohm-meter. Also prove with an example its suitability for very low resistance measurement.

2.	Explain the (i) Electrodynamometer type (ii) Rectifier Type of AC measuring instruments.

3. a)	Give a circuit of an AC coupled amplifier to amplify DC signals when the input and out put are chopped.

- Explain the operation of an all-electrical chopper circuit using FET’s.

4.	Explain the working of capacitance measuring meter using Phase shift characteristic of RC circuit.  What are its applications?

5. a)	With neat circuit diagram, explain the function of associated circuits that are used for CRT operation

b)	Explain how the light is emitted on the screen of a CRO.

6. a)	Explain the working functions of each block of a sampling oscilloscope in full detail.

b)	Compare the merits and demerits of sampling oscilloscope

7. a) 	What is the difference between a wave analyzer and harmonic distortion analyzer?

b) 	Explain with the help of block diagram the working of a 	harmonic distortion analyzer?

8. a)	Draw and explain the temperature compensated crystal oscillator circuit

b)	List the suggestions to be followed to attain maximum accuracy in a frequency counter.

c)	Explain the basic principle behind the extension of frequency range of        counter.

###

Code No. 221001

Set No.

3

II-B.Tech. II-Semester Examinations April/May, 2003

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Derive the expression for Rs Source Resistance for both DC ammeter and DC voltmeter and also discuss their types in detail.

2. a)  	Explain the thermo instrument for measuring both AC and DC.

b)		Write short notes on crossed-coil PF meter.

3. a)	Explain 1) R.M.S voltage 2) Form Factor for a sinusoid

b)	A saw tooth voltage is applied to an average responding AC voltmeter with a scale calibrated in terms of the R.M.S value of a sine wave. Calculate 1) The form factor of the saw tooth voltage.   2) Error in the meter indication.

e

150 v

3s                t

T

4. a)	Explain what you understand by Q of an inductor with reference to series resonant circuit?

b)	Give one method of measurement of Q of a coil

5. a)	What are the advantages of dual trace over dual beam for multiple trace oscilloscopes?

b)	How is the vertical axis of an oscilloscope deflected? How does this differ from horizontal axis?

6. a)	Explain the working function of each blocks of a digital storage oscilloscope.

b)	How does the digital storage oscilloscope differ from the conventional storage oscilloscope using a storage CRT? What are the advantages of each?

Contd…2

Code No. 221001				.2.				Set No. 3

7.	What is meant by distortion factor? How can this factor be measured? Explain the operation with the help of block diagram. Give the specifications with typical values of distortion factor meter.

8. a)	Explain the basic principle and working of an electronic frequency counter. Explain how period can be measured.

b)	List the detailed specifications of electronic frequency counters.

###

Code No. 221001

Set No.

4

II-B.Tech. II-Semester Examinations April/May, 2003

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a) 	Derive the value of Torque and Deflection of the galvanometer with its dynamic     behavior.

b)	What value of shunt resistance is required for using 50A meter movement with an internal resistance of 250 for measuring 0-500mA.

2. a)	Explain principle and working of powerfactor meter.

b)	Calculate the value of series resistance required to extend the 0-150V range of a 22,000/V meter to 0-1000V? Also find the power rating.

3. a)	State the general guidelines followed when AC current measurements are made.

b)	State the general characteristics of a digital voltmeter. Name 4 types of DVM’s and explain the principle and working of any one.

4.	Write a note on sources of error encountered in capacitance   measuring meter, explain parasitic inductance, resistance and capacitance.

5. a)	Explain the relationship between trigger pulse and the sweep in an oscilloscope

b)	Explain the block diagram of a dual time base for delayed trigger

6. a)	Briefly explain the working operation of storage and sampling oscilloscope

b)	What precautions must be taken when using a sampling oscilloscope?

7. a)	Discuss in detail about AF square wave generator .

b) 	Explain the importance of Wide band amplifier in the block   diagram of a signal generator

8. a)	Draw and explain the logic diagram of a time base used for a frequency     counter.

- Draw and explain the input signal processing circuit for the frequency          counter.

- Define gating error.

###