Code No:220404

Set No.

1

II-B.Tech. II Semester Examinations April/May, 2003

ANALOG COMMUNICATIONS

(common to Electronics and Communication Engineering, Electronics and Telematics.)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Compare the advantages and disadvantages of telephony over telegraphy.

b)	Draw the circuit diagram of a duplex telegraph system and describe its operation. What is the function of line balance in such a circuit and indicate its form.

2.a)	Give the complete analysis of a diode detector and  explain its operation. Obtain an expression for its efficiency. Account for distortion and discuss the methods to reduce them.

b)	What is Amplitude modulation? Show that a   nonlinear  device can  be used for generating AM signal. What are its limitations?

3.a)	Describe the relationship between FM and PM. Derive the FM equation for Narrow Band and Wide Band FM signals and explain their spectral features.

b)	What is zero crossing detectors? Explain how it works and can be used as an FM demodulator?

4. a)	Explain the Foster – Seely discriminator.

b)	Explain the demodulation of FM signals.

5.a)	What is simple automatic gain control? What are its functions? What is        delayed AGC and what are its merits compared to simple AGC?

b) 		Discuss the considerations in the choice of I.F. and the design of I.F. stage.

6.		What are the causes for fading in radio transmission? Explain in detail. Describe 	briefly different diversity systems for radio reception to reduce fading.

7.		Draw the phasor diagram for an angle modulated signal corrupted by additive 	noise and explain.

8.a)	Why  is it not desirable  to modulate the RF oscillator  of a  transmitter  directly?

b) 	What is the  function  of  buffer  amplifier?

- - -

Code No:220404

Set No.

2

II-B.Tech. II Semester Examinations April/May, 2003

ANALOG COMMUNICATIONS

(common to Electronics and Communication Engineering, Electronics and Telematics.)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What are the advantages and disadvantages of double current over single current working? Give examples where double current working is preferred and explain the reasons.

b) 	Give the use of a telegraph relay. Describe a polarized relay bringing out clearly the purpose of permanent magnet in it.

2.a)	Draw the block diagram of a phase cancellation   SSB generation and   explain how the carrier and unwanted side bands are suppressed . What changes  are necessary to suppress the other sideband?

b)	Calculate the percentage of power   saving    when   the     carrier    and     one       of the sidebands are suppressed in an AM wave modulated to a depth of (i) 100% (ii)50%.

3.a)	An angle modulated signal has the form vt=100cos2Πfc t +4sin2000Πt            when  fc= 10 MHz. (a)Determine the average transmitted power.(b) Determine the peak phase deviation.(c) Determine the peak frequency deviation.(d) Is this an FM or a PM signal?  Explain.

b)	Bring out the comparison between FM and AM.

4. a)	Explain the method of FM demodulator.

b)	Distinguish between envelop detection and synchronous detection methods.

5.a)	What are the advantages of Superheterodyne receiver as compared to a receiver ?

b) 	Explain clearly what is meant by image frequency in a superheat receiver and 	how it can be eliminated.

c) 	Define conversion transconductance of a mixer.  With the help of a typical circuit diagram, explain the working of a separately excited mixer.

6.	What is meant by fading? Explain with suitable figures and examples. Explain the principle of frequency and space diversity techniques employed to reduce the effect of fading.

Contd….2

Code No:220404				-2-				Set No:2

7.	Draw the schematic diagram of the modulator demodulator for FM and prove the signal to noise power ratio at the demodulator output

(SNR)0, fm=3Ac2 k f 2 p /2NoW3.

8.a)	Write short  notes  on FSK.

b) 	Discuss the  operation  of a plate modulated class C amplifier in a radio 	transmitter with  the  help  of  circuit  diagram. What are  conditions  for the class 	C amplifier  to operate with constant efficiency throughout the modulation cycle.

- - -

Code No:220404

Set No.

3

II-B.Tech. II Semester Examinations April/May, 2003

ANALOG COMMUNICATIONS

(common to Electronics and Communication Engineering, Electronics and Telematics.)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define the terms and explain

i) Busy hour 		ii) Traffic units

iii)Full availability 	iv) Grade of service

b) 	What is the difference between grading and trunking and  Smooth pure chance traffic.

2.a)	Explain the operation of a square law detector and compare it with envelope detector.

b)	The signal vt = 1+0.1cos ω1 t + 0.1 cos 2ω1 t cosωc t  is detected by a square law detector Vo=2v2.  Draw the amplitude -frequency characteristic of vo(t).

3.a)	Explain how the frequency modulation is generated using Armstrong system with neat block diagram.  In which circumstances can we dispense with the mixer?

b)	When the modulation frequency in FM system is 400 Hz and modulating voltage is 2.4v the modulating index is 60.  Calculate the maximum deviation.  What is the modulation index when the modulating frequency is reduced to 250Hz and the modulating voltage is simultaneously raised to 3.2V?

4. a)	Explain the Foster – Seely discriminator.

b)	Explain the demodulation of FM signals.

5.	Write short notes on the following:-

a). Automatic gain Control.

b) Limiters in FM receivers.

c) Mixers.

6.	List the distinguishing features of communication receiver. Explain briefly the operation of each where necessary, and show some of the more important circuits or block diagrams.

7.a)	Draw the receiver model and explain its various functional blocks.

b) 	Define (i) input SNR

(ii) output SNR

(iii)channel SNR

(iv) figure of merit.

Contd…..2

Code No:220404				-2-				Set No:3

8. a)	Explain the operation of FSK transmitter  with  block diagram.

b) 	What is pre-distortor   circuit?  Explain its operation.

- - -

Code No:220404

Set No.

4

II-B.Tech. II Semester Examinations April/May, 2003

ANALOG COMMUNICATIONS

(common to Electronics and Communication Engineering, Electronics and Telematics.)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What is the use of telegraph relay? Describe a polarized relay bringing out clearly the purpose of a permanent magnet in it.

b) 	Define telegraph distortion. Explain briefly three types of telegraph distortion.

2.a)	Explain  with the help of a block diagram SSB signal generation using phase discrimination method.  State the advantage of SSB modulation over DSB.

b)	Consider a composite wave obtained by adding a non coherent carrier Ac cos 2Π fc t + φ   to DSB-SC wave Xt cos2Π fc t where Xt is the message waveform.  This composite waveform is applied to ideal envelope detector.  Find the resulting detector out put.

Evaluate this for (1) φ = 0 and(2)φ ≠ 0 and Xt << Ac .

3.a)	Distinguish between phase and frequency modulation.  Show that FM can be derived using PM and vice versa with the help of  differentiator or integrator networks.

b)	Compute the bandwidth requirement for the transmission of FM signal having a frequency deviation 75  KHz and an audio bandwidth of 10KHz.

4. a)	Explain the method of FM demodulator.

b)	Distinguish between envelop detection and synchronous detection methods.

5.a)	Mention briefly the advantages of superheterodyne receiver over TRF receiver.

b) 	Explain why local oscillator frequency should be higher than the input signal

frequency.

c) 	Write about Image frequency and double spotting.

6.	Draw block diagram of a generalized communication receiver. Explain special features of each of the block.

Contd…..2

Code No:220404				-2-				Set No:4

7.	Derive for SNR and Figure of merit in AM receivers.

8. a)	List the  basic functions of a radio transmitter and the corresponding functions of the receiver .

b) 	What  are  the  few  drawbacks  for   connecting  the  oscillator   directly  to  the  	aerial in  radio  transmitters.

- - -