Code No: 220253

OR

II-B.Tech. II-Semester Examinations, April/May, 2003

ELECTRONIC DEVICES

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the function of potential barrier in a PN Junction. Derive the expressions for the built-in-potential. Describe qualitatively how the width to this barrier varies with the applied voltage.

b)	A silicon p-n diode operates at a forward voltage of 0.4 volts. Calculate the factor by which the current will be multiplied when the temperature is increased from 25oC to 150oC.

2.a)	Discuss the early effect. What are the effects produced by base width modulation?

b)	Derive an expression for Ic versus IB for a transistor configuration in the active region.

c)	Draw a typical circuit to bias JFET and explain its operation.

3.a)	Define diffusion capacitance CD of a forward biased diode and prove that CD=τIηVT. Discuss the nature of CD for a sinusoidal input.

b)	A sample of germanium is doped to the extent of 1014 donor atoms/cm3 and

7x 103 acceptor atoms/cm3. At the temperature of the sample the resistivity of pure germanium is 60 cm. If the applied electric field is 2 V/cm find the total conduction current density.

4.a)	Design a self bias circuit for a silicon transistor having  = 50, VBE = 0.6V. The desired operating point is VCE = 5V, IC = 1mA. Assume Rc = 3.5k, S10 and VCC =10V.

b)	Explain in what respects compensation techniques differ from stabilization techniques for biasing a transistor. Describe few compensating techniques.

c)	What are the advantages of MOSFET over JFET?

5.a)	Show various FET biasing techniques.

b)	Explain the operation of MOSFET with necessary diagrams.

c)	With neat diagrams explain forward blocking, reverse blocking and forward conduction states in S.C.R.

(Contd…2)

Code No: 220253				-2-				OR

6.a)	Why is that filtering is easier for FWR outputs, explain the terms ‘Ripple factor, PIV, regulation and rectification efficiency’ as referred to rectifier circuits.

b)	A FWR makes use of a -section filter with two 8F capacitors and one 20H choke. The secondary voltage is 300V with respect to center tap. If the load current is 40mA, determine the d.c. output voltage and ripple without neglecting the choke resistance of 300.

7.a)	Explain the transistor current components?

b)	Explain how the transistor acts as an amplifier?

c)	Explain the common-emitter configuration output characteristics.

8.a)	Define electrostatic deflection sensitivity in a CRT. Derive an expression for the same.

b)	A magnetic field of 0.01 wb/m2 is applied along the axis of a CRT. A field of   104 V/m is applied to the deflecting plates. If an electron leaves the anode with a velocity of 106cm/sec along the axis, how far from the axis will it be when it emerges from the region between the plates. The length of the deflecting plates along the tube axis is 2cm.

***     ***     ***