Code No.220356

OR

II-B.Tech. II-Semester Examination April/May 2003

KINEMATICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechatronics)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Show that the locus of the mid-point of the link connecting to the two slides in an elliptical trammel is a circle.

b) 	Define frame of a machine? Which link of the machine is known as frame?

2.a)	Explain Hart’s mechanism in detail. What are its applications?

b)	Explain the working of Scott – Russel mechanism and modified

Scott – Russel mechanism.

3.		In a Whit worth quick return motion, the crank AB rotates about the fixed centre 	A.  The end B operates a slider reciprocating in a slotted link, rotating about a 	fixed centre D, 5 cm vertically above A.  The crank AB which is 10 cm long, 	rotates in a clockwise direction at a speed of 100 r.p.m.  Find the angular 	acceleration of a slotted link for the configuration in which AB has turned through 	an angle of 450 past its lowest position

4.a)	Explain, with the help of a neat sketch, the space centrode and body centrode

b)	In a pin jointed four bar mechanism ABCD, the lengths of various links are as follows: AB = 25 mm; BC = 87.5 mm; CD = 50 mm and AD = 80 mm.  The link AD is fixed and the angle BAD = 1350.   If the velocity of B is 1.8 m/s in the clockwise direction, find (i) velocity and acceleration of the mid point of BC, and  (ii) angular velocity and angular acceleration of the link CB and CD

5.a)	What is the function of a steeing gear.  What are the mechanisms used in general.  Explain any one of them.

b)	In a Davis Steering gear the distance between the pivot's of the front axle is 1 metre and the wheel base is 2.5 metres find the inclination of the track arm to the longitudinal axis of the car. When it is moving along a straight path.

6.a)	Explain the procedure for drawing the displacement, velocity and acceleration diagrams for a radial cam with simple Harmonic motion of the follower.

b)	Give the expression for maximum velocity and acceleration of follower during ascent and descent with respect to the above motion

(Contd…2)

Code No.220356				-2-				     OR

7.a) 	State and prove the law of gearing for constant velocity ratio and show how the involute profile satisfies the condition?

b) 	A pair of spur wheels with involute teeth is to give a gear ratio 3:1.   The arc of approach is not being less than the circular pitch and the smaller wheel is the driver.   Pressure angle is 20 0.   What is the least number of teeth that can be used on each wheel and find the addendum of the wheel in terms of the circular 	pitch?

8.  	An epicyclic gear train, as shown in figure below is composed of a fixed annular 	wheel A having 150 teeth.  The wheel A is meshing with wheel B which drives 	wheel D through an idle wheel C,D being concentric with A.  The wheels B and C 	are carried on an arm which revolves clockwise at 100 r.p.m. about the axis 	of A and D.  If the wheels B and D have 25 teeth and 40 teeth respectively, 	find the number of teeth on C and the speed and sense of rotation of C.

***	***	***