Code No.:220552

OR

II-B.Tech. II Semester Examinations April/May, 2003

ELECTRONIC CIRCUITS AND LINEAR IC’S

(common to Computer Science and Engineering, Computer Science and Information Technology)

Time: 3 hours.							      Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Derive expression of dc or average value of voltage and rms value of voltage of a full wave rectifier with resistive load.

b)	Show that a full wave rectifier is twice as efficient as a half wave rectifier.

c)	A full wave rectifier has a center-tap transformer of 100-0-100V and each one of the diode is rated at Imax=400ma and Iav=150ma. Neglecting the voltage drop across the diodes, determine

i)	the value of load resistance that gives the largest dc power output

ii)	dc load voltage and current and

- PIV of each diode.

2.a)	Draw a fixed biased circuit and derive expression for the stability factor.

b)	If the various parameters of CE amplifier which uses the self bias method are VCC=12V, R1=100K, R2=5 K RC=1 K RE=2 K and =100 find

i)	the coordinates of operating point and

ii)	stability factor.

Assume the transistor is of silicon.

3.a)	State and briefly explain Backhausen criterian for oscillation.

b)	Draw the circuit diagram of colpitts oscillator. How are feedback requirements met in it and explain the operation of the circuit.

4.a)	What is the difference between ideal and practical OP-AMP.

b)	For the following differential Amplifier using ideal op-amp find the output voltage VO and show that the output corresponding to common mode voltage

VC=(V1+V2)2 is zero if  R'R=R2R1. Find VO also.					Contd…2

Code No.: 220552				-2-					   OR

5.a)	What are the advantages of active filters? Explain wideband band pass filter together with it’s amplitude response.

b)	What is phase shifter? With respect to schematic explain the operation.

6.a)	Distinguish  between RC and LC oscillator circuit. Explain any one of the LC oscillator using OPAmp.

b)	What is meant by voltage regulation? With a basic block diagram explain the operations.

7.a)	Explain the frequency compensation technique in connection with OPAmp.

b)	Distinguish between Timer and Counter. Explain briefly together with block diagram the operation of 555 times as monostable multiciprator.

8.	Write short notes on:

a)	PLL.

b)	RC coupled Amplifier.

c)	feedback amplifier.

^^^