Code No:220205

Set No.

1

II-B.Tech. II Semester Examinations April/May, 2003

ELECTROMECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the working principle of transformer and derive the emf equation.

b)	A single phase 50 Hz transformer has 100 turns on the primary  and 400 turns on 	the secondary winding.  The net cross-sectional area of core is 250 cm2.  If the 	primary winding is connected to a 230 V 50 Hz supply, determine

(i)  The EMF induced in the secondary winding

(ii) The maximum value of flux density in the core.

2.a)	Derive the condition for  zero voltage regulation.  Also show that the magnitude of maximum voltage regulation equals to per unit value of leakage impedance.

b)	A 20 KVA 2000/200 V single phase transformer has the following parameters.

H. V. Winding: R1 = 3 , X1 = 5.3 ,

L. V. Winding: R2 =  0.05 , X2 = 0.1 

Find the voltage regulation at

(i).  At a power factor of 0.8 lagging

(ii). UPF

(iii) 0.707 power factor leading

3.a)	What are the conditions required for the parallel operation of two transformers.

b)	Derive the equations for the currents supplied by each transformer when two 	transformers are operating in parallel with equal voltage ratios.

4.a)	Draw the diagrams of the following transformer connections.

(i)	Scott connection, 	(ii)	Zig-Zag,

(iii)	V-V, 			(iv)	T-connection (3-phase to 3-phase)

b)	A 50 Hz Scott-connected transformer supplies an unbalanced 2-phase load at 	200 V per phase. For the leading phase (phase “A”) the load has a resistance of 	10 ohms and an inductance of 42.3 mH. For the other phase, the load consists of 	a resistor of 13.3 ohms and a capacitor of 318 micro-farads in series. Neglecting 	magnetizing current and the internal impedance of the transformer, calculate the 	line currents on the 3-phase side. The main transformer primary/secondary turns 	ratio is 12/1.

Contd…..2.

Code No.220205				:: 2 ::				Set No.1

5.a)	Does the induction motor have any similarities with the transformer? Compare 	the similarities and differences between them.

b)	Show that a rotating magnetic field is produced in the air-gap, when a balanced 	three-phase ac supply is given to the stator of a 3-phase induction motor. Justify 	your claim with necessary mathematical equations.

6.a)	Derive the relationship between the rotor copper losses and rotor input in a         3-phase induction motor.

b)	A 40 h.p., 3-phase induction motor has a full load slip of 4%.  The stator losses amount to 4% of the input and the mechanical losses are 1% of the output.  If the current in each rotor phase is 50A., find the resistance per phase of the rotor and the efficiency of this machine.

7.a)	Draw and explain the phasor diagram of a 3 phase induction motor.

b)	A 20 hp 400V, 50Hz, 3-phase induction motor runs at 970 rpm at full load.  The mechanical losses total 600W.  If the stator losses are 750 W find (i) rotor copper losses (ii) efficiency of the motor at full load.

- 8.a)	Explain how speed can be controlled by inserting external resistance in rotor winding of 3-phase slip ring Induction motor with the help of slip-torque characteristics.

b)	A 400V,3-phase,4-pole,50HZ,1440 rpm slip ring induction motor is driving a constant torque load. What is the external resistance to be inserted to run the motor at 1200 rpm if the rotor resistance per phase is 0.3 ohm.

@@@@@

Code No:220205

Set No.

2

II-B.Tech. II Semester Examinations April/May, 2003

ELECTROMECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Prove that the EMF induced in the windings of the transformer will lag behind the flux by 90o.

b)	A 100 KVA 11000/230V 50 Hz transformer has H.V. winding resistance of 0.1  and a leakage reactance of 0.4 The L.V.winding has a resistance of 0.006  and leakage reactance of 0.01 .  Find the equivalent winding resistance, reactance and impedance referred to the H. V. and L. V. sides.  Convert these to per unit values.

2.a)	A single phase 120 KVA 2000/200 V 50 Hz transformer has impedance drop of 9 	% and resistance drop of 4.5 %.  Find

(i) The regulation at 0.8 power factor lagging on full load.

(ii) At what power factor is the regulation zero.

b)	The efficiency of 1000 KVA 110/220 V 50 Hz single phase transformer is 98.5 % 	at half full load  at 0.8 power factor leading and 98 %  at full load, UPF.  	Determine   (i).  Iron loss    (ii).  Copper loss  (iii).  Maximum efficiency at UPF

3.a)	Derive the equation for saving in copper in using Auto transformer when 	compared to two winding transformer.

b)	Obtain the equivalent circuit of an auto transformer.

4.a)	Give the constructional features of “CORE” and “SHELL” types of 	transformers, state their advantages and disadvantages.

b)	A 2-phase system supplies through a Scott-connected transformer, a 3-phase 	star connected load in which the line to neutral voltages are balanced but the 	phase currents are as follows:

Phase –A (connected to teaser transformer): Ia= 50 A at 0.866 pf leading

Phase-B: Ib=70.7 A at 0.966 pf leading

Phase-C: Ic=50 A at upf

Neglecting magnetizing current and the internal impedance of the transformer, calculate the supply currents and PF. The 2-phase line voltage is 3300 V and the 3-phase line voltage is 400 V.

Contd…..2.

Code No.220205				:: 2 ::				Set No.2

5.a)	Explain how the torque is produced in the rotor of an induction motor?

b)	(i) Show that the relative speed between resultant rotor field and resultant stator 	field of a 3-phase induction motor is zero, (ii) Derive the expression for the 	rotor e.m.f and rotor current of an induction motor.

- 6.a)	Derive a general expression for the torque developed in a 3-phase induction motor.

- b)	A 3-phase, 4-pole, 400V, 50Hz induction motor develops 15 bhp at 1425 rpm with a power factor of 0.8.  The mechanical losses amount to 0.75 hp.  Determine for this load (i) Slip (ii) rotor copper losses  (iii) gross torque developed in synchronous watts and (iv) line current.  Take the stator losses 1200 watts.

- 7.a)	Prove that in a 3-phase induction motor

- Starting Torque/Maximum Torque = 2/1+2 ,

- Where  is the ratio of rotor resistance to rotor standstill reactance per phase.

- b)	A 30 hp, 3-phase induction motor has a full load slip of 4%.  The stator losses are 1100W and the frictional and windage losses amount to 600W.  If the current in each rotor phase is 30 A, find the resistance per phase of the rotor and the efficiency of the machine.

- 8.a)	Explain the no load and blocked rotor tests.

- b)	A 3-phase,4-pole machine,50HZ squirrel cage induction motor has  rotor leakage impedance of 1+j0.2 ohm per phase stand still voltage of 100V per phase driving a constant torque load at 0.03 slip. What is speed of the motor, if

- i)   Supply voltage is increased by 25% and frequency is constant

ii) Supply voltage is increased by 25% and frequency is decreased by 25%.

@@@@@

Code No:220205

Set No.

3

II-B.Tech. II Semester Examinations April/May, 2003

ELECTROMECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain why hysterisis and eddy current losses occur in a transformer.

b)	A transformer on load takes 1.5 amps at a power factor of 0.2 lagging when 	connected across 50 Hz 230 V supply.  The ratio between primary and secondary 	number of turns is 3.  Calculate the value of primary current when secondary is 	supplying a current of 40 amps at a power factor of 0.8 lagging .  Neglect the 	voltage drop in the windings.  Draw the relevant phasor diagram.

2.a)	What are the sources of heat in a transformer.  Describe briefly various methods 	used for cooling of transformers.

b)	A  40 KVA single phase transformer  has got maximum efficiency of 97 % at 80 	% of full load at UPF.  During the day, the load on the transformer is as follows.

No. of hours		Load		Power factor

9			6 KW		     0.6 lag

8			25 KW		     0.8 lag

7			30 KW		     0.9 lag

Determine the All day efficiency of the transformer.

3.a)	Explain how a two winding transformer can be converted into an auto 	transformer?  What is its new rating?

b)  	Two single phase transformers with equal turns have impedances of (0.5 + j3) 	ohm and (0.6 + j10) ohm with respect to the secondary.  If they operate in 	parallel, determine how they will share a total load of 100KW at p.f. 0.8 lagging?

4.a)	Discuss the construction details of the three phase transformers with necessary 	diagrams. Mention their advantages and disadvantages of different 3 	transformers.

b)	Draw the phasor diagrams and winding connection of a three-phase transformer 	for (i) Group 1: phase displacement of zero degrees, (ii) Group 2: phase 	displacement of 180 degrees.

Contd…..2.

Code No.220205				:: 2 ::				Set No.3

5.a)	Explain why an induction motor cannot develop torque when running at 	synchronous speed. Define the slip speed of an induction motor and deduce how 	the frequency of rotor currents and magnitude of rotor emf are related to slip.

b)	A 4-pole induction motor is energized from a 50 Hz supply. If the machine runs 	on full load at 2% slip, determine the (i) rotor speed with respect to resultant 	rotor field, (ii) stator resultant field speed with respect to rotor structure, and (iv) 	frequency of rotor currents.

- 6.a)	Sketch and explain the torque slip characteristics of 3 phase induction motor.

- b)	A 440V, 4 pole, 50Hz, 3-phase star connected slip ring induction motor has a voltage of 80V between the slip rings, when full voltage is applied to the stator and the slip rings are open circuited with the rotor stationary.  The stator current is 2A at 0.2 p.f. lagging.  The resistance and standstill reactance per phase of the rotor winding are 0.05 and 0.25 ohms respectively.  The rotor winding is star connected.  For the motor running with slip rings short circuited and a slip of 5% determine (i) the mechanical power developed (ii) rotor copper losses and (iii) stator current and power factor.  Neglect the stator resistance and reactance.

- 7.a)	Draw the phasor diagram of 3-phase induction motor.

- b)	A 6-pole, 440V, 3-phase, 50 Hz induction motor has the following parameters of its circuit model (referred to the stator on an equivalent-star basis).

- R1 = 0.0 (stator copper-loss negligible);

- X1 = 0.7

- R’2 = 0.3

- X’2 = 0.7

- Xm = 35

- Rotational loss = 750W

- Calculate the net mechanical power output, stator current and power factor when the motor runs at a speed of 950rpm.

- 8.	Explain the following methods of speed control with slip-torque characteristics.

a) Frequency control.

b) stator voltage control.

@@@@@

Code No:220205

Set No.

4

II-B.Tech. II Semester Examinations April/May, 2003

ELECTROMECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	“Transformer should not be connected to a DC supply”.  Explain.

b)	A single phase transformer with 10 : 1 turns ratio and rated at 50 KVA, 2400/240 	volts, 50 Hz is used to step down the voltage of a distribution system.  The L.V.  	side voltage is kept constant at 240 V.  Find the value of load impedance on L. V. 	side so that the transformer will be loaded fully.  Find also the maximum flux in 	the core if L.V. side has 23 turns.

2.a)	Write a short note on All day efficiency of the transformer.

b)	Find the All day efficiency of single phase transformer having maximum 	efficiency of 98 % at 15 KVA at UPF and loaded as follows.

12 hours  ---		2 KW at 0.5 power factor lagging

6 hours  ---		12 KW at 0.8 power factor lagging

6 hours  ---		no load

3.	A 20KVA, 2300/230V, two winding transformer is to be used as an auto 	transformer, with constant source voltage of 2300V.  At full load of unity power 	factor, calculate the power output, power transformed and conducted.  If the 	efficiency of the two winding transformer at 0.6p.f. is 96%, find the auto 	transformer efficiency at the same power factor.

4.a)	Discuss in detail the effect of  (i) current harmonics, (ii) voltage harmonics on 	the performance of three phase transformers.

b)	Explain how the harmonics can be suppressed using Star/Delta earthing 	transformer. Draw the relevant connection diagram.

5.a)	Explain the principle of operation of a three-phase induction motor and the 	meaning of the term slip. How does slip vary with the load?

b)	Most induction motors have a full load slip of 0.05 or less. The measured speed 	of a 60 Hz motor at rated load is 575 rpm. (i) How many poles does the motor 	have and what is its synchronous speed? (ii) what is the full-load slip? (iii) if no-	load slip is 0.01, what is the percentage speed regulation? (iv) what is the 	frequency of rotor voltage at no-load, full-load and the instant of starting?

Contd…..2.

Code No.220205				:: 2 ::				Set No.4

- 6.a)	Prove that in a 3-phase induction motor Starting Torque/Maximum Torque = 2/1+2, where  is the ratio of rotor resistance to rotor stand still reactance per phase.

- b)	A 3300V 24 poles 50 Hz 3-phase star-connected induction motor runs at 247rpm, while supplying full load torque.  Rotor resistance and stand still reactance per phase are 0.016 and 0.265 ohms respectively.  Find out (i) speed at maximum torque and (ii) ratio of full load torque to maximum torque.

- 7.a)		Discuss briefly about Deep-bar/Double-Cage rotor of 3-phase induction motor.

- b)	The impedances at standstill of the inner and outer cages of a double-Cage rotor are (0.01 + j 0.5)  and (0.05 + j 0.1)  respectively.  The stator impedance may be assumed to be negligible.  Calculate the ratio of the torque due to the two cages when running with a slip of 5%.

- 8.a)		Draw the neat sketch of Scharge motor and explain the principle of operation with 	necessary phasor diagram.

b) 	Explain how the speeds above and below synchronous speeds are obtained in scharge motor.

@@@@@