Code No. 220405

Set No.

1

II-B.Tech. II-Semester Examinations April/May, 2003

CONTROL SYSTEMS

(Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering, Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a)	Discuss Mason’s gain formula.  Obtain the overall transfer function C/R from the signal flow graph shown.

b)	Discuss the effects of negative feedback on the following

i) Gain of the system

ii) Band width

iii) Sensitiveness to parameter variation.

2. a)	Draw the analogous (use force-voltage analogy) electrical system for the mechanical system.  Shown

Fig.

b)	Describe the operation of synchro as an error detector.  Derive the necessary Transfer function.

Contd…2

Code No. 220405					.2.			Set No. 1

3. a)	Define the following terms

i) Steady state error.	ii) Settling time	iii) Peak overshoots.

b)	Sketch the transient response of second order system and derive the expression for rise time and peak over shoot.

4. a)	Consider a feedback system having the characteristic equation.

1+k(s+1)(s+1.5)(s+2)=0

It is desired that all the roots of the characteristic equation have real parts less than -1.  Determine the range of ‘k’ to satisfy this criteria.

b)	The open-loop transfer function of a control system with positive feedback is given by G(s)=ks(s2+4s+4)

Sketch the root locus diagram of the system as a function of ‘k’.

5. a)	Sketch the Bode plot (magnitude and phase plot) for the open loop transfer function.

GH(s)=1s(1+2s)(1+s)

b)	Define Gain Margin and Phase Margin.  Explain the significance of these terms to find the system’s stability.

6. a)	Sketch the Nyquist plot for the given open loop transfer function and discuss closed loop system stability. GH(s)=10s(1+0.2s)(s−1)

b)	 Discuss the effect of adding ploes and zeroes to Nyquist plot.

7. a)	A feedback system is characterized by the closed loop transfer function.

C(s)U(s)= 10(s+4)s(s+1)(s+3)

Construct state model for this system and give block diagram representation of state model.

b)	What are the properties of state transition matrix?

8.	Write short notes on the following:

i)	Lead compensator.

ii)	Effect of adding poles and Zeroes to root locus.

iii)	Amplidyne

###

Code No. 220405

Set No.

2

II-B.Tech. II-Semester Examinations April/May, 2003

CONTROL SYSTEMS

(Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering, Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a)	For the system represented by the following equations, find the transfer functionX(s)U(s) by signal flow graph technique.

x = x1 + 3 u

x1 = -a1x1 + x2 + 2u

x2= -a2x1 + 1u

b)	Find the transfer function x(s) / E(s) for the electromechanical system shown.

Fig.

2. a)	Discuss the advantages and disadvantages of proportional, proportional derivative, proportional integral and proportional integral derivative control system.

b)	A unity feedback system is characterized by an open loop transfer function G(s)=ks(s+10).

Determine the gain k so that the system will have a damping ratio of 0.5.  For this value of k determine settling time, peak overshoot and time to peak overshoot for unit step input.

Contd…2

Code No. 220405					.2.			Set No. 2

3. a)	A system oscillates with frequency w, if it has poles at s = jw and no poles in the right half s-plane.  Determine the values of ‘k’ and ‘a’, so that the system shown in fig.  oscillates at a frequency of 2rad/sec.

Fig.

b)	A unity feedback system has an open-loop transfer function

G(s)=ks2(s+2)

i) By sketching a root locus plot, show that the system is unstable for all values of k.

ii) Add a zero at s=-a (0 a  2) and show that the addition of zero stabilizes the system.

4. a)	Consider the feedback system shown.

(i) Find the value of ‘k’ and ‘a’ to satisfy the following frequency domain specifications.

Mr = 0.4		Wr  = 11.55 rad/sec.

(ii) For the values of ‘k’ and ‘a’ determined in (i), Calculate the settling time and bandwidth of the system.

Fig.

b)	Sketch the Bode plot showing the magnitude in decibels and phase angle in degrees as a function	of log frequency for the transfer function shown below.

GH(s)=10s(1+0.5s)(1+0.1s)

Determine the gain cross over frequency.

Contd…3

Code No. 220405				.3.				Set No. 2

5. a)	Sketch the Nyquist plot for the following transfer function G(s)=k(s+1)s(1+0.2s)(1+0.5s)

Determine the range of ‘k’ such that closed loop system is stable.

b)	Discuss the significance of constant M and N circles to find the closed loop system’s stability.

6.	The open loop transfer function of a unity feedback control system is given by G(s)=8s(1+0.1s+0.001s2)

Draw Bode plots and design a series compensator such the system has a phase margin m = 45o.

7. a)	Obtain state variable representation for the system with transfer function G(s)=ks3+a3s2+a2s+a1

b)	The state equation of a linear system is given by X=01

Obtain the state Transition Matrix.

8.	Write short notes on the following:

(i)	Effect of adding poles and Zeros to root locus.

(ii)	Nyquist stability criterion.

(iii)	AC Servo motor.

###

Code No. 220405

Set No.

3

II-B.Tech. II-Semester Examinations April/May, 2003

CONTROL SYSTEMS

(Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering, Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a)	Draw a signal flow graph and evaluate the closed-loop transfer function of a system whose block diagram is given as follows:

Fig.

b)	Distinguish the performance of armature-controlled DC motor and field-controlled DC motor.

2. a)	What are the advantages of negative feedback? Explain the effect of negative feedback on bandwidth and sensitiveness to parameter variation in closed loop control system.

b)	Draw the electrical analogous circuit of the mechanical system shown below.

Contd…2

Code No. 220405				.2.				Set No.3

Fig.

3. a)	List the transient response specifications.  Derive the mathematical formulae for peak overshoot, rise time of the response of a second order control system for unit step input in terms of damping ratio and undamped natural frequency.

b)	The open loop transfer function of a unity feedback system is given by G(s)=ks(Ts+1), where k and T are positive constants.  By what factor should the amplifier gain be reduced so that the peak overshoot of unit step response of the system is reduced from 75% to 25%.

4. a)	A unity feedback control system is characterized by the open loop transfer function.

G(s)=k(s+13)s(s+3)(s+7)

Using the Routh criterion, calculate the range of values of k for the system to be stable.

b)	Sketch the root locus plot of unity feedback system with an open loop transfer function GH(s)=ks(s+2)(s+4)

Find the range of ‘k’ for which the system has damped oscillatory response. What is greatest value of ‘k’ which can be used before continuous oscillations occur?

Contd…3

Code No. 220405				.3.				Set No.3

5. a)	Sketch the Bode plots in magnitude and phase versus frequency for the following function.

GH(s)=10(1+0.5s)s(1+0.1s)(1+0.2s)

b)	Define Gain Margin and Phase Margin. Explain the significance of gain margin and phase margin to find the closed-loop system stability.

6. a)	Sketch the Nyquist plot for a system with the open loop transfer function GH(s)=k(1+0.5s)(s+1)(1+10s)(s−1)

Determine the range of values of k for which the system is stable.

b)	Explain the effect of adding poles and zeroes to Nyquist plots.

7.	State the properties of state transition matrix.  Obtain the time response of the following system

x1x2=   10

where u(t) is a unit step occurring at t=0 and XT  [0]= [1    0]

8.	Write short notes on the following:

i)	Determination of Static error Co-efficients  (kp  kv   ka) from Bode plots.

ii)	Effect of adding poles and zeros to root locus.

- Lead, Lag and lead-lag compensation networks.

###

Code No. 220405

Set No.

4

II-B.Tech. II-Semester Examinations April/May, 2003

CONTROL SYSTEMS

(Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering, Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1. a)	Using block diagram reduction techniques, find the closed-loop transfer functions of the system whose block diagram is given as

Fig.

b)	Explain the effect of negative feedback on sensitivity of given control system (parameter variations by use of feedback).

2. a)	Distinguish the two phase servo motor and normal induction motor.  Derive the Transfer function of two phase servo motor.

b)	Draw the schematic diagram of a positional servo mechanism.  Find the transfer function of positional servo mechanism.

Contd…2

Code No. 220405				.2.				Set No. 4

3. a)	A unity feedback system is characterized by an open loop transfer functionG(s)=ks(s+10).

Determine the gain k so that the system will have a damping ratio of 0.5.  For this value of k determine settling time, peak overshoot and time to peak overshoot for a unit step input.

b)	The open loop transfer function of a servo system with unity feedback is G(s)=10s(0.1s+1).

Evaluate the static error constants kp, kv and ka.  Evaluate the dynamic error using dynamic error Co-efficients, when the system is subjected to an input given by the polynomial.

r(t)=ao+a1t+a2t22

4. a)	A feedback control system has an open loop transfer function.

G(s) H(s)=ks(s+3)(s22s+2).

Find the root locus as k is varied from 0 to α (infinity).

- The characteristic equation of a servo system is given by

aos5 + a1s3 + a2s2 + a3s + a4 = 0

Determine the conditions which must be satisfied by the coefficients of the characteristic equation for the system to be stable.

5. 	Sketch the Bode plot for a unity feedback system, characterized by the open-loop transfer function G(s)=k(1+0.2s)(1+0.025s)s3(1+0.001s)(1+0.005s)

Show that the system is conditionally stable.  Find the range of values of k for which the system is stable.

6. a)	By use of Nyquist Criterion, determine whether the closed loop system having the following open-loop transfer function is stable or not.

GH(s)=1+4ss2(1+s)(1+2s)

b)	Derive the necessary equations to discuss constant M and N circles.  Explain the significance of these circles to find the stability of closed loop control system.

Contd…3

Code No. 220405				.3.				Set No. 4

7. a)	For the electrical system shown, choose v1(t) and 2(t) as state variables and write down the state equations in vector matrix form.  Given values of R = 1M and C = 1µf.

Fig.

b)	A linear time invariant system is described by the state equation

x1  =  10

where u is a unit step function.  Compute the solution of this equation assuming initial conditions as xo =1.

8.	Write short notes on the following.

(i) Synchro error detector Transfer function.

(ii) Proportional Derivative and proportional integral control systems.

(iii) Effect of adding pole and zeroes to Nyquist plot.

###