Code No:220451

OR

II B.Tech. (II-Semester) Examination, April,May 2003

SWITCHING THEORY AND LOGIC DESIGN

(Electronics and Communication Engineering, Electronics and Control Engineering, Electronics and Instrumentation Engineering, Electronics & Telematics)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	The following is a string of ASCII characters whose bit patterns have been converted in to hexadecimal for compactress: 4A EF 68 6E 20 C4 EF E5. of the eight bits in each pair of digits the leftmost is a parity bit. The remaining bits are ASCII code.

i)	Convert to bitform & decode the ASCII.

ii)	Determine the parity used: odd or even.

b)	Convert the following numbers to decimal : (4310)5 . (198)12 , (16.5)16.

2.a)	The solution to the quadratic equation x2 – 11x + 22 = 0 is x = 3 and x  = 6. What is the base of numbers.

b)	Convert the decimal number 345 to binary in two ways :

i)	Convert directly to binary.

ii)	Convert first to hexadecimal then from hexadecimal to binary. Which

method is faster?

3.a)	Simplify the following Boolean function using K –map

F (w,x,y,z) = ( 0,1,2,4,5,6,8,9,12,13,14).

b)	Draw a NAND logic diagram that implements the complement of the following function:

F (A,B,C,D) = ( 0,1,2,3,4,8,9,12).

4.	Design a combinational circuit that converts a 4 bit gray code to 4 – bit  binary number. Implement the circuit with EX – OR gates.

5.a)	Implement a full adder with two 41 multiplexers.

b)	A Combinational circuit is defined by the following three Boolen functions

F1= x'y'z'+xz	F2  = xy'z'+x'y	F3 =  x'y'z+xy

Design the circuit with a decoder and external gates.

6.a)	What is the difference between serial and parallel transfer ? Explain how to convert serial data to parallel and parallel data to serial. What type of register is needed?

b)	Design a 4 – bit binary synchronous counter with D – flip flops.

Contd…2

Code No:220451		-2-				OR

7.	Design a sequential circuit specified by the state diagram given below using

T – flipflops.

8.a)	Draw the ASM chart for Binary multiplier and explain.

b)	Explain how ASM chart differs from a conventional flow chart.

$$$