Code No:220854

OR

II-B.Tech. II Semester Examinations April/May, 2003

FLUID MECHANICS

(Chemical Engineering)

Time: 3 Hours							  Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

---

1.	A circular plate 3metres in diameter is submerged in water in such a way that 	the greatest and least depth of the surface (below water surface) are 2 metres and 	1 metre respectively. Calculate (i) the total pressure on one side of the surface 	(ii) the position of center of pressure.

2.a)	What do you mean by turbulence ?

b) 	What do you mean by one, two, three dimensional flow?

c)	Derive the continuity equation for one dimensional fluid flow.

3.a)	Derive an expression for the loss due to sudden contraction in pipe flow .

b)	A horizontal pipe of diameter 450 mm is suddenly contracted to a diameter of 200mm. The pressure intensities in the large and smaller pipe are given as          14 N/cm2 and 12 N/cm2 respectively.  Find the loss of head due to contraction if Cc = 0.62. Also determine the rate of flow of water.

4.a)	State the Bernoulli’s theorem for compressible flow. Derive an expression for

Bernoulli’s equation when the process is isothermal and adiabatic.

b)	Measuring the Mach angles a is an accurate means of measuring supersonic velocities for sharp nosed objects such as projectiles. Estimate the velocity and the Mach number of a projectile which reflects a shock wave of angle a=30o in air at a pressure of 93.2kN/m2 and at a temperature of 3oC.

5.	A fluidized bed containing 32,500 kgs of 0.15 mm sand is to be fluidized with air at 4000C and a pressure of 17 atm abs in a cylindrical vessel of 3.05 m in diameter. The density of particle is 2690 kg/m3. The viscosity of air is 0.032 C.P. Calculate (i) Minimum height of fluidized bed (ii) The pressure drop in the bed and (iii) Critical superficial air velocity. Minimum bed porosity is 0.55.

6.	Explain the following in brief:

a) Gate valves and globe valves

b) Plug cocks and ball valves

c) Check valves.

Contd…..2.

Code No. 220854					::2::				OR

7.	A centrifugal fan is used to take flue gas at rest and at a pressure of 730mm Hg and a temperature of 900C  and discharge it at a pressure of 760mm Hg and a velocity of 50 m/sec.  Calculate the power needed to move 17000 m3/hour of gas.  The efficiency of the fan is 60% and the molecular weight of the gas is 32.0.

8.	Explain Venturimeter in detail. Also derive the expression for discharge through a 	venturimeter.

@@@@@