Code No. 221852

OR

II-B.Tech. II-Semester Examinations April/May, 2003

THERMODYNAMICS AND KINEMATICS

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1. a)	Differentiate between (i) Heat engine and refrigerator (ii) Micro state and macro state.

b)	Describe thermal equalibium and explain Zeroth law of thermodynamics?

2. a)	State the first law of thermodynamics and describe the relation between heat and work?

b)	The temperature of 3.5kg of gas in a ligid contains is increased from 22oC and to 39oC by heating it.  The heat transferred during the heating process is 25kJ.  The specific heat ratio and the molecular weight of gas are 1.4 and 28 respectively.  Calculate the work done and change in infernal energy for the gas?

3. a)	Derive Joules Thompson’s coefficient and show that its value is Zero for ideal gas?

b)	An insulated rigid tank initially at zero pressure is connected through a value to an infinite source supply line of steam at 600kPa and 250oC.  The valve is slowly opened to allow the steam to flow into the tank until the pressure reaches 600kPa, and at which point the valve is closed.  Calculate final temperature?

4. a)	Explain the entropy and disorder?

b)	State and explain the claussius statement of second law of thermodynamics?

5. a)	Derive the Maxwell’s relations from the basic thermodynamic relations?

b)	Show that dh=CpdT+V−T∂V∂Tpdp?

6. a)	Derive the equilibrium constant for a chemical reaction?

b)	Determine the maximum quantity of work obtainable from 1kg of stack gases at 250oC and pressure of 1 bar.  If the lowest surrounding temperature is 25oC.  Take Cp = 1.04 kJ/kg K and also calculate irreversibility?

7. a)	Discuss the criteria for chemical equilibrium and derive equilibrium constant?

b)	Water Vapour (H2O) and carbon monoxide (CO) reacts according to the reaction H2O + CO→H2 + CO2 at 2000K.  Determine the equilibrium constant?

8. 	Write short notes on any THREE from the following:

a) Kinetic theory of gases  b) Kirchoff’s equation  c) Thermodynamic temperature scale   d) Collision theory   d) Third law of Thermodynamics.

###