Code No. 220102

Set No.

1

II-B.Tech. II-Semester Examinations April/May, 2003

STRENGTH OF MATERIALS - II

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five question

All questions carry equal marks

---

1.	Determine the principal stresses and the planes carrying them, for the stress system mentioned below. (Tensile stress is positive) Also find the stresses on a plane inclined 300 to x-axis.

x = 50 MPa; y = -25MPa;  xy = 30 MPa.

2.	Briefly explain the theories of failures and compare them.

3.	A solid shaft is subjected to a torque of 100 N-m.  Find the necessary shaft diameter, if the allowable shear stress is 100N/mm2 and the allowable twist is 3o per 10m length of the shaft.  Take C= 1 x 105 N/mm2.

4.	In a close-coiled helical spring, the diameter of each coil is to be 12 times that of the wire of the spring and the maximum shear stress is not to exceed 50 N/mm2.  Maximum permissible deflection under a load of 400N is 80mm.  Using a shear modulus of 8.4 x 104 N/mm2, determine (i) Diameter of the coil (ii) Number of coils.

5.	A 3 meters long column has its one end fixed and other end free.  The column has rectangular section of 150mm x 100mm size.  The column is subjected to a load of 100kN with an eccentricity measured on centroidal axis parallel to 100mm side.  Determine the eccentricity such that the net stress induced on one  of its sides is zero.  Take E=200GPa.

6.	A dam section is 8 meters high, the maximum depth of water impounded being 7.5 meters.  The water face of the dam is vertical.  The top width of section is 1 metre.  The weight of masonry is 22000 N/cum.  While the weight of water is 9810 N/cu.m.  Find the minimum width required. Coefficient of friction between masonry is 0.6.

7. a)	Derive the Lame’s equations.

b)	Calculate the thickness of metal necessary for a cylindrical shell of internal diameter of 80mm to withstand an internal pressure of 25 N/mm2, if the maximum permissible tensile stress is 125N/mm2.

Contd…2

Code No. 220102				.2.				Set No. 1

8.	A curved beam, rectangular in cross- section is subjected to pure bending with couple of +400 N-m.  The beam has width of 20mm, and depth of 40mm and is curved in a plane parallel to the depth.  The mean radius of curvature is 5 mm.  Find the position of the neutral axis, and the ratio of the maximum to the minimum stress.  Also, plot the variation of the bending stress across the section.

###

Code No. 220102

Set No.

2

II-B.Tech. II-Semester Examinations April/May, 2003

STRENGTH OF MATERIALS – II

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five question

All questions carry equal marks

---

1. 	A rectangular block of material is subjected to a tensile stress of 100 MPa on one plane and a tensile stress of 75 MPa on a plane at right angles, together with a shear stress of 50 MPa on the same planes.  Find

(i) The principal stresses.

(ii) The direction of the principal planes

(iii) The greatest shear stress.

2. a)	Explain the need of failure theories in design of members.

b)	Explain in detail.

(i) The maximum principal strain theory and

(ii) The distortion energy theory.

3.	Derive the basic torsion equation TJ=qR=Cθl state all the assumptions made.

4.	A close coiled helical spring of 20 turns has to be designed such that the maximum shear stress produced does not exceed 150Mpa and the stiffness of the spring does not fall below 20kN/cm.  Determine the diameter of spring wire and mean diameter of spring C=0.8 x 105 MPa.

5.	A hollow cylindrical cast-iron column of 150mm external diameter and 15mm thickness, 3.6m length is hinged at one end and fixed at the other.  Find (i) the ratio of Euler’s and Rankine’s loads (ii) The length for which the critical load by Euler’s and Rankine’s formula with be equal.

E= 8.4 x 104 N/mm2 , fc = 525 N/mm2 and α = 1/1600.

6. a)	What do you understand by the kern of Section?

b)	A square hollow chimney of outside dimension 1500 mm and thickness 150mm, is 30m high.  Dead weight of the material of the chimney is 25 kN/m3. If a wind load of intensity 2.5kN/m2 acts through out the height of the chimney, determine the maximum and minimum intensities of stresses induced at the base of the chimney.

Contd…2

Code No. 220102				.2.				Set No. 2

7. 	A C.I pipe has 200mm internal diameter 50mm metal thickness, and carries water under a pressure of 5N/mm2.  Calculate the maximum and minimum intensities of circumferential stress and sketch the distribution of circumferential stress intensity and the intensity of radial pressure across the section.

8. a)	Define centroidal principal axes of a section.

b)	Determine the principal moments of inertia for an angle section 100mm x 75mm x 8mm with 100 mm leg Vertical.

###

Code No. 220102

Set No.

3

II-B.Tech. II-Semester Examinations April/May, 2003

STRENGTH OF MATERIALS - II

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer and Five question

All questions carry equal marks

---

1.	If x = 1 = 0 and y = 2 = -20 MPa using Mohr’s circle of stress, find the stresses acting on a plane defined by  = + 30o.

2.	The load on a bolt consists of an axial pull of 8000N together with a transverse shear of 3000N.  Estimate the diameter of the bolt required according to

(i) Maximum Principal stress theory

(ii) Maximum shear stress theory

(iii) Distorsional strain energy theory.

Elastic limit in simple tension is 270N/mm2 and factor of safety of 3 is to be applied.  Poisson’s ratio = 0.25.

3.	A shaft is required to transmit 40kw power at 200 r.p.m.  The maximum torque may exceed the mean by 60%. Determine the cross-section if

(i) it is solid circular.

(ii) it is hollow circular having external diameter 1.5 times the internal.

4. 	A helical spring is made of 12mm diameter steel wire by winding it on a 120mm diameter mandrel.  If there are 10 active coils, what is the spring constant?  What force must be applied to the spring to elongate it 10mm?

5.	A round steel rod of diameter 15mm and length 2 meters is subjected to a gradually increasing axial compressive load.  Using Euler’s formula find the buckling load.  Find also the maximum lateral deflection corresponding to the buckling condition.  Both ends of the rod may be taken as hinged.  Take E=2.1 x 105 N/mm2 and the yield stress of steel = 250N/mm2.

6.	A masonry column has rectangular cross-section ABCD.  AB=CD=4.8m, BC = AD = 3.6m.  The resultant vertical load on the column is 640kN acting at a distance of 1.2m from AB and 1.5m form BC.  Calculate the value of stress at each of the four corners A, B, C and D.

Contd…2

Code No. 220102				.2.				Set No.3

7. a)	State assumptions made in lame’s theory.

b)	A hollow cylinder open at ends has 150mm internal diameter and 200mm external diameter.  It is subjected to an internal pressure of 25N/mm2.  Determine

(i)	maximum hoop stress, maximum radial stress.

(ii)	Maximum shear stress.

(iii)	Increase in internal and external diameter.

Take E = 2 x 105 MPa and v = 0.25.

8. 	A beam of rectangular section 100mm wide and 150mm deep is subjected to a bending moment of 12kN-m.  The trace of the plane of loading is inclined at 45o to the y-y axis of the section.  Locate the neutral axis of the section and calculate the maximum bending stress induced in the section.

###

Code No. 220102

Set No.

4

II-B.Tech. II-Semester Examinations April/May, 2003

STRENGTH OF MATERIALS - II

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer and Five question

All questions carry equal marks

---

1.	At a point in a material, the resultant stress on a certain plane is 400kN/m2 tensile, inclined at 30o to the normal for the plane.  The normal stress on an orthogonal plane is 300kN/m2 compressive.  Find the resultant stress on the second plane, principal stresses and their planes.

2.	A solid shaft is subjected to a B.M. of 15kN-m and a tiwisting moment of     10kN-m.  Determine the diameter of the shaft, if the material of the shaft yields at 125MPa under direct tension, based on

(i) Maximum principal stress theory

(ii) Maximum shear stress theory

(iii) Maximum shear strain energy theory

A factor of safety of 3 is to be applied with poisson’s ratio of 0.25.

3.	Design a hollow circular steel shaft that can withstand a maximum shear stress of 50MPa and a maximum twist 2o in a 6m length subjected to a torque of 12kN-m.  Take C= 83GPa.

4.	A close coiled helical spring 10cm mean diameter is made of 20 turns of 1 cm dia.  Steel rod.  The spring carries an axial load of 100N.  Find the shearing stress developed in the spring and the deflection of the load.  Assume modulus of rigidity 84GPa.

5. a)	Write the assumptions made in Euler’s theory of columns.  Also mention its limitations.

b)	A T-section has flange 150mm x 20mm and web100mm x 20mm.  It is used as a column with both ends fixed.  If the column is 3.4m long.  Calculate the crippling load.  Take E=2 x 105 N/mm2.

6.	A simply supported beam of rectangular section of 80mm to 120mm carries a U.D.L of 40kN/m over a span of 1m and an axial compressive force of 5kN.  Calculate (i) maximum and minimum fibre stress. (ii) fibre stress at a point 0.25m from the left end of the beam and 40mm below the Neutral axis.

Contd…2

Code No. 220102				.2.				Set No. 4

7. a)	Briefly explain compound cylinders.

b)	A spherical shell of 80mm internal diameter has to withstand an internal pressure of 30N/mm2.  Calculate the thickness of the shell if the maximum permissible tensile strength is 80N/mm2.

8.	A beam of angle section 100mm x 75mm x 8mm is  simply supported over span of 1m with 100mm leg vertical.  A U.D.L of 8kN/m is applied throughout the span.  Determine (i) Maximum bending stress, (ii) Direction of neutral axis; and (iii) Deflection at the centre.

###