Code No.220204

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	With the neat diagrams discuss the following:

i)   Penstocks		(ii)  Water Hammer

b)	Write short notes on:-  (i)   Choice of size and    (ii)  No. of Generating Units.

2.a)	What is the use of an electron precipitator along with the modern boilers? Explain its working.

b)	What is 'feed water '? What are the problems due to impurities in feed water? How they can be eliminated?

3.	Write notes on auxiliaries of boilers.

4.	Discuss the following:

- Half life period	(b) Energy mass equivalence 	(c) Binding energy

5.a) 	How do you solve the single phase AC distribution system in which the power factors of the load currents are referred with respect to receiving end voltage.

b) 	A single phase ac distributor AB 300 metres long is fed from end A and is loaded as follows: (i) 100 A at 0.707 power factor lagging 200 metres from point A. (ii) 200 A at 0.8 power factor lagging 300 metres from point A.

The total resistance and reactance of the distributor is 0.2 ohms and 0.1 ohms per 	kilometer. Calculate the total voltage drop in the distributor. The load power 	factors are referred to the voltage at the far end.

6.a) 	What is sub-station ?. Name the factors that should be taken care of while designing and erecting a sub-station ?

b) 	Compare the indoor and outdoor sub-stations

7.a) 	Explain how load curves help in the selection of size and number of generating units.

b) 	A factory has a maximum load of 240 kW at 0.8 pf lagging with an annual 	consumption of 50 000 units. The tariff is Rs. 350 per kVA maximum demand 	plus Rs. 2 per unit. Calculate the flat rate of energy consumption. What will be the 	annual saving if pf is raised to unity?

8.a) 	Discuss the important points to be taken into consideration while selecting the size and number of units.

b) 	Give the basis for expressing the cost of electrical energy as (a + b * kW + c * kWh) and explain the factors on which a, b and c depend.

***     ***     ***

Code No.220204

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain how reservoir capacity is determine from mass curve.

b)	Define specific speed of a turbine and mention its significance.  A turbine 	develops 400 HP at a speed of 1500 RPM with an available head of 80 mtrs.  	Calculate its specific speed.

2.	Explain the basic principle of working of pumped storage plants.  Mention the advantages and disadvantages of the scheme.

3.a)	Explain the function of chimney and precipitator.

b)	Discuss the need of cooling towers and list out various types of cooling towers.

4.a)	Explain the structure of an atom. What is the difference between Atomic number and Mass number and Mention their relevance in nuclear reaction.

b)	Write shot notes on :

i)	Isotopes with examples	(ii)  Rate of Radio-active decay.

5.a) 	Explain the method of economic choice of conductor size of distribution system?

b) 	The cost of a 3 phase overhead transmission line is Rs. (25,000 a + 2500) per KM 			where a is the area of cross section of each conductor in cm2. The line is 				supplying a load of 5 MW at 33 KV and 0.8 pf lagging assumed to be constant 			throughout the year. Energy costs 4 paise per KWh and interest and depreciation 			total 10% per annum. Find the most economical size of the conductor. Given that 			specific resistance of conductor material is 10-6 ohm-cm.

6.a) 	What is distribution system ?. How will you classify it ?

b) 	Explain the following :

- Feeder		(ii)  Distributor	(iii)  Service mains

7.a) 	Define the term “Diversity factor” and prove that the load factor of a supply 	system is improved by increase in diversity of load.

b) 	Calculate annual bill of a consumer whose maximum demand is 100 kW, pf = 0.8 	lagging and load factor = 60%. The tariff used is Rs. 475 per kVA of maximum 	demand plus Rs. 2.5 per kWh consumed.

8.a) 	What do you understand by (i) base load and (ii) peak load of a power station?

b) 	Explain how load curves help in the solution of size and number of generating 	units.

***     ***     ***

Code No.220204

Set No.

3

II-B.Tech. II-Semester Examination April/May 2003

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What do you understand by run-off river plant?  Describe how a run-off river plant is operated under different flow conditions?

b)	Discuss the difference between Kaplan, Francis and Pelton turbines and state the type of power plants they are suitable for.

2.a)	 What are the requirements of a boiler being used for a power station? How do you classify them?

b)	Explain briefly horizontal straight water tube boiler. What are its merits against fire tube boiler?

3. 	Discuss steam station controls with reference to boiler control, turbine control and electrical control.

4.		Differentiate between  (a) Fissile and fertile materials	(b) Fission and Fusion

c) Natural and enriched uranium

5.a) 	Explain Kelvin’s Law for cables and list out its drawbacks

b) 	Calculate the most economical cross-section of the conductor of a 4-KM long 	transmission line if it delivers a maximum current of 400 amperes and the year’s 	copper losses are such as to be caused by a constant current of 100 amperes. The 	energy cost is 10 Paise per KWh. The cost of the line is Rs.(50,000 a + 8,000) per 	KM. Resistance per KM per square cm of copper conductor is 0.173. The annual 	rate of interest and depreciation is 10 %.

6.a) 	What is the need of sub-station in a power system ?

b) 	Why do we use isolators on both sides of circuit breakers ?

c) 	What are the various Electrical quantities measured and monitored in a sub-		station.

7.a) 	What do you understand by tariff? Discuss the objectives of tariff.

b) 	A diesel station supplies the following loads to various consumers: Industrial consumer = 	1500 kW; commercial establishment = 750 kW; Domestic power = 100 kW; Domestic 	light = 450 kW. If the maximum demand on the station is 2500 kW and the number of 	kWh generated per year is 45x105, determine (i) the diversity factor and (ii) the annual load factor.

8.a) 	Define the term “Diversity factor” and prove that the load factor of a supply 	system   is improved by increase in diversity of load.

b) 	What do you understand by tariff? Discuss the objectives of tariff.

***     ***     ***

Code No.220204

Set No.

4

II-B.Tech. II-Semester Examination April/May 2003

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What are the factors, which should be considered while selecting a site for a hydro-electric power plant?

b)	With a neat sketch, explain the schematic arrangement of Hydro-electric power plant.

2.	What do you understand by dry bottom furnace and wet bottom furnace? Explain with relevant diagrams.

3. 	What are the functions of Economizer and superheater in a thermal power plant?

4.a) 	Explain why the nuclear power plants are considered very useful for base load plants?

b) 	Why is a site near a sea or river or lake and away from thickly populated area        	 considered an ideal site for nuclear power plants?

5.a) 	Explain Kelvin’s law. When will the Kelvin’s law fail to give the most economical size of conductor. Explain the limitations in detail.

b) 	Find the best curret density for a 3 phase overhead line if the line is in use for 	2,600 hours per annum, and if the conductor costs Rs.1,500 per tonne, has a 	specific resistance of 1.73  μΩ-cm. and weighs 0.062 gm per cubic cm. Energy 	cost is 5 paise per unit and interest and depreciation together amount to 12 % per 	annum of the conductor.

6.a) 	Draw and explain the sectionalized double busbar system.

b) 	With suitable diagrams explain the main and transfer bus arrangement.

7.a) 		Discuss the important points to be taken into consideration while selecting the 	size and number of units.

b) The annual working cost of a power station is represented by the formula Rs. (a + b* kW + c*kWh) where the various terms have their usual meaning. Determine the values of a, b and c for a 60 MW station operating at annual load factor of 50% from the following data: (i) capital cost of building and equipment is Rs 15x106, (ii) the annual cost of fuel, oil, taxation and wages of operating staff is  Rs.  30 00 000, (iii) the interest and depreciation on building and equipment are 10% per annum, (iv) annual cost of organization and interest on cost of site etc. is Rs. 15 00 000. The annual working cost of a power station is represented by the formula Rs. (a + b* kW + c*kWh) where the various terms have their usual meaning. Determine the values of a, b and c for a 60 MW station operating at annual load factor of 50% from the following data: (i) capital cost of building and equipment is Rs 15x106, (ii) the annual cost of fuel, oil, taxation and wages of operating staff is  Rs.  30 00 000, (iii) the interest and depreciation on building and equipment are 10% per annum, (iv) annual cost of organization and interest on cost of site etc. is Rs. 15 00 000.

The annual working cost of a power station is represented by the formula Rs. (a + b* kW + c*kWh) where the various terms have their usual meaning. Determine the values of a, b and c for a 60 MW station operating at annual load factor of 50% from the following data: (i) capital cost of building and equipment is Rs 15x106, (ii) the annual cost of fuel, oil, taxation and wages of operating staff is  Rs.  30 00 000, (iii) the interest and depreciation on building and equipment are 10% per annum, (iv) annual cost of organization and interest on cost of site etc. is Rs. 15 00 000.

The annual working cost of a power station is represented by the formula Rs. (a + b* kW + c*kWh) where the various terms have their usual meaning. Determine the values of a, b and c for a 60 MW station operating at annual load factor of 50% from the following data: (i) capital cost of building and equipment is Rs 15x106, (ii) the annual cost of fuel, oil, taxation and wages of operating staff is  Rs.  30 00 000, (iii) the interest and depreciation on building and equipment are 10% per annum, (iv) annual cost of organization and interest on cost of site etc. is Rs. 15 00 000.

The annual working cost of a power station is represented by the formula Rs. (a + b* kW + c*kWh) where the various terms have their usual meaning. Determine the values of a, b and c for a 60 MW station operating at annual load factor of 50% from the following data: (i) capital cost of building and equipment is Rs 15x106, (ii) the annual cost of fuel, oil, taxation and wages of operating staff is  Rs.  30 00 000, (iii) the interest and depreciation on building and equipment are 10% per annum, (iv) annual cost of organization and interest on cost of site etc. is Rs. 15 00 000.

The annual working cost of a power station is represented by the formula Rs. (a + b* kW + c*kWh) where the various terms have their usual meaning. Determine the values of a, b and c for a 60 MW station operating at annual load factor of 50% from the following data: (i) capital cost of building and equipment is Rs 15x106, (ii) the annual cost of fuel, oil, taxation and wages of operating staff is  Rs.  30 00 000, (iii) the interest and depreciation on building and equipment are 10% per annum, (iv) annual cost of organization and interest on cost of site etc. is Rs. 15 00 000.

The annual working cost of a power station is represented by the formula Rs. (a + b* kW + c*kWh) where the various terms have their usual meaning. Determine the values of a, b and c for a 60 MW station operating at annual load factor of 50% from the following data: (i) capital cost of building and equipment is Rs 15x106, (ii) the annual cost of fuel, oil, taxation and wages of operating staff is  Rs.  30 00 000, (iii) the interest and depreciation on building and equipment are 10% per annum, (iv) annual cost of organization and interest on cost of site etc. is Rs. 15 00 000.

The annual working cost of a power station is represented by the formula Rs. (a + b* kW + c*kWh) where the various terms have their usual meaning. Determine the values of a, b and c for a 60 MW station operating at annual load factor of 50% from the following data: (i) capital cost of building and equipment is Rs 15x106, (ii) the annual cost of fuel, oil, taxation and wages of operating staff is  Rs.  30 00 000, (iii) the interest and depreciation on building and equipment are 10% per annum, (iv) annual cost of organization and interest on cost of site etc. is Rs. 15 00 000.

The capital cost of a hydro-power station of 50MW capacity is Rs 1,000 per KW.  The annual depreciation charges are 10% of the capital cost. A royalty of Re 1per KW per year and Re 0.01 per KWh generated is to be paid for using the river water for generation of power. The maximum demand on the power station is 40 MW and annual load factor is 60 . Annual cost of salaries, maintenance charges etc is Rs 7,00,000. If 20% of this expense is also chargeable as fixed charges, calculate the generation cost in two part form. . 	A generating station has the following daily load cycle:

Time (hour)         0-6        6-10        10-12     12-16        16-20        20-24

Load (MW)          40          50            60           50              70             40

Draw the load curve and find the (i) maximum demand (ii) units generated per 	day (iii) average load and (iv) load factor.

8.a) 	Discuss the important points to be taken into consideration while selecting the 	size and number of units.

b) 	Discuss the various methods of determining the depreciation of the equipment.

***     ***     ***