Code No:220303

Set No.

1

II-B.Tech. II Semester Examinations April/May, 2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What do you understand by air standard cycle for engine operation and list out the 	assumptions implied.

b)	What is meant by mechanical cycle of I.C. engine operation and explain the types.

2.a)	Describe phenomenon of pre-ignition in S.I.engines and discuss its effect on the performance.

b)	Discuss the factors which promote pre-ignition.

3.a)	Explain the terms ‘delay period’ and ‘knocking’ as referred to for C.I. engines.

b)	Compare the normal combustion phenomenon in S.I. and C.I. engines.

4.		During the trail of a four stroke diesel engine the following observations were 	recorded:

Area of the indicator diagram		= 475 mm2

Length of indicator diagram		= 62 mm

Spring number				= 1.1 bar / mm

Diameter of piston 			= 100 mm

Length of the stroke			= 150 mm

Engine RPM				= 375

Determine (a) indicated mean effective pressure (b) indicated power in kW.

5.a)	Explain the terms effective swept volume and displacement volume of the compressor.

b)	A single acting compressor has zero clearance, stroke of 20cm and piston diameter 15cm.  When the compressor is operating at 235rpm and compressing air from 10N/cm2, 25oC to 41N/cm2, find (i) the volume rate of air handled and (ii) the ideal  power required.

6.a)	Explain the working of an axial flow compressor with mechanical details with a 	sketch.

b)	What do you understand by degree of reaction.

Contd……2.

Code No.220303				:: 2 ::				Set No.1

7.	A vapour compression refrigeration system works between temperature limits of

200C and –100C. The vapour at the end of isentropic compression is 0.9 dry. Assuming there is no cooling find the COP of the system. Also find the percentage change in COP if the vapour temperature after the compression is dry.

| Temperature,K | Enthalpy in kJ/kg |  |
| --- | --- | --- |
|  | hf | hg |
| 293 | 0.28 | 4.6 |
| 263 | -0.14 | 5.0 |

8.a)	Explain the working of year round air conditioning system with a neat diagram.

b)	What are the psychrometric properties? Give the different applications of psychrometrics?

@@@@@

Code No:220303

Set No.

2

II-B.Tech. II Semester Examinations April/May, 2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the principle of working of a four stroke S.I. engine with a neat sketch.

b)	How does S.I. engine respond to varying load conditions? Explain the mechanism.

2.a)	Explain the desirable characteristics of fuels for S.I.engines.

b)	Discuss the density and time factors affecting detonation in S.I.engines.

3.a)	“The factors which promote knock in C.I. engines will reduce knock in S.I. 	engines”. Justify the statement.

b)	What are the ignition characteristics desirable for C.I. engine fuels and explain.

4.	A single cylinder internal combustion engine gave the following results when put   	to a trial test.

Area of the indicator diagram			=12 cm2

Length of the indicator diagram		= 8 cm

Spring scale 					= 2.8

Diameter of the piston				= 21 cm

Stroke length 					= 28 cm

Speed of the engine				= 370 RPM

Determine IP of the engine when (a) working on 4 stroke cycle and (b) working on two stroke cycles.

5.a)	What is the principle of operation of the reciprocating compressor valves?

b)	An air compressor has a piston displacement of 2200 cm with a clearance of 5%.  It receives air at 110kPa.  There is a pressure drop of 3.5kPa through the suction valves.  The discharge valves also leaks and a drop of 5% occurs in delivery pressure.  Using n=1.35, calculate and plot the volumetric efficiency for the discharge pressures of 350, 700 ,1000, 1500,2000 and 2500kPa.

6.a)	How does the working of centrifugal compressor differ from the axial flow 	compressor. Explain.

b)	For a degree of reaction of 50%, draw the velocity diagrams for the rotor.

Contd……2.

Code No.220303				:: 2 ::				Set No.2

7.	A vapour compression refrigerator circulates 4.5 kg of NH3 per hour condensation takes place at 300C and evaporation at –150C. The temperature after isentropic compression is 750C and the specific heat of the superheated vapour is              2.82 kJ/kg K. Determine (i) COP, (ii) ice produced in kg/hr from water at 200C and ice at 00C. Take enthalpy of fusion of ice = 336 kJ/kg  and specific heat of water is 4.187 kJ/kg K.

| Saturation temp 0C | Enthalpy, kJ/kg |  | Entropy, kJ/kg K |  |
| --- | --- | --- | --- | --- |
|  | hf | hg | sf | sg |
| -15 | 112.3 | 1426 | 0.457 | 5.549 |
| 30 | 323.1 | 1469 | 1.204 | 4.984 |

8.a)	Write a note on psychrometric chart and its utility value in airconditioning.

b)	Explain the working of winter air-conditioning system with a neat sketch.

@@@@@

Code No:220303

Set No.

3

II-B.Tech. II Semester Examinations April/May, 2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	For a petrol engine explain the fuel system with a line diagram. How does  it help control the load variation.

b)	How could an engine be identified whether it is a petrol or diesel engine.

2.a)	Discuss the desirable principles of combustion chamber design for S.i.engines.

b)	How does preignition leads to detonation and vice versa.

3.a)	How C.i. engine Fuels are rated? Explain the methodology.

b)	Why does a turbulent type of combustion chamber experience difficulty in starting. Explain.

4.		In  a test with a four cylinder four stroke petrol engine, the following results are 	obtained for a particular setting and speed

BP with all cylinders working				=24.0 kW

BP with No.1 cylinder cutoff				=16.2 kW

BP with No.2 cylinder cutoff				=16.7 kW

BP with No.3 cylinder cutoff				=16.8 kW

BP with No.4 cylinder cutoff				=17.3 kW

Estimate the IP of the engine and its mechanical efficiency.

5.a)	Why polytropic compression is used instead of isothermal and adiabatic compression?

b)	Air is to be compressed in a single stage reciprocating compressor from 101.3kPa and 15oC to 700kPa.  The free air delivery is 0.3m3/min when the compressor speed is 1000rpm.  If the compressor is single acting and has a stroke/bore ratio of 1.2,  calculate the bore size required.

6.a)	List the various types of rotary compressors?

b)	Explain with a neat sketch, the working of a roots blower.

Contd……2.

Code No.220303				:: 2 ::				Set No.3

7.	A 5 tonne vapour compression refrigeration system using NH3 works between 400C and –100C. There is no subcooling and the gas is dry and saturated at the end of the compression. Calculate COP and flow rate of refrigerant?

| Saturation temp 0C | Enthalpy, kJ/kg |  | Entropy, kJ/kg K |  |
| --- | --- | --- | --- | --- |
|  | hf | hg | sf | sg |
| -10 | 154.0 | 1450 | 0.8296 | 5.755 |
| 40 | 390.5 | 1490 | 1.644 | 5.156 |

8.a)	Define the terms relative humidity and specific humidity and illustrate its application with respect to air-conditioning?

b)	What do you understand by air-conditioning and what are its applications in modern day living.

@@@@@

Code No:220303

Set No.

4

II-B.Tech. II Semester Examinations April/May, 2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Why do you need  lubrication in i.c. engines and name the types of lubrication.

b)	Explain the principle of working of a battery ignition system  with a neat sketch.

2.a)	Explain how induction swirl is created in C.I. Engine combustion chambers.

b)	Discuss the advantages and disadvantages of induction swirl.

3.a)	What are the requirements of fuel injection in C.i. engines and how do they differ 	with the type of combustion chamber.

b)	Why do turbulent type chambers cause higher heat transfer than the non-turbulent 	type of C.I. engine?

4.		A four cylinder four stroke petrol engine of 6.35 cm bore and 8.9 cm stroke was 	tested at full throttle and constant speed.  The fuel supply was fixed at             	0.064 kg / min and the plugs of the cylinders were short circuited without 	change 	of speed, the brake torque being correspondingly adjusted.  The power 	measurements were as follows:

With all cylinders 				=12.3  kW

With cylinder No.1 cutout			=8.90  kW

With cylinder No.2 cutout			=8.80  kW

With cylinder No.3 cutout			=8.70  kW

With cylinder No.4 cutout			=8.85  kW

Estimate the indicated power of the engine under these conditions.  Find indicated thermal efficiency of the engine if the calorific value of fuel used is 43680 kJ / kg.  Compare this efficiency with the air standard value, if clearance volume of one cylinder is 70.3 cm3.

5.a)	Give two merits of rotary compressor over reciprocating compressor?

b)	2 kg/s of air enter the LP cylinder of 2 stage compressor. The overall pressure ratio is 9:1. The air at inlet to the compressor is at 100 KPa and 35°C. The index of compression in each cylinder is 1.3. Find the intercooler pressure for perfect intercooling. Also find the minimum power required for compression and percentage saved over single stage compression.

Contd……2.

Code No.220303				:: 2 ::				Set No.4

6.a)	Under what circumstances would you recommend the use of multistage compressor

b)	Explain with a neat sketch, the working of a vane blower?

7.	An NH3 vapour compression refrigeration system working between condenser temperature of 300C and evaporator temperature of –120C. The temperature of the refrigerant after isentropic compression is 40.50C. The saturated liquid refrigerant is expanded. If the specific heat of superheated vapour refrigerant is 2.94 kJ/kg K, Calculate the COP?

| Saturation temp 0C | Enthalpy, kJ/kg |  | Entropy, kJ/kg K |  |
| --- | --- | --- | --- | --- |
|  | hf | hg | sf | sg |
| -12 | 365 | 1672 | 3.9946 | 9.0006 |
| 30 | 562 | 1711 | 4.6897 | 8.4802 |

8.a)	What are commonly used refrigerants? Write the merits and demerits of Freon Group?

b)	Draw the typical layout of air conditioning unit and explain.

@@@@@