Code No.220402

Set No.

1

II-B.Tech. II-Semester Examination April/May 2003

ELECTRICAL TECHNOLOGY

(Common to Computer Science and Engineering, Computer Science and Information Technology, Electronics and Communication Engineering, Electronics and Computer Engineering, Computer Science and Systems Engineering and Electronics and Telematics)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Why brushes and commutator are necessary for operation of a D.C. Machine.

b)	How D.C. Generators are classified?

c)	The armature of a 6-pole d.c. generator has a wave winding containing 664 	conductors. Calculate the generated e.m.f. when flux per pole is 0.06 weber and 	the speed is 250rpm. At what speed must the armature be driven to generate an 	e.m.f. of 250V if the flux per pole is reduced to 0.058 weber?

2.a)	With the help of neat sketches, explain torque-speed characteristics of the

i) D.C. Series  ii) D.C. Shunt motors.

b)	Find the torque in N-m exerted by a 4-pole series motor whose armature has 1200 	conductors connected up in a 2-circuit winding. The motor current is 10A and the 	flux per pole is 0.02Wb.

3.a) 	Draw the equivalent circuit of a transformer and show how the constants of

primary and secondary windings may be combined to give a simplified equivalent circuit with the values of constants given in terms of secondary winding.

b) 	Explain the constructional details of 1-phase transformer.

4.a) 	Define efficiency of a transformer. Obtain the condition for maximum efficiency.

b) 	A 25 kVA, 2500 / 250 V, single-phase transformer gave the following test results.      O.C. test (LV side): 250 V, 1.4 A, 105 Watts.

S.C. test (HV side): 105 V, 8 A, 320 Watts.

Compute the equivalent circuit parameters referred to LV side and HV side. Also obtain percentage regulation at full load with 0.8 power factor lagging.

5.a) 	Sketch the torque-speed curve of an induction motor and indicate how this will change when rotor resistance is varied.

b)	A 40 kW, 3-phase slip ring induction motor of negligible stator impedance runs at 	a speed of 0.96 times synchronous speed at rated torque. The slip at maximum 	torque is 4 times the full load value. If the rotor resistance of the motor is 	increased by 5 times, determine (i) the speed, power output and rotor ohmic loss 	at rated torque (ii) the speed corresponding to maximum torque. Neglect 	mechanical losses.

(Contd…2)

Code No.220402				-2-			Set No.1

6.a)	Explain the tests to be conducted for determining synchronous impedance

b) 	A 100KVA, 6.6KV, 3phase star connected synchronous generator has a 	synchronous reactance of 25 ohm per phase. It supplies full load current at 0.75 	p.f lagging and a rated terminal voltage. Compute the terminal voltage for the 	same excitation when the generator supplies full load current at 0.8 p.f leading.

7.a)	Describe any two methods of starting the synchronous motors.

b)	The input to an 1100v, 3phase star connected synchronous motor is 60A.The 	effective resistance and synchronous reactance per phase is 1 ohm and 30 ohm 	respectively. Find the power supplied to the motor and the induced e.m.f for a 	power factor of 0.95 leading.

8.a)	Discuss the function of an ac tachometer.  Explain its construction and 	operations.

b) 	What are the applications of stepper motor and synchro’s?

***     ***     ***

Code No.220402

Set No.

2

II-B.Tech. II-Semester Examination April/May 2003

ELECTRICAL TECHNOLOGY

(Common to Computer Science and Engineering, Computer Science and Information Technology, Electronics and Communication Engineering, Electronics and Computer Engineering, Computer Science and Systems Engineering and Electronics and Telematics)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the basic principle of operation of a D.C. Generator?

b)	Discuss the reasons for failure of self excitation, and suggest the remedies.

c)	A 250V d.c. shunt machine has line current of 80A. It has armature and field 	resistances of 0.1 and 125 respectively. Calculate power developed in 	armature when running as i) generator  ii) motor.

2.a)	With  a neat sketch, explain the working of a 3-point starter for d.c. shunt motor.

b)	A 6-pole, 250V series motor is wave-connected. There are 240 slots and each slot has 4 conductors. The flux per pole is 1.75X10-2 Wb when the motor is taking 80A. The filed resistance is 0.05, the armature resistance is 0.1 and the iron and frictional loss is 0.1kw. Calculate i) Speed, ii) bhp and iii) shaft torque.

3.a) 	Draw the phasor diagram of transformer under loaded conditions.

b) 	Explain the principle of working of 1-phase transformer on no-load conditions.  	Also explain the nature of no-load current.

4.a) 	Explain how the equivalent circuit parameters can be obtained from open circuit and short circuit tests.

b) 	A 300 kVA, 11000 / 440 V, single phase, 50 Hz, transformer gave the following test results. Open circuit test on LV side a normal voltage and frequency input 1.3 kW, 21.1 amp; short circuit test HV side with voltage 600 V, input 2.80 kW, 15.0 amps. Calculate the efficiency and regulation for full load at 0.8 p f lagging.

5.a) 	How the torque-speed characteristics of a motor are modified,  if rotor resistance is increased.

b) 	(i)A 3-phase, 6-pole, slip-ring induction machine is directly driven from the shaft by a 4-pole 3-phase synchronous motor. If stator of both the machines are given a 50 Hz supply, what frequencies are available at the rotor slip-rings of the induction machine.

(ii) A 3-phase, 50 Hz, induction motor has a starting torque which is 1.25 times full-load torque and a maximum torque which is 2.50 times full-load torque. Neglecting stator resistance and rotational losses and assuming constant rotor resistance, find (i) the slip at full-load (ii) the slip at maximum torque and (iii) the rotor current at starting in per unit of full-load rotor current.

(Contd…2)

Code No.220402				-2-			Set No.2

6.a)	Explain the constructional details of a synchronous machine giving the reasons 	for making two different types of rotors.

b)	A 100KVA, 3000V, 50 Hz,3-phase star connected alternator has effective armature resistance of 0.2 ohm. A field current of 40 A produces short circuit current of 200A and on open circuit e.m.f of 1100V(line value). Calculate the full load percentage regulation at a power factor of 0.8 lagging.

7.a)	Explain auxiliary motor starting of synchronous motors.

b) 	A 3-phase alternator is rated at 5 KVA, 110V, 26.3A, 50 Hz and 1200 r.p.m. The stator resistance between terminals as measured with dc is 0.2 ohm. With no load and rated speed the stator line voltage is 160V for a field current of 4A.At rated speed, the short circuit stator current per terminal is 50A for a field current of 4A.compute voltage regulation of alternator at 0.8 p.f. Lagging. Using synchronous impedance method.

8.a)  	What is a stepper motor?  Enumerate its advantages and applications.

b) 	With neat sketch, explain the working principle of shaded-pole single-phase 	Induction Motor.

***     ***     ***

Code No.220402

Set No.

3

II-B.Tech. II-Semester Examination April/May 2003

ELECTRICAL TECHNOLOGY

(Common to Computer Science and Engineering, Computer Science and Information Technology, Electronics and Communication Engineering, Electronics and Computer Engineering, Computer Science and Systems Engineering and Electronics and Telematics)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Mention the reasons for compounding D.C. Generator. Neatly sketch and explain the external characteristics of a D.C. Compound Generator.

b)	A short shunt compound generator delivers a load current of 30A at 220V and has 	armature, series field and shunt field resistances of 0.05, 0.03 and 200 	respectively. Calculate the induced emf and the armature current. Allow 1.0V per 	brush for contact drop.

2.a)	Explain with a neat sketch the principle of operation of a D.C. Motor.

b)	A 4-pole series motor has 944 wave-connected armature conductors. At a certain load the flux per pole is 34.6 mWb and the total mechanical torque developed is 209 Nm. Calculate the line current taken by the motor and the speed at which it will run with an applied voltage of 500V. Total armature resistance is 3.

3.a) 	Draw the phasor diagrams of a transformer: (i) on open circuit (ii) on load indicating     in each case what the various phasors represent.

b) 	Derive the emf equation of 1-phase transformers and evaluate the emf/turn, if flux is 0.01wb, at a frequency of 50 Hz.

4.a) 	Describe the method of calculating regulation and efficiency of a single-phase

transformer by open circuit and short circuit tests.

b) 	Obtain the equivalent circuit of a 200 / 400 V, 50 Hz, single phase transformer 	from the following test data:     O.C. Test: 200 V, 0.7 A, 70 W – on L.V. Side;

S.C. Test: 15 V, 10 A, 85 W – on H.V. Side.

Also calculate the secondary voltage when delivering 5 kW at 0.8 lag. V1=200V.

5.a) 	In a polyphase induction motor show that the rotor must rotate in the direction of that of the rotating magnetic field.

b)	A 3-phase squirrel cage induction motor has a rotor starting current of 6 times its full-load value. The motor has a full-load slip of 5%. Determine (i) the starting torque in terms of full-load torque; (ii) the slip at which maximum torque occurs; and (iii) maximum torque in terms of full-load torque.

(Contd…2)

Code No.220402				-2-			Set No.3

6.a) 	How emf is induced in the armature of alternator. What are the facctors that cause a change of the alternator terminal voltage?

b) 	A 3phase, 10 pole star connected alternator runs at 700 r.p.m. It has 120 stator slots with 10 conductors per slot, if the flux per pole is 0.056 wb. Determine the phase and line induced e.m.f.

7.a)	Explain the working principle of the synchronous motor on no load and on load with the help of phasor diagrams.

b) 	A 3phase, 440V, 50HZ Star connected synchronous motor develops 7.4KW. The effective resistance per phase of the stator winding is 0.5 ohm. The motor operates at power factor of 0.75 lagging. Iron and mechanical losses amount to 500W and the excitation loss is 650W. Calculate (i) armature current (ii) efficiency of the motor.

8.a) 	Give the description of A.C tachometer and mention its applications.

b) 	Write a short note on shaded pole type servo-motor.

***     ***     ***

Code No.220402

Set No.

4

II-B.Tech. II-Semester Examination April/May 2003

ELECTRICAL TECHNOLOGY

(Common to Computer Science and Engineering, Computer Science and Information Technology, Electronics and Communication Engineering, Electronics and Computer Engineering, Computer Science and Systems Engineering and Electronics and Telematics)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the procedure to find critical field resistance and critical sped of a D.C. Machine. Also explain the importance of these terms.

b)	The magnetization characteristics of a shunt generator at 1,000rpm is as follows:

O C Volts		62.5	107.5	155.0	196.5	231.0	256.0	275.0	283.0

Field Amperes 	1.0	2.0	3.0	4.0	5.0	6.0	7.0	8.0

Estimate the no-load terminal voltage of the machine when run at 800rpm with 20 	ohms field circuit resistance.

2.a)	From the first principles, derive an equation for torque developed in a D.C. Motor.

b)	A 20kw, 250V d.c. shunt generator has armature and field resistances of 0.04and 200 respectively. Determine the total armature power developed when working    (i) as generator delivering 20kw output and (ii) as a motor taking 20kw input.

3.a) 	Draw the phasor diagram of a 1-phase transformer with lagging loads. Explain.

b) 	A single phase, 440/220V, 5 KVA, 50Hz transformer, draws a no-load current of 0.8A at a.p.f of 0.25 lag on h.v side.  Determine the magnitising component, careless component, emf / turn on either side and the primary current when the secondary current is 18A at a p.f. of 0.9 lag.

4.a) 	Derive the expression for approximate regulation for a transformer operating on lagging power factor load.

b) 	A 50Hz, 1-phase transformer has a turn ratio of 6. The resistances are 0.9 and 0.03, and the reactances are 5 and 0.13 per high voltage and low voltage windings respectively. Find (i) the voltage to be applied to the high voltage side to obtain full load current of 200A in the low voltage winding on short circuit, (ii) the power factor on short circuit.

5.a) 	Explain the different losses in a 3-phase induction motor.

b) 	In a 3-phase induction motor, the stator reactance equals the rotor reactance at standstill while each resistance is one-fourth of this value. If the motor develops 220 Nm at 3% slip, what will be its (i) starting and (ii) Maximum torque?

Ignore no-load current.

(Contd…2)

Code No.220402				-2-			Set No.4

6.a) 	Explain neatly the constructional features of an alternator.

b) 	A 3 ph, 10-pole alternator has 2 slots per pole per phase on its stator with 10 	conductors per slot. The air gap flux is distributed sinusoidally and equals to 	0.005wb.The stator has a double layer winding with a coil span of 150 electrical 	degrees. If the alternator is running at 600r.p.m, Calculate the e.m.f generated per 	phase at no load.

7.a)	Explain the principle of working of synchronous motor.

b) 	A 3phase, 1385v star connected synchronous motor having synchronous reactance 	of 2ohm per phase and negligible resistance takes an input of 207.8kw with an 	induced e.m.f of 916.5v per phase. Calculate the motor line current and its power 	factor.

8.a) 	How is the direction of rotation reversed for each of the following.

- Resistance –start induction run motor and

- Capacitor start induction run motor.

b) 	Explain the construction and operation of split – phase induction motor.

***     ***     ***