Code No.220353

OR

II-B.Tech.  II Semester Supplementary Examinations April/May 2003

Thermal Engineering - I

(common to Mechanical Engineering, Production Engineering,  Mechatronics)

Time: 3 hours						      Max.Marks:70

Note:	(i)   Answer any FIVE questions

(ii)  All questions carry equal marks

(iii) Thermodynamic property tables and charts may be used, if required.

(iv)  Any data, needed, may suitably be assumed and stated.

---

1.a)	For a vapour power plant it is the Rankine cycle and not the barnot cycle, that is the practical cycle.  Explain.

b)	Explain the function and the purpose of the following as connected to a steam boiler. (i) fusible plug, (ii) injector, (iii) feed check valve and  (iv) super heater.  State whether each is a mounting or an accessory.

c)	A coal fired boiler raises steam at the rate of 680kg/hour at a gauge pressure of 10 bar used with a superheat of 55oC.  The boiler burns coal at the rate of 90kg per hour.  The calorific value of the coal is 29100 kJ/kg.  The temperature of feed water is 42oC and the specific heat of superheated steam is 2.1 kJ/kg-K Determine the equivalent evaporation and the efficiency of the boiler.

2.a)	Explain what regeneration and reheating are and how they improve the performance of Rankine cycle.

b)	Explain the advantages and disadvantages of water tube boilers when compared to fire tube boilers.

c)	Explain what balanced draught is as related to a boiler.  Determine the quantity of air used per kg of coal burnt in a boiler having a chimney of 32m height producing a draught of 2cm of water, when the temperature of the chimney gases is 370oC and that of the outside air is 29oC.  The density of outside air is 28.84 x 10-6 N/CC.

3.a)	Explain critical pressure and supersaturated flow as related to steam nozzles.  What is the effect of friction on the flow of stream through a nozzle?

b)	Explain the salient features of jet and surface condensers and state where each type is preferred.

c)	Steam enters a condenser at 35oC.  The vacuum in the condenser is 70cm of Hg, while the barometer reads 760mm of Hg.  Boiling water at 22oC enters the condenser and leaves at 34oC.  Determine the vacuum efficiency and condenser efficiency.

Contd…..2.

Code No.220353				:: 2 ::					OR

4.a)	With a simple sketch explain the salient features of De Laval turbine.  What are its limitations?

b)	Considering a four stage reaction turbine, sketch the pressure and velocity variation along the flow passage.

c)	In an impulse turbine there are two rings of moving blades separated by fixed blades.  The tips of moving blades are inclined at 30o to the plane of motion.  The velocity of steam issuing out of the nozzle is 620m/s and the mass flow is 5kg/s.  Steam is discharged axially.  Assuming no friction losses determine the blade speed, the diagram efficiency and the power developed.

5.a)	What are the uses of compressed air?  As related to compressed air, explain what the following are (i) Fan, (ii) Blower, (iii) Positive displacement compressor and (iv) Dynamic compressor.

b)	What are the essential elements of a reciprocating air compressor?  What is meant by free air delivered (FAD) and why is it necessary to state the same?

c)	A single cylinder single acting reciprocating compressor delivers 7 cubic meters of free air per minute at a mean piston speed of 182 meters per minute.  Air is compressed from an initial pressure of 1 bar to a final pressure of 7 bar.  The index of compression and expansion is the same and is 1.25.  Determine the volumetric efficiency, bore, stroke and speed of the compressor.  Take stroke of the piston as 1.5 times the diameter of the cylinder bore, clearance volume as 1/18th of the stroke volume and suction conditions to be the same as atmospheric conditions.

6.a)	What is multistage compression?  What are its advantages and disadvantages?

b)	Explain the salient features of a centrifugal compressor.  What are slip factor, power input factor and prewhirl as related to centrifugal compressor?

c)	A 150 kW power motor drives an axial flow compressor which compresses air from a state of 1 bar pressure and 30oC temperature to a final state of 4 bar pressure.  Determine the volume of air at entrance conditions handled by the compressor.  Assume an isentropic efficiency of 85 percent.

7.a)	Explain the working of air cycle refrigeration system with a simple schematic, P-v and T-s diagrams.  What are the differences between the above cycle and the reversed barnot cycle?

b)	What are the properties to be possessed by an ideal refrigerant?

c)	A 15 TR (tons of refrigeration) capacity vapour compression system using     R12 as the refrigerant works between -8oC evaporator and 30oC condenser temperature.  Saturated condensed liquid enters the throttle value while saturated vapours from the evaporator enter the compressor.  The compression is isentropic.  Determine the COP and power required.  Take cp of the vapour R 12 as 0.733 kJ/kg-K.

8.a)	With a neat sketch showing the principal components, explain the working principle of vapour absorption refrigeration system.

b)	What are the requirements of air conditioning?  With a neat schematic layout of a typical air conditioning plant, explain the salient features.  What are dew point temperature and relative humidity as applied to moist air.

@@@@@