Code No. 210252

OR

II B.Tech I Semester Supplementary Examination June, 2003.

ELECTRICAL CIRUCITS

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks:70

Answer any Five questions

All questions carry equal marks

- - -

1.	The current after t=0 in single circuit element is as shown in the following figure. Find the voltage across the element at t=6.5 s, if the element is

(a)  10 K (b) 15mH  (c) 0.3nF with Q(0) = 0

Also draw the waveforms of the voltage in each case.

2.a)	Find the voltage value in the following network using Kirchhoff’s laws.

b)	Write the dot convention for coupled circuits. Obtain the dotted equivalent for the circuit shown below and find the equivalent inductive reactance.

Contd…2.

Code No.210252		                -2-					OR

3.a)	Sinusoidal voltages across two circuit elements in series have maximum values 10V and 16V, while their sum has a maximum value of 21V. Give complete trigonometric expressions for the three voltages.

b)	A voltage wave has the following shape.

i)  find the average and effective values of the voltages.

ii) If the voltage of part (i) is applied to a 50 ohm resistor find the power 			    dissipated in watts.

4.a)	Obtain the phasor voltage VAB in the two branch parallel circuit of the following figure.

- A capacitor is in series with a 2 ohm resistor and carries a current of 10A to store an energy of 0.25 joules at 100V a.c. supply. What is the supply frequency ? And which is the current and phase difference, if the frequency become 10,000 Hz?

5.	A coil of  resistance 20 ohms and inductance 100 mH is connected in series with a capacitance of 40 F across 100V, 50 Hz a.c. supply. Calculate (i) magnitude of current  (ii) P.F  (iii) phase angle (iv) voltage across each element (v) Draw a neat phasor diagram showing all quantities.

6.	A series R-L-C circuit with R=3K, L=10H and C=200 F has a constant voltage source V=50 volts applied at t=0. (a) Obtain the current transient, if the capacitor has no initial charge (b) Sketch the current and find the time at which it is a maximum.

Contd…3.

Code No.210252		                -3-					OR

7.	An R-L circuit with R=300  and L=1H has voltage v=100 cos (100t + 45o) applied by closing a switch at t=0. Obtain the resulting current for t>0 and sketch it.

8.	In the network shown below the switch is closed at t=0; and there is no initial charge on either of the capacitors. Find the resultant current i (use laplace transforms).

-x-