Code No: 211003

Set No.

1

II-B.Tech.  I-Semester Supplementary Examination - June 2003

ELECTRO MAGNETIC THEORY

( Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering )

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Derive an expression for the field intensity due to uniform surface charge s.

b) 	Three point charges 3,4,5 coulombs are situated in free space at the three corners 	of an equilateral triangle with side 5 Cm. Find the energy density within the 	triangle.

2.a)	What is meant by dielectric polarization. How is it effected by changing the 	dielectric material.

b) 	A parallel plate capacitor has its dielectric changed from r1=2 to r2 =6.  It is 	noted that the stored energy remains fixed. Examine the changes if any in V, C, 	D, E, Q.

3. 	A single phase circuit comprises of two parallel conductors A and B, 1cm radius 	and 1m apart. The conductors carry +10A and –10A respectively .Determine the 	magnetic field intensity at the surface of each conductor and also in the space 	exactly mid way between A and B.

4.a)  	State  and prove the boundary   conditions  on the tangential   component of             	time varying   E-field   and the normal  component  of time varying   D-field            	at  the  surface of   discontinuity.

b)	A square loop  with  sides  20cm by  20cm is located in free space  adjacent  to a 	straight conductor  which  carries a  sinusoidal  current  of  0.5 A   at  5KHz. Two 	of the sides  of the loop  are  parallel  to the conductor  and located at  5 cm and 	25 cm  from the conductor. If a small gap  is  introduced  into the loop,  determine 	the induced  voltage  across   the gap   and  its proper polarity.

5.a) 	What is wave polarization? Explain the different types of polarization with 	analytical treatment.

b) 	A 100 V/m plane wave of frequency 300 MHz travels in an infinite, loss less 	medium having r = 1, r = 9,  = 0 mhos/m.. write the complete time domain 	expressions for the E and H field vectors.

(Contd…)

Code No: 211003.			. . 2 . .				Set No: 1

6.a) 	Explain how materials are classified depending on relaxation time.

b) 	A large copper conductor r = 1, r = 1,  = 5.87 x 10 6 mhos/m. supports a 50 Hz

plane wave . compute , , ,  and phase velocity of propagation.

7.a) 	Explain the terms reflection and refraction.

b) 	Explain the reflection of plane waves by a perfect conductor for normal incidence.

8.a) 	Explain the significance of  Poynting vector .

b)	An incident plane electromagnetic wave of 10 MHz and having Ex=3.0 V/m and 	traveling along positive Y-axis strikes an aluminium sheet at normal incidence. 	Calculate the average power densities of incident and reflected waves.

~ ~ ~ ~

Code No: 211003

Set No.

2

II-B.Tech.  I-Semester Supplementary Examination - June 2003

ELECTRO MAGNETIC THEORY

( Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering )

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	 State and explain the concept of uniqueness theorem.

b) 	An uncharged infinite sheet of metal of thickness “t” is inserted between two 	infinite conducting plates separated by a distance of “d”. Find the capacitance of 	the condenser per unit area with and without metal sheet.

2.a)	State and explain boundary conditions for electric fields.

b)	Five equal point charges Q=20 nC, are placed at X=2,3,4,5 and 6 calculate 	potential at origin.

3. 	Derive a general expression for the field B at any point along the axis of a 	solenoid and sketch the variation of B from point to point along the axis.

4.a)  	What are the  transformer  and motional  electromotive forces  (emfs) in the  	context of  Faraday’s law ?

b)  	In a medium  characterized  by  = 0,  = 0,  =0 and  E=20 sin (108 t - z ) ay 	(V/m) calculate   and H using Maxwell’s equations.

5.a) 	For a uniform plane wave Hy = 20 A/m in free space. The wave propagates in +z 	direction at a frequency300 MHz. Find the phase velocity, wave length, intrinsic         	impedance and the magnitude of E field.

b)	If r = 1, r = 5,  =  5.78 x 107 mhos/m for a certain medium , find the relaxation 	time for the medium.

6.a) 	Define the term intrinsic impedance of the medium, hence derive an expression 	for the same for a conducting medium, in terms of medium constants.

b) 	Determine the loss per kilometer for a plane wave propagating in certain medium	at a frequency 2.5 MHz , if r = 1, r = 12,  =  4.5 x 10-5 mhos/m for the medium

7. 	Explain the reflection of plane waves by a perfect conductor for oblique 	incidence.

8.	Find the variation of an average Poynting vector and instantaneous Poynting 	vector for a plane wave within a good conductor and interpret the results.

~ ~ ~ ~

Code No: 211003

Set No.

3

II-B.Tech.  I-Semester Supplementary Examination - June 2003

ELECTRO MAGNETIC THEORY

( Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering )

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a) 	Discuss the various methods of solving the static field boundary value problems.

b) 	Obtain by means of Laplace’s equation the potential distribution between two 	concentric spherical conductors separated by a single dielectric. The inner 	conductor of radius ‘a’ is at potential V0 and the other conductor of radius ’b’ is at 	zero potential.

2.a)	Show that the force on a point charge anywhere within a circular ring of uniform 	charge density is zero, provided the point charge remains in the plane of the ring.

b) 	It is required to hold three equal point charges of +Q each, in equilibrium at the 	corners of an equilateral triangle. Calculate the point charge, which will do this if 	placed at the center of a triangle.

3. 	Two narrow circular coils A and B have a common axis and are placed 15cm 	apart.  Coil  A  has 10 turns of radius 5cm with a current of 2A passing through it . 	Coil B has a single turn of radius of 8cm. If the magnetic field  at the center of 	coil A is to be zero . What current must be passed through coil B.

4.a)  	State   the   boundary   conditions  on  electromagnetic  fields  at   the  surface  of

discontinuity.

b) 	Given  the time-varying  magnetic field B=(0.5ax +  0.6ay - 0.3az) cos5000t 	(Tesla)  and  a  square  filamentary  loop with  its corners  at  (2,3,0), (2,-3,0), 	(-2,3,0), and (-2,-3,0),  find  the time varying   current   flowing  in the loop if  the 	total  loop resistance   is  400K.

5.a)	What is a uniform plane wave?

b) 	Prove that uniform plane electromagnetic waves are transverse.

6.a)	Define the terms for a dielectric

i)	Dissipation factor.		ii)	Power factor.

b)	Calculate the intrinsic impedance , the propagation constant , and the wave 	velocity 'u'  for a conducting medium in which  = 58mhos/m, r=1 at a 	frequency f=100MHz.

(Contd…)

Code No: 211003.			. . 2 . .				Set No: 3

7.a) 	Explain the reflection of plane waves by a perfect dielectric for normal incidence.

b) 	Explain law of sines.

8.a) 	Describe  the  Poynting vector and discuss its interpretation for the case of a static	point charge Q located at the center of a small loop of wire carrying direct current.

b) 	Assuming current density constant over the conductor cross section in the 	Question (a), find the Poynting vector within the wire and interpret this in terms 	of the distribution of dissipation.

~ ~ ~ ~

Code No: 211003

Set No.

4

II-B.Tech.  I-Semester Supplementary Examination - June 2003

ELECTRO MAGNETIC THEORY

( Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering )

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	State and explain Gauss law. Derive an expression for the potential distribution 	about long parallel wires.

b) 	Prove that the Electric field strength at any point in a homogeneous, isotropic 	medium is just the negative of the potential gradient at that point. Justify how this 	relation is simpler in finding the electric field due to the charges.

2.a)	What are the important properties of the potential in a charge free region that can 	be obtained from the Laplace’s equation.

b) 	The region between two concentric right circular cylinders contains a uniform 	charge density   use Poisson’s equations to find V.

3.	A steady direct currnet 1 Amps flows in a wire bent in the form of a square of side 	“2a”. Assume 	that the ‘z’ axis passing through the center of the square is normal 	to the plane of the square. Find the magnetic field intensity H at any point 	on the 	z axis.

4.a)  	Explain  the inconsistency   of  Ampere’s   law   in time-varying  fields.

- The  electric vector E of  an electromagnetic  wave in free  space  is  given by  the  expressions

Ex  = Ez = 0  ;  Ey =  A cos (t – z/c)

Using  Maxwell’s  equations   for  free-space  conditions  determine expressions

for the  components  of the magnetic  vector  H.

5.a)	What do you mean by wave polarization? Explain different types of 	polarizations.

b)	A wave is resultant of two elliptically polarized waves one with components

Ex = 5cost, Ey=3sint and the other with components Ex=sint,

Ey=sin(t+45o). For the resultant wave draw the polarization ellipse and 	determine the axial ratio.

(Contd…)

Code No: 211003.			. . 2 . .				Set No: 4

6.a)	Explain EM wave propagation in a lossless medium.

b)	A charged particle travels at a velocity u in the field of a plane EM wave in free 	space, the velocity of particle being parallel to the direction of propagation of the 	wave. Show that the net transverse force is in the direction of E, but that it tends 	to zero as the velocity of the particle approaches c.

7.a) 	Explain the reflection of plane waves by a perfect dielectric for oblique incidence.

b) 	What is the time average power flow and instantaneous power flow associated 	with a uniform plane wave in a material medium.

8. 	Interpret the Poynting vector about a parallel-plate capacitor charged from zero to

some final charge Q. Repeat for an inductor in which current builds up from zero 	to some final value. Repeat for each of these cases as charge and current is made 	to decay from a given value to zero. For the inductor, use a straight section of 	wire as example.

~ ~ ~ ~