Code No: 210454

OR

II-B.Tech. I-Semester Supplementary Examinations June, 2003

ELECTRICAL SCIENCE

(Common to Electronics & Communication Engineering, Chemical Engineering, Metallurgy &Material Technology, Computer Science & Information Technology, Electronics & Instrumentation Engineering, Bio-Medical Engineering, Electronics & Control Engineering)

Time: 3 hours						                    Max. Marks: 70

Answer any five questions

All questions carry equal marks

- - -

1.a)	State and explain Kirchoffs Laws.

b)	Find the Equivalent Resistance between A and B.

2.a)	Briefly explain how alternating voltages are generated.

b)	A square coil of 10cm side and 100 turns is rotated at a uniform speed of 1000 revolutions per minute, about an axis at right angles to a uniform magnetic field of o.5wb/m2. Calculate the instantaneous value at induced electromotive force, when the plane of the coil in i)   at right angles to the field 	ii)   in the plane of the field.

3.a)	What do you understand by power factor? How can it be improved?

b)	In a series-parallel circuit the parallel branches A and B are in series with C. The impedances are ZA = (4+j3);	ZB = 4-j16/3);	Zc=(2+j8)0hms. If  the current IC=(25+j0), draw  the complete phasor diagram determining the branch currents and voltages and the total voltage. Calculate the complex power for each branch and the whole circuit.

4.a)	Give a brief note on lap and wave windings.

b)	A lap connected d.c. generator has 8 poles and 120 slots with 8 conductors in each slot. If the flux/pole is 0.035Wb(I)find the emf generated when the speed is 600rpm.(ii) What should be the speed of rotation if the induced emf is to be 500V?

5.a)      Why is the speed of a shunt motor practically  constant ?

b)	Draw the characteristics of shunt and series   motors.

Contd…2

Code No: 210454				-2-				         OR

6.a)	On what factors the induced EMF  in the transformer windings depends. Justify          	the answer with appropriate derivation.

b)	A double wound 1- phase transformer is required to step down from 1900V to        	240V, 50Hz . It is to have 1.5V per turn. Calculate the required number of turns        	on the primary and secondary windings respectively. The peak value of flux        	density is required to be not more than 1.5 wb/m2 . Calculate the required cross        	sectional area of the steel core. If the output is 10KVA, calculate the secondary        	current.

7.a)	Draw the slip torque characteristic of an induction motor and mark the operating 			region.

b)	A 3- phase , 6 pole , 50Hz induction motor has a slip of 1% at no load and 3%  at full load.   Find

i) Synchronous speed		ii) No load speed

iii) Full load speed		iv) Frequency of rotor current at standstill

v) Frequency of rotor current at full load.

8.a)	Sketch and describe the construction of a moving coil ammeter and give the principle of operation.

- A moving coil instrument gives full scale deflection with 15mA and has a resistance of 5W. Calculate the resistance of the necessary components in order that the instrument may be used as i) a 2A – Ammeter ii) a 100V voltmeter.

^^^