Code No.220255

OR

II-B.Tech. II-Semester Supplementary Examination January - 2003

ELECTROMECHANICS – II

(Electrical and Electronics Engineering)

Time:3 hours								Max.Marks:70

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	With circuit diagrams, explain the procedure for conducting OC and SC tests on the given single-phase transformer to predetermine its regulation and efficiency.

- A 10 KVA, 450/120 Volts, 50 Hz transformer gave the following test results:-

O.C.test (L.V.side): 120 volts, 4.2Amps, 80 Watts.

S.C.test (H.V.side): 9.65 volts, 22.2 Amps, 120 watts.

Calculate (i) Approximate regulation at 0.8 power factor lagging and

ii) Efficiency at half full load at 0.8 pf leading.

2.a)	What are the various losses occurring in a transformer on load? Discuss their dependance on the load, applied voltage and frequency.

- Deduce the condition for maximum efficiency of a transformer.

- A transformer has maximum efficiency of 0.98 at 15 KVA at unity power factor. During the day it is loaded as follows:

2kw at 0.5 pf for 12 hours, 12kw at 0.8 pf for 6 hours and 18kw at 0.9 pf for 6 hours. Find the all-day efficiency.

3.a)	State the conditions of parallel operation of two 1-phase transformers.

b)	Two 1-phase transformers, one at 300KVA and the other 200KVA operate in parallel and supply a load of 400 KVA at 0.8 pf lagging. Their respective resistances and reactances are (1% and 6%) and (12% and 4.8%) respectively. Calculate the loading and p.f of each transformer.

4.a)	A 3-phase step-up transformer is connected to 500 3 volts mains and takes a current of 240 Amps. Calculate the secondary line voltage, line current and output at unity power factor for star/delta connection. Assume the turns ratio to be 12.

b)	A scott connected transformer set supplies two single phase furnaces A and B, each of 120 volts from a 3-phase 6600 volts system. If the furnace connected to teaser transformer takes 300 kw at unity p.f and the other connected to main transformer takes 495 kw at 0.8 p.f  lagging, determine the line currents taken from the 3-phase mains. Neglect losses.

(contd…2)

Code No.220255			-2-			OR

5.a)	Derive an expression for the torque of an induction motor and obtain the condition for maximum torque.

b)	A 400V, 3-phase star connected induction motor has an equivalent T-circuit consisting of a stator impedance of (1+j2), an equivalent impedance of               (1.2 + j1.5)  at stand still and a magnetizing branch impedance of (4+j40)  per phase. Determine the current, efficiency, power factor and output when slip is 5%. Assume frictional losses of 250 w.

6.a)	Develop from first principles an equivalent circuit that can be used to predict the steady state performance of a 3-phase induction motor.

b)	The power input to a 3-phase induction motor is 100kw. The stator losses are equal to 2kw. Find the mechanical power output and rotor copper losses, if the motor runs with a slip of 4%. Mechanical losses are equal to 2.5kw.

7.a)	Explain the principle of operation of an induction generator. How is it made self excited?

b)	A 3-phase squirrel cage induction motor has a short circuit current equal to 5 times the full load current. Find the starting torque as a percentage of full load torque if the motor is started by i) Direct switching	(ii) Star delta starter and          (iii)  An auto transformer the starting current in case is limited to 2.5 times the full load current and the full load slip is 4%.

8.	Write short notes on any TWO of the following:

- Auto transformer

- Buchholtz relay

- Schrage motor.

***     ***     ***