Code No: 220353

### OR

# II B.Tech. II Semester Supplementary Examinations January,  2003

# THERMAL ENGINEERING - I

(Common to Mechanical Engineering, Production Engineering, Mechatronics, Mechanical Manufacturing Engineering)

Time: 3 Hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.  a)	Sketch a layout of a steam power plant and explain its working.

- Sketch Bobcock and Wilcox boiler and explain its working.

- What is meant by equivalent evaporation?  Explain its importance in boilers.

2.  a)	Sketch a pressure gauge of Bourdon type and explain its working.

- Differentiate boiler Accessories and Mountings.

- Explain the methods of improving steam power plant.

3.  a)	Analyse flow through nozzles.

- What is the criterion to decide nozzle shape?  Explain.

- What is Wilson Line?  Explain.

4.  a)	What are requirements of condensing plant?  Discuss.

- What is Air Pump?  Explain its working?

- A simple steam turbine nozzle ring is supplied with 180 kg/hr of dry  steam  at  7.5 kgf/cm2.  The exhaust pressure is 0.5 bar, nozzle angle is 20o, nozzle efficiency is 75%, Blade out let angle is 25o,  Blade speed is 225 m/sec,  Blade velocity coefficient is 0.75, power loss is 1.25 kW.  Calculate  (i) Net output at the shaft,  (ii) The dryness fraction of steam in exhaust passage when the speed is small.  (iii) Dryness of steam just as it leaves the turbine rotor blading.

5.  a)	Classify the air compressor and describe them.

- Sketch a Lyshola compressor and explain its working.

- A single stage, single acting reciprocating air compressor compresses 15 kg/min. of air from 150 kPa, 25oC to 700 kPa and delivers to a receiver.  Law of compression is PV1.25 = Constant,  Mechanical efficiency is 85%. Find the power input, neglecting the losses due to clearances, leakage and cooling.

6.  a)	Discuss Euler’s equation as applied to energy transfer between fluid and rotor.

- Discuss the velocity compounding of steam turbine.

- Discuss the methods to reduce rotor speed of a steam turbine.

Contd…..2.

## Code No: 220353 				:: 2 ::					OR

7.	In an aqua-ammonia absorption refrigeration system, heat is supplied to the generator, by condensing steam at 0.2 MPa, 90% quality.  The temperature to be maintained in the refrigerator is – 10oC and the ambient temperature is 30oC.  Estimate the maximum C.O.P. of the refrigerator.  If the actual C.O.P. is 40% of the maximum C.O.P. and the refrigeration load is 20 Tonnes, what will the required steam flow rate be?

8.	Write short notes on any FOUR of the following:

- Height of the Chimney.

- Under cooling

- Pressure compounding

- Multistage compression

- Vapour absorption Refrigeration.

@@@@@