Code No:221401

Set No:

1

II.B.Tech II-Semester. Supplementary Examinations. January,2003

ELECTRONICS DEVICES AND CIRCUITS

(Mechatronics)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Sketch the energy-band picture for

(i) an  intrinsic (ii) an n-type (iii) a p-type semiconductor.  Indicate the positions of the Fermi, the donor and acceptor levels.

b)	Explain the principle of converting an intrinsic into n-type and p-type semiconductors and describe the properties of each.

2.a)	Explain the V-I characteristics of a Tunnel diode with the help of energy band diagrams.  Give the applications of Tunnel-diode.

b)	In what range of current should a 4V zener diode be operated so that 1 mA change in current causes a maximum of 40 mv across the diode and the zener voltage does not fall below 3.9V when the temperature rise is 500C.

3.a)	Explain how transistor acts as an amplifier.

b)	Draw the circuit of a set of biasing circuit to the CE  amplifier and derive the stability factor S.

4.a)	Using the hybrid п-model determine  fT .

b)	Derive the expression for upper cut-off and lower cut-off frequencies.  Mention how fL and fH are affected if two or more stages are connected.

5.a)	Draw the n-channel FET.  Explain the working of FET with V-I characteristics.

b)	Explain how FET is used as voltage variable resistor.

6.a)	What is negative feedback?  What are the advantages?  Discuss.

b)	Derive the Input resistance and output resistance of voltage-series feed back amplifier.

7.a)	What is positive feedback?  Give two Bark Hausen conditions required in order for sinusoidal  oscillations to be sustained explain.

b)	Draw the circuit of a wien bridge oscillator.  Derive the expression for frequency of oscillation.  Will oscillations takes place if bridge is balanced.

8.	Write short note on the following;

a)	Hybrid п-model

b)	Colpilts oscillator

c)	Bridge rectifier.

********

Code No:221401

Set No:

2   2

II.B.Tech II-Semester. Supplementary Examinations. January,2003

ELECTRONICS DEVICES AND CIRCUITS

(Mechatronics)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Define donor and acceptor impurities.  Explain with energy band diagrams.

b)	Derive the diffusion and transition capacitance of a diode.

2.a)	Distinguish between zener breakdown and  Avalanche multiplications.

b)	Draw the circuit for full wave rectifier with п-section filter, explain its operation.  Derive the expression for the ripple factor.

3.a)	What is biasing?  Why biasing is required for transistor?  Explain.

b)	Derive the voltage gain, current gain, Input resistance, output resistance of a transistor using h-parameters.

4.a)	Discuss the effect of emitter bypass capacitor on the low frequency of an R-C coupled CE amplifier.

b)	A BJT having hie=1500 Ω, hfe = 100, hre = 2x10-5 and hoe = 25 nA/V is used as an emitter follower amplifier with RS=lKΩ and RL=1KΩ.  Determine for the amplifier Avs = Vo/Vs, AIS = Io/IS, Ri and Ro.  Derive the expressions used

5.a)	Explain FET as voltage variable resistor.  Draw the V-I characteristics of FET.

b)	For the JFET amplifier shown in fig.  below determine.

(i) Mid-band voltage gain AVS=V0/Vs

(ii) The lower 3-dB cut-off frequency fL

Assume gm = 5 millimhos,  rds = 20 KΩ

VOD

20K          1F

0.1 F	V0

10K



VS	500K	5K	50F

Contd….2

Code No:221401				-2-			Set No:2

6.a)	Discuss the effect of feedback on R in for all the four topologies.

b)	Identify the feedback topology  and derive the expression for Vo/Vi

VDD

Rd

VO

Ri

+

VS  -                                            RS

7.a)	What is Barkhausen’s criterion!  Explain its significance.

b)	With an internal circuit explain how oscillations are produced in a crystal.

8.	Write short note on the following:

a)	Zener regulation

b)	Tunnel phenomena

c)	Hartley’s oscillator.

*******

Code No:221401

Set No:

3   2

II.B.Tech II-Semester. Supplementary Examinations. January,2003

ELECTRONICS DEVICES AND CIRCUITS

(Mechatronics)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Draw the energy band diagrams for conductors, semiconductors and insulators?  And explain how they are classified based on energy band diagram.

b)	For the circuit shown below determine the current read by an ammeter and voltage d rop across each diode.

Io = 1 nA        Io = 1nA

Ammeter

10V

2.a)	Draw the circuit of a Bridge rectifier without filter and explain its operation.  Derive the expressions for ripple factor, regulation and rectifier efficiency.

b)	Design of full wave rectifier with an L-C filter to provide 9V d.c at 100 mA with a maximum ripple of 2%.

3.a)	Explain how transistor acts as an amplifier

b)	Determine AI,AV,RI and RO for a single stage CE amplifier with an emitter resistor.

4.a)	Describe the effects of emitter by-pass capacitor and coupling capacitors in CE amplifier on frequency response.  Derive an expression for lower 3-dB frequency due to ce only.

b)	Draw the hybrid п-model at high frequencies.  Derive the input and output

impedances.

5.a)	Explain the static characteristics of enhancement type and depletion type MOSFET.

b)	The amplifier stage shown uses an n-channel FET having IDSS = 1mA, VP = -1V.  If the quiescent drain to  ground voltage is 10V.  Find R1

Contd…..2

Code No:221401				-2-			Set No:3

24 V

56 K

R1

1MΩ

4K

6.a)	How does negative feedback affect RO?  Discuss with reference to all the four topologies.

b)	Derive the expression for Avf, AIf, Rif, Rof for current shunt feedback.

7.a)	Derive the expression for frequencies of oscillation of phase-shift oscillator using an FET.

b)	Explain how the oscillations can be sustained in a crystal.

8.	Write short note on the following:

a)	Tunnel diode

b)	R-C coupled amplifier

c)	Effect of Band width in cascade amplifiers.

********

Code No:221401

Set No:

4   2

II.B.Tech II-Semester. Supplementary Examinations. January,2003

ELECTRONICS DEVICES AND CIRCUITS

(Mechatronics)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Draw the typical energy band diagrams for:

(i) Conductors (ii) Semi-conductors (iii) Insulators and explain

b)	Differentiate between Diffusion capacitance and Transition capacitance of P- N Junction diode and obtain the expression for Transition capacitance.

2.a)	Explain tunneling phenomena in a diode and  it’s characteristics.

b)	The avalanche diode regulates at 50V over a range of diode currents from 5 to 40 ma.  The supply voltage V=200.  Calculate R to allow voltage regulation from a load current IL = O up to Imax, the maximum possible value of IL.  What is I max?

R

IL

+

V	            VL                          	RL

-

3.a)	Define peak inverse voltage and what is it’s value in full-wave rectifier circuit.  And also, obtain expression for D.C load voltage.

b)	Design a bridge rectifier power supply using L-section filter to give a D.C output of 24V at 100mA with a ripple not exceeding 0.05%.

4.a)	Compare the performance of CB,CC and CE amplifiers with reference voltage gain, current gain, input resistance and output resistance.

b)	A transistor has its parameters given by hie=2kΩ, hfe=50,hre=6x10--4 and hoe=25x10-6 Ω0.  The load resistance is 5 kΩ and the source resistance is 1 kΩ

Determine the voltage gain, current gain, input and output resistances of the amplifiers in CE configuration.  Derive the formula used.

5.a)	Use low frequency FET model  find expressions for AV and Zo of a source follower.

b)	A FET is connected in common drain mode with RS=1K, rd = 10k, =50.  Find AV.

Contd…..2

Code  No:221401				-2-			Set No:4

6.a)	What is feed back?  Draw the different feedback amplifiers?  Discuss the R in and R out of all feedback amplifiers.

b)	Derive the expression for transconductance in current-series feedback amplifier.

7.a)	Draw the circuit diagram of Hartley and colpit’s oscillator and explain is detail.

b)	Draw the electrical equivalent of a crystal sketch the reactance vs frequency function.  Over what portion of the reactance curve do we desire oscillations to take place when the crystal is used as part of sinusoidal oscillator?

8.	Write short note on the following:

a)	Darlington amplifier

b)	R-C coupled amplifier

c)	FET as a voltage variable resistor.

*********