Code No:220452

OR

II.B.Tech II-Semester. Supplementary Examinations. January,2003

ELECTRONICS CIRCUITS

(Common to  Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering, Electronics and Control Engineering)

Time: 3 hours.							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain the need for biasing a BJT.  Why do the operating point of an amplifier using BJT shift with temperature?

b)	Design a self-bias circuit for a BJT having VBE=0.6V and β0=60.  The desired operating point is VCE=4V and IC=2mA.  With S10.  Assume VCC=10V and

RC=2.2 KΩ  show the circuit diagram with all the component values.

2.a)	An amplifier uses a cascade of fun BJT stages.  Suggest suitable choice of configurations  for each stage, if it is to be a high gain amplifier with high input impedance and low output impedance.  Justify your answer.

b)	A BJT is found to have hie = 1.5 KΩ, hfe =100, hre=10-5 and hoe=25A/V.  It is used as a CE amplifier with Rs=1KΩ and RL=3 KΩ.  Determine for the amplifier AIS= Io/IS AUS=Vo/VS, Ri and Ro.  Derive the expressions used in your calculations.

3.	For the two stage amplifier shown in Figure1, determine AVS= V0/VS, Ri and Ro.  Assume that the transistors are identical and have hie=2KΩ, hre=100, hre=2.5x10-4 and hoe = 25 A/V as parameters.



Rs

5KΩ                        +VCC

+

1KΩ	  +VCC	V0

VS -                                              5KΩ

Ri	Ro

Figure:1

4.a)	Explain how would you arrive at the hybrid-п equivalent circuit model in CE configuration at high frequencies.  Explain the different parameters  involved in it.

b)	A BJT is found to have fT= 500 MHZ, rbb1=50Ω, gm=100 mm hos, hfe=95 and cb1C = 5 PF.  If is used as a CE amplifier with Rs=1KΩ and RC=2KΩ.  Determine for the amplifier (i) its mid-band voltage gain (volvs) and (ii) the upper 3-dB cutoff frequency ‘fh’.

Contd….2

Code No:220452                                -2-	OR

5.	An RC coupled JFET amplifier consists of 3 identical stages as shown in figure2.  Determine for the amplifer.

a)	Over-all mid-frequency voltage gain of the cascaded stage

b)	‘fL’ the lower 3-dB cutoff frequency of each stage and overall lower 3-dB cutoff frequency.

c)	‘fh’ the upper 3 dB cutoff frequency to each stage and over all upper 3dB cut off frequency.  Assume for JFET, gm=5mmhos, rds=10KΩ, cgs=5PF, cgd=4PF and Cds=3PF.

+VDD

10KΩ

.004F	.004F

+

+	500KΩ

Vi      500KΩ	                                         VO

2KΩ                 - 50F

Figure:2	-

6.a)	What is feedback the amplifiers?  Prove that negative feedback in amplifiers increases the band width and improves signal to noise ratio.



b)	For the feedback amplifier circuit shown in figure.3, determine (i) Avj=Vo/VS, (ii) Rif and (iii) Roj.  Assume that the transistors are identical and have hie=1.5KΩ and hfe=100 as parameters with hre=hoe=0

+VCC

Rs		         5KΩ	5KΩ

+

+            1KΩ                                                   1KΩ

Vo

VS-                                                 49KΩ        1KΩ

-

Rif	Rof

Figure:3

7.a)	What are the Barkhausen conditions of oscillations in electronic systems?  What are their significance?  What are the factors which effect the frequency stability of an oscillator?

b)	Draw the circuit diagram of a colpitts oscillator using a BJT and explain how the oscillations grow.  Show its small signal equivalent circuit.  Derive the expressions. For the frequency of oscillations and condition for sustained oscillations.

8.	Write short notes on the following:

a) Thermal run-away in BJT amplifiers (b) Biasing of JFETs and MOSFETs

(c) Went-bridge oscillator.

********