Code No.220254

OR

II B.Tech II Semester Supplementary Examinations January, 2003.

ELECTRICAL CIRCUITS-II

(Electrical and Electronics Engineering)

# Time: 3 hours							Max. Marks: 70

## Answer any Five questions

All questions carry equal marks

- - -

1.a)	Show that the three phase power measurement in a balanced or unbalanced, star or delta connected load can be obtained by using two waltmeters  with the help of circuit and phasor diagrams.

- A delta connected balanced 3-phase load is supplied from a 3-phase 400 volts, 50 Hz supply. The line current is 20 Amps, and the power taken by the load is 10 kw. Find (i) The impedances in each branch (ii) the line currents, power factor and power consumed if the same load is connected in star.

2.a)	A 3-phase, four-wire 400 volts, ABC system supplies a star connected load in which ZA = 10 ∠ooΩ, ZB = 15 ∠30oΩ and Zc = 25∠−30oΩ. Find the line currents and the neutral current .

- A 3-phase, 3-wire 400 volts, RYB system has a star connected load with

ZR = 20∠−30oΩ, ZY = 15∠0oΩ and ZB = 10∠30oΩ. Find the line currents and 	the neutral displacement voltage.

3.a)	Draw the dual of the circuit shown in the figure below.

- Explain the following terms with illustrations: (i) Link and (ii) Tree

In the network shown below, consider branches 1,3 and 4 as forming a tree. Identify the link currents with the loop currents and write a tie-set schedule for the network. Therefrom, obtain equilibrium equations on loop basis. R1= 1 ; R2 = R4 = R6 = 2; R3=R5= 3.

Contd…2.

Code No.220254		-2-						OR

4.a)	State and explain Therenin’s theorem. Discuss its significance and give the applications.

  - Use Principle of superposition to find the current in the j10 ohms reactor connected between terminals A and B in the circuit shown below.

5.a)	State and explain Maximum power transfer theorem and give its applications.

- Determine the resistance between the terminals PQ of the network shown below using star-delta transformation.

6.a)	Write the loop equations in matrix form for the coupled network shown below.

- Given L1= 0.2 H, L2 = 0.1 H,  M = 0.1 H and R=10 in the circuit shown below. Find I, for v1 = 142.3 Sin 100t.

Contd…3

Code No.220254			-3-						OR.

7.a)	Derive the necessary expressions to represent ABCD parameters in terms of hybrid parameters of a two port network.

- Determine the driving point impedance of the network shown below.

8.a)	Find Z-parameters for the two-part network shown below.

b)	For the two-port network shown below, determine the transfer admittance,        Y12 (5).

- x-