Code No:220552

OR

II.B.Tech II-Semester. Supplementary Examinations. January,2003

ELECTRONICS CIRCUITS AND LINEAR ICs

(Common to Computer Science and Engineering and Computer Science and Information Technology)

Time: 3 hours.							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Why is a bleeder resistor used in an LC filter?

b)	What are the advantages and disadvantages of п-filter?

c)	Is it possible for a power supply to have a regulation in excess of 100%?  How?

2.a)	What is the chief advantage of operating single ended amplifier in class B?

b)	Show that the input impedances of two transistor amplifiers in CE and CC configurations are same subject to the condition that RL=0.  Also show that the output impedances are also same subjected to the conditions that the input is open circuited.

3.a)	What are the advantages of push pull circuit over that of a single Transistor amplifier?

b)	Explain how negative feed back may turn into positive feedback at some frequency.

4.a)	Derive the condition for oscillation for a Hartley oscillator.

b)	How would you suppress low-frequency parasilcs in an oscillator?

5.a)	Draw the circuit diagram of summer and sign changer using ODAM and explain.

b)	The input to an operational amplifier differentiator with loop gain AV=A() is a ramp voltage  V=t.  Show that the output voltage

V0 (t) = AαRC1−A.

6.a)	What are the advantages and disadvantages of SMPS?

b)	Draw a typical switched mode power supply circuit and explain its operation.

7.a)	Explain the application of PLL as a demodulator for AM and FM signals.

b)	Discuss the design considerations for the sample and hold circuits using an

Op-Amp.

8.	Write notes on:

a)	Transducer amplifier

b)	MOSFET biasing circuit

c)	IC voltage regulator.

**********