Code No: 220252

### OR

# II B.Tech. II Semester Supplementary Examinations January,  2003

# THERMO DYNAMICS

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.  a)	What do you mean by reversible and irreversible process?  Give examples.

- Define and explain ‘Zeroth Law of Thermodynamics’.

- A refrigeration plant for a food store operates as a reversed carnot heat engine cycle.  The store is to be maintained at a temperature of –5oC and the heat transfer from the store to the cycle is at the rate of 5 kW.  If heat is transferred from the cycle to the atmosphere at a temperature of 25o C, Calculate the power required to drive the plant.

2.  a)	Why only in constant pressure non-flow process the enthalpy change is equal to heat transfer?

b)	A gas occupies 9 m3 at a pressure of 2 bar.  If it is compressed isothermally to 1.5m3, determine the final temperature.

c)	An evacuated bottle is fitted with a valve through which air from the atmosphere at 760 mm Hg and 25oC is allowed to flow slowly to fill the bottle.  If no heat is transferred to or from the air in the bottle, what will its temperature be when the pressure in the bottle reaches 760 mm Hg.

3.  a)	Define a pure substance.  Is water a pure substance?

- Draw T-S and P-h chart for steam.

- Dry and saturated steam at a pressure of 11 bar is contained in a closed metallic drum.  Due to cooling, the temperature in the drum reduces to 105oC.  Calculate (i) the pressure in the vessel and (ii) heat lost in cooling.

4.  a)	Prove that air standard efficiency for an engine working as Otto cycle is

1−1rγ−1

b)	The pressure limits in an Otto air cycle are 100 kN/m2 and 2000 KN/m2 respectively.  The compression ratio is 4.  Calculate  (i) Thermal efficiency       (ii) mean effective pressure.

Contd……2.

#### Code No: 220252				:: 2 ::					OR

5.  a)	Explain the main difference between a two stroke cycle internal combustion engine.

b)	With help of neat sketches describe the working of a four stroke cycle internal combustion engine.

6.  a)	Explain the difference between impulse turbine and reaction turbine.

b)	In a stage of an impulse turbine provided with single row wheel, the mean diameter of the blades is 1 m.  It runs at 3000 r.p.m.  The steam issues from the nozzle at a velocity of 350 m/s. and the nozzle angle is 20o.  The rotor blades are equiangular.  The blade friction factor is 0.86.  Determine the power developed if axial thrust on the end bearing of a rotor is 10 kN.

7.  a)	Explain with a neat sketch the closed cycle gas turbine plant.

b)	A gas turbine plant works between the temperature limits of 1152o K and 288o K, isentropic efficiencies for compressor and turbines are 0.85 and 0.8, respectively.  Determine the optimum ratio for maximum work output and also for maximum cycle thermal efficiency.

8.  a)	Why is the air compressed in stages with intermediate water looking preferably to the original temperature.  What is the optimum value of intermediate pressure in two stage compression with perfect inter cooling.

b)	Explain the principle of working of rotary compressor with the help of line diagram.

@@@@@