Code No:220155

### OR

# II B.Tech. II Semester Supplementary Examinations January,  2003

# MECHANICS OF SOLIDS - II

(Civil Engineering)

Time: 3 Hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.  a)	At a certain point in a Bracket the stresses on two mutually perpendicular planes are 120N/mm2 tensile and 60 N/mm2 tensile.  The shear stress across these planes is 30 N/mm2.  Find using the Mohr’s stress circle, the principal stresses and maximum shear stress at the point.

- At a point in the material only complementary shear stresses are introduced as shown in figure Determine the Principal plane & principal stresses.

2.	A thick cylindrical shell is of 200 mm internal diameter and has to withstand on internal pressure of 25 N/mm2.  Calculate the thickness of the metal necessary for the cylinder on the basis of (i) Maximum Principal stress theory (ii) Maximum Principal strain theory.

3.	Derive the Lame’s equations for the compound cylinders from the basic principles.

4.  a)	Derive the Rankines empirical formulae for columns and explain briefly.

b)	Derive the formulae for crippling load for Euler column when both ends of the column are hinged.

5.	A shaft has to transmit 140 hp at 160 rpm.  The shear stress not to exceed 65N/mm2 & twist in a length of 350 cms not to exceed 1o.  Find suitable diameter of shaft.  Take N = 0.8 x 105  N/mm2.

Contd……..2

Code No: 220155				:: 2 ::					OR

6.  a)	Explain in brief about various types of springs with neat sketches.

b)	A spring is made of steel wire 4mm in diameter.  The diameter of the spring is 40mm.  The number of coils in the spring is 6.  In the unloaded condition the clearance between the coils is 1mm.  What compressive force should be applied to the spring to make clearance to disappear.  Take N = 0.8 x 105 N/mm2.

7.	Determine the principal moments of Inertia for an un-equal angle section 60x40x60 mm as shown in figure.

8.	A circular beam of rectangular cross section of dia. 10m supported on 8 equally spaced columns and is subjected to VDL of 4 KN/m.  Determine the location of critical section and design values of bending moment.  Shear force and torque.

@@@@@