Code No.220204

Set No.

1

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are pumped storage plants?  Describe with a neat sketch, the principle of operation of such a plant.

b)	Discuss the various factors that affect the location of site of a hydel power plant.

2.a)	Explain the following with reference to hydel power plant (i) Mass curve           (ii) Hydrograph  (iii) Specific speed of the turbine  (iv) Penstock  (v) Surge tank.

b)	A hydro plant operates at a head of 100m and a discharge of 200 m3/sec.  If the efficiency of the turbine is 90%, find the power developed.

3.	Give the layout of a typical thermal power plant and explain the function of different components in the layout.

4.a)	Explain the function of the following components and state the materials used for each in nuclear reactor (i) Moderator  (ii) Reflector  (iii) Shim rod  (iv) Safety rod  (v) regulating rod.

b)	Briefly explain the relative merits of different types of nuclear reactors used in nuclear power plants.

5.a)	What is Kelvin’s law and explain its significance.  State the limitations of Kelvin’s law.

b)	Determine the most economical cross section of conductor for a 3-phase 8 km length line to supply a load at a constant voltage of 33 kV.  During the 24 hour period, the load is 3000 kW for 10 hours, 2000 kW for 6 hrs and 1000 kW for 8 hrs at unity power factor.  The capital cost per km of the line is Rs.(6250+5000a) where ‘a’ is in square cms.  Interest and depreciation charges are 8% and the cost of energy is 50 paise per unit.

6.a)	With the help of a single line diagram, explain the layout of an outdoor substation.

b)	Explain the different bus bar arrangements in a substation and their relative merits.

7.a)	Explain the following factors and their significance (i) Load factor  (ii) Diversity factor  (iii) Utilisation factor  (iv) Plant capacity factor.

b)	What are ‘load curves’ and ‘load duration curves’.  Discuss their utility in the economics of generation.

Contd……2

Code No.220204				-2-				       Set No.1

8.a)	Obtain a two part tariff for the consumers of a supply undertaking which generates 390 x 106 kWhr per annum and has a maximum demand of 130 MW connected to it.  The various costs are as follows.

Fuel cost: Rs.5 x 106, generation cost Rs.2.4 x 106, transmission cost Rs. 5 x 106 and distribution cost Rs.3.4 x 106.  Of these items 90%, 10%, 5% and 7% respectively are allocated for running costs and the remainder being fixed charge.  The loss between the consumer and the generating station is 10% of the generated energy.  If the load factor of the station is raised to 40% for the same maximum demand find the saving in overall cost per kWhr.

- Briefly explain the sinking fund method of calculating depreciation.  State its relative merits over the other methods.

- - -

Code No.220204

Set No.

2

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain mass-curve and its use to estimate reservoir capacity and the rate of discharge that will be available when a reservoir has been built.

b)	From the data given below plot a mass curve for the period and calculate (i) the size of necessary reservoir  (ii) the rate at which the flow would be available with reservoir.  Assume that site for necessary dam to provide storage in the reservoir is available.

| Week : | 1 | 2 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flow (m3/sec): | 200 | 300 | 700 | 900 | 800 | 600 | 1000 | 500 | 400 | 500 | 700 | 100 |

2.a)	Give a neat layout of a typical 500 MW thermal station indicating clearly the position of various station auxiliaries.

b)	Discuss briefly about (i) Boilers  (ii) Chimney  (iii) Cooling Towers.

3.a)	With a neat sketch explain the working of a BWR reactor.

b)	What are the advantages and disadvantages of BWR reactor.

c)	Explain in detail the various nuclear fuels that are used in Nuclear power stations.

4.a)	What do you understand by load curve?  Explain clearly how can you determine load duration curve and integrated load duration curves from

| Time | 6A.M. | 7A.M. | 8A.M. | 9A.M. | 10A.M. | 11A.M. | 12 Mid Night | 1P.M. | 2P.M. | 3P.M. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load in MW | 27 | 29 | 30 | 32 | 32.5 | 33 | 34 | 35 | 35 | 35 |

| Time | 4P.M. | 5P.M. | 6P.M. | 7P.M. | 8P.M. | 9P.M. | 10P.M. | 11P.M. | 12 Mid Night | 1A.M. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load in MW | 35 | 32 | 30 | 27 | 37 | 38 | 36 | 30 | 29 | 25 |

Contd……2

Code No.220204				-2-				         Set No.2

| Time | 2A.M. | 3A.M. | 4A.M. | 5A.M. |
| --- | --- | --- | --- | --- |
| Load in MW | 14 | 15 | 20 | 22 |

Draw the load curve, load duration curve and the integrated load duration curve for the above given data.

5.a)	Discuss the different methods of determining the depreciation of  electrical power plant.

b)	A generating station has a maximum demand of 10,200 KW and a yearly load factor of 60% generating costs inclusive of station capital costs are Rs.100/- per annum per KW plus 4 paise per KWH.  Data for transmission and distribution system of this station is as follows:

Transmission: Annual fixed charges Rs.25,000/- , Diversity factor 1.2

Distribution: Annual fixed charges Rs.500,000/-, Diversity factor 1.5

If the efficiency of transmission is 93% and that of distribution (inclusive of substation losses) is 88%.  Find the yearly cost per KW demand and the running charge per KWH (i) at the sub station (ii) at the consumer’s premises.

6.a)	A single phase A.C. system supplies a load of 200 KW and if this system is converted to 3-phase, 3-wire A.C. system by running a third similar conductor.  Calculate the 3-phase load that can now be supplied if the voltage between the conductors is the same.  Assume the P.F and transmission efficiency to be the same in the two cases.

b)	A two wire ring main is loaded as shown below and bed from A.  The feeder cable has a cross-section of ‘a’ cm2 and the ring main cross-section of ‘b’ cm2 per conductor.  The distances are given in meters.  Find the cross-section ‘a’ and ‘b’ which will give the least volume of copper in a system.  If the maximum voltage drop from A is not to exceed 150 volts. P=1.73  ohm-cm.

Contd……3

Code No.220204				-3-				       Set No.2

7.a)	Draw the layout of a rain and Transfer Bus bar system and explain.  Also state the advantages and disadvantages of the system.

b)	What are the advantages of outdoor substation when compared to Indoor substation.

8.	Write a short note on any THREE of the following:

(a) Pumped storage plants   (b) Super Heaters  (c) Reflectors and Coolants in Nuclear Reactors  (d) Group Switching.

- - -

Code No.220204

Set No.

3

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are the factors of selection of site for hydroelectric station?

b)	What is “run off”?  Discuss about the “hydrograph”.

2.a)	What are the characteristics of a water turbine?

b)	A turbine in a hydel plant develops 2400 hp with a head of 400 ft.  Determine the specific speed of the turbine if it has to run at a speed of 500 rpm.

3.a)	What are the types of fuels used in Thermal power plants?  Briefly discuss.

b)	What is feed water?  What are the problem due to impurities in fuel water?  How they can be eliminated.

4.a)	How you will explain mechanism of energy release in a nuclear reaction.

b)	What are the advantages and disadvantages of the nuclear power station?

5.a)	What are the classifications of the nuclear reactors?  Describe briefly.

b)	List out main parts of a reactor and briefly state their function.

6.	Write short notes on various systems of D.C. distribution.

7.a)	Explain two part tariff.

b)	A consumer has an annual consumption of 70,080 kWh.  The charge is         Rs.100 per kWh of maximum demand plus 5 paise per kWh.   (a) Find the annual bill and the overall costs per kWh if the load factor is 40%.  (b) What is the overall cost per kWh if (i) consumption was reduced by 25% with the same load factor (ii) Consumption remains the same as in (a) but the load factor is reduced to 30%.

8.	Write short notes on the following:

a) Electrostatic Precipitators

b) Surge tank

c) Layout of substation.

- - -

Code No.220204

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	How hydel plants are classified?  Discuss briefly.

b)	What are the characteristic of water turbine?

2.a)	Describe briefly how a water turbine is controlled?

b)	A river has a discharge of 500 lit/sec and the available head is 100 m. will it be advisable to machine efficiency to be 90%.

3.a)	What are the factors to be considered for selection of the site for a thermal power station.

b)	Why pulverized fuel is preferred?  What are the types of pulverized fuel burners.

4.a)	What are the types of nuclear reaction?  Describe briefly.

b)	Explain with a simple block diagram working of a nuclear power station.

5.a)	Define the following terms in connection with the power supply system:              (i) Connected load  (ii) Maximum demand  (iii) Demand factor  (iv) load factor.

b)	A Thermal power station has an annual fixed cost of Rs.350.40 per kW of installed capacity.  The fuel and operating costs are 4 paise per kWh generated.  Find the cost of electrical energy generated per kWh for station load factors of 25, 50, 75 and 100%.  Plot a curve showing the variation of cost of energy with load factor.

6.a)	Write short note on various systems of d.c. distribution.

b)	What is Kelvin’s law?  What is most economical size of conductor.

7.	How do you classify substations (a) indoor  (b) out door substations.  Draw lay out diagrams of the above.

8.	Write short note on the following:

a) Penstock

b) Kaplan turbine

c) Electrostatic precipitator

d) Fission.

- - -