Code No:220853

OR

II.B.Tech II-Semester. Supplementary Examinations. January,2003

CHEMICAL ENGINEERING THERMODYNAMICS

(Chemical Engineering)

Time: 3 hours.							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks.

---

1.	Explain the following and give two examples of each

a)	(i)	Extensive and Intensive properties.

(ii)	Reversible and irreversible processes.

(iii)	State function and path function.

b)	Water flows through a horizontal coil heated from the outside by high temperature flue gases.  As it passes through the coil, the water changes state from 2 atm (202.66 KPa) and 180o F (82.2oC) to 1 atm (101.33 KPa) and 250o F (121.1oC).  Its  entering  velocity  is  10  ft/sec  (3.05 m/s)   and  its  exit  velocity  is  600  ft/s

(182.9 m/s) Determine the heat transferred through the coil per unit mass of water.  Enthalpies of the inlet and outlet water streams are

inlet    = 148 Btu/lbm (344.2 KJ/kg)

output = 1168.8 Btu/lbm (2718.5 KJ/kg)

2.  a)	Explain the application of cubic equations of state.

b)	Determine the molar volume of n-butane at 510oK and 25 bar by each of the following.

(i)	The ideal gas equation

(ii)	The generalized compressibility factor correlation

(iii)	The generalized virial - coefficient correlation.

3.  a)	If 10 moles of ethylene is heated from 200oC to 1000oC in a steady flow process at approximately atmospheric pressure.  What is its entropy change.  Give that,

A = 1.424; B = 14.394 x 10-3;  C = -4.392 x 10-6;  D = 0.

b)	One mole of an ideal gas is compressed isothermally but irreversibly at 400 K from 3 bar to 7 bar in a piston cylinder device.  The work required is 35% greater than the work of irreversible isothermal compression.  The heat transferred from the gas during compression flows to a heat reservoir at 300 K.  Calculate the entropy changes of gas, reservoir and D S total.

4.  a)	For acetone at 20oC and 1 bar b = 1.487x10-3 / 0C , K = 62X10-6/bar,

V = 1.287 cm3gm  Find   (i)	The value of ∂P∂Tv

(ii)	The pressure generated when acetone is heated at constant volume from 20oC and 1 bar to 30oC.

(iii)	The volume change when acetone is changed from 20oC and 1 bar to 0oC and 10 bar

Contd…….2

Code No:220853			-2-					OR

b)	Show that

(i)	∂V∂PH=∂V∂PT+TCp  ∂V∂TP2−VCP ∂V∂TP

(ii)	∂A∂Ts=CvPT(∂POT)v−S

5.  a)	Derive Maxwell’s equations, explain their importance.

b)	Estimate VR, HR and SR for steam at 200oC and 1400 kpa by using generalized correlations. Given that   TC = 647.3 K,  PC = 220.5 bar, VC = 56 cm3/mol

W = 0.344.

6.	Calculate Z and V for methanol vapour at 200oC and 10 bar by the following equations.

a)	A truncated virial equation Z=PVRT=1+BV+CV2 with the following values of virial coefficients, B = -219 cm3/mol, C = -17,300 cm6/mol.

b)	The truncated virial equation Z=PVRT=1+BPRT with a value of B from the generalized pidzer correlation.

c)	The Red lich -Kwong equation with estimates of a and b given, Tc = 512.6K,

Pc = 81 bar W = 0.564.

7.  a)	Explain about the variations of heat capacity of a substance with temperature.

b)	How much heat would be absorbed or released from hydrogenation of acetylene to ethane at standard conditions.

C2 H2 (g) + 2H2 (g) 	 	 C2 H6(g)

- Use heat of combustion data

- Use heat of formation data.

Combustion data at 25oC : C2 H2 : DHoC = -310620 cal/gm

C2 H6 : DHoC = -372820 cal/gm

H2 : DHoC = -68317 cal/gm

Heat of formation data at 25oC:

C2 H2 : DH¦ = 54194 cal/g.mole

C2 H6 : DH¦ = -20236 cal/g.mole

8.	Write short notes on any two of the following.

(i)	Phase rule and criterion for equilibrium.

(ii)	Heat effects of Industrial reactions.

(iii)	Thermodynamic properties of an ideal bas.

@@@@@