Code No.222102

Set No.

1

II-B.Tech. II-Semester Supplementary Examination January - 2003

ELECTRONICS AND INSTRUMENTATION

(Aeronautical Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	With the help of a neat diagram explain the operation of a cathode ray oscilloscope (CRO). Explain about each block in detail.

b)	What is deflection sensitivity of a CRO. Derive an expression for the deflection sensitivity (electrostatic) of a CRO.

- Why Aquadog coating is used on the inner surface of the CRT.

2.a)	With neat energy band diagrams explain the working of conductors, insulators and semiconductors.

b)	With a neat diagram explain the operation of an unbiased PN junction. Mention neatly the potential gradients on both P side and n side.

3.a)	With a neat circuit diagram explain the operation of a Full-Wave rectifier. Define ripple factor and regulation with respect to rectifier.

b)	A full-wave rectifier uses two diodes, the forward resistance of each diode is 20. The transformer r.m.s. secondary voltage from center-tapped to each end of the secondary is 50v and the load resistance is 980. Determine   (i) The average value of load current and    (ii) the r.m.s. value of load current    (iii) Ripple factor.

4.a)	With help of a neat circuit diagram and V-I characteristics explain the operation of a varactor diode.

b)	What is tunneling? Explain about the operation of a tunnel diode with its circuit diagram and VI characteristics.

5.a)	Explain with the help of neat circuit diagrams, how a JFET can be used as a voltage variable resistor and voltage controlled attenuator.

b)	A transistor connected in CE configuration has values of IE = 10mA and  = 0.987. Determine the value of IC and IB for the given transistor.

6.a)	With help of a neat construction diagram explain the working of a enhancement type MOSFET. Also mention some advantages of N-channel MOSFETs over P-channel MOSFETs.

b)	With the help of a neat circuit diagram explain the operation of an UJT and draw its VI characteristics. Also give a few applications of UJT.

(contd…2)

### Code No.222102			-2-			Set No.1

7.a)	Write in detail about the following:

i)   Linkages and gears	(ii) Geneva mechanism

iii) Belt chain and Friction drives.

- Write in detail about flapper nozzle and flip-flops.

8.	Write short notes on the following:

- Dials and scales

- Pointers and Indicating mechanism

- Bellows and Boosters.

***     ***     ***

Code No.222102

Set No.

2

II-B.Tech. II-Semester Supplementary Examination January - 2003

ELECTRONICS AND INSTRUMENTATION

(Aeronautical Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	With a neat constructional diagram explain the operation of a cathode ray tube (CRT).

- Why is a delay line used in the vertical section of the oscilloscope?

- What is the velocity of electrons that have been accelerated through a potential of 2000V.

2.a)	What is doping? Distinguish between the intrinsic and entrinsic semi-conductors with the help of their atomic structural diagrams.

b)	Explain the operation of a diode in both forward and reverse bias conditions. Explain what is potential barrier.

3.a)	Derive expressions for diffusion and transition capacitances of a PN junction diode.

b)	A half-wave rectifier is used to supply 30V d.c. to a resistive load of 600. The diode has a forward resistance of 25. Find the maximum value of the ac voltage required at the input.

4.a)	With a neat circuit diagram and V-I characteristics explain the operation of a photo diode. Also give a few applications of photodiode.

b)	With a neat circuit diagram explain how a Zener diode is used as a voltage regulator. Differentiate between Zener break down and avalanche break down of a Zener diode.

5.a)	Explain how a p-n-p transistor can be used as an amplifier. Draw the input and output characteristics of a pnp transistor. Also explain how the ‘’ is calculated from the graph.

b)	The common-base d.c. current gain of a transistor is 0.967. If the emitter current is 10mA, what is the value of the base current?

6.a)	With neat structural diagrams explain the operation of JFET and UJT along with their characteristics also give two applications for each one.

b)	A JFET has VGS(off) = -20V and IDSS = 12mA. Plot the trans conductance curve for the device using VGS values of OV, -5V, -10V, -15V and –20V.

(contd…2)

### Code No.222102			-2-			Set No.2

7.a)	Write in detail about the following:

- Pivots and bearings

- Rack and pinion

- Pointers and indicating Mechanism

b)	Write in detail about Geneva mechanism.

8.	Write short notes on the following:

- Pneumatic operated values

- Pneumatic relays

- Pneumatic cylinder motors

- Flip-flops.

***     ***     ***

Code No.222102

Set No.

3

II-B.Tech. II-Semester Supplementary Examination January - 2003

ELECTRONICS AND INSTRUMENTATION

(Aeronautical Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain how voltage, current, frequency and phase are measured using a CRO.

b)	With neat diagrams explain the different types of probes used in CRO for measurements.

c)	Why is an attenuator probe used?

2.a)	Explain the operation of a PN junction diode in both forward and reverse bias conditions. Also draw the V-I characteristics of the diode in both the cases.

b)	Distinguish between conductors, insulators and semiconductors with the help of their neat energy band diagrams.

3.a)	With a neat circuit diagram explain the operation of a bridge rectifier. Define the terms PIV and rectifier efficiency with respect to rectifier.

b)	A half-wave rectifier circuit supplies 100mA d.c. to a 250 load. Find the d.c.output voltage, the PIV rating of the diode and the r.m.s. voltage for the transformer supplying the rectifier.

4.a)	Discuss in detail about the effect of temperature on a PN junction diode.

b)	Draw the VI characteristics of a Zener diode and the Zener and avalanche breakdown in Zener diode with neat diagrams.

5.a)	Explain how a transistor can be used as an amplifier.

- Obtain the relationship between α and β of a transistor.

- What is the effect of temperature on the reverse saturation current Ico of a transistor.

6.a)	With a neat circuit diagram explain the operation of JFET and draw its VI characteristics. Also define drain resistance (rd), transconductance (gm) and amplification factor (μ) and give relationship between them.

b)	With the help of a neat construction, diagram explain the operation of depletion type MOSFET and also draw its static characteristics.

7.a)	Write about the following in detail:

i)   Belt chain and Friction drives	(ii) Counters and Ratchets

iii) Bellows and Boosters.

- Write in detail about pneumatic relays and pneumatic operated valves.

(contd…2)

#### Code No.222102			-2-			Set No.3

8.	Write short notes on the following:

- Flapper nozzle

- Escapement

- Integrators

- Linkages.

***     ***     ***

Code No.222102

Set No.

4

II-B.Tech. II-Semester Supplementary Examination January - 2003

ELECTRONICS AND INSTRUMENTATION

(Aeronautical Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Define deflection sensitivity of a CRO. With a neat diagram explain vertical and horizontal deflection systems of a CRO.

- Derive an expression for magnetostatic deflection sensitivity of a CRO.

- List out the various applications of CROs.

2.a)	Explain the operation of an unbiased PN junction with the help of a neat diagram also explain how electric field intensity and potential gradients are distributed on both P-side and N-side.

b)	With neat energy band diagrams distinguish between intrinsic and entrinsic semiconductors. Explain the terms donor level acceptor level and fermilevel on the energy band diagrams.

3.a)	Compare the performance of the three types of rectifiers i.e. Half wave, Full wave and Bridge rectifiers with respect to the average current, rms current, ripple factor, PIV and rectifier efficiency.

b)	A silicon diode is operating at 25oC with a forward voltage of 0.6V and a current of 0.5amp. If the current is held constant at 0.5 amp, what voltage will exist across the diode at 75oC.

4.a)	With a neat circuit diagram explain how a Zener diode can be used as a voltage regulator.

b)	Explain the operation of a Tunnel diode with the help of a neat circuit diagram. Also draw V-I characteristics of a tunnel diode.

5.a)	Draw the output characteristics of a BJT in CE configuration. Explain about the three regions of operation with respect to the characteristics.

b)	With a neat diagram explain the base width modulation in BJTs. What is early effect?

6.a)	Compare BJT and FET and give some important differences between them.

b)	The data sheet of a certain JFET indicates that IDSS equal to 15mA and VGS(off) equal to –5V. Determine the draincurrent for VGS equal to 0V, -1V and –4V.

c)	Give a few applications of MOSFETS.

(contd…2)

#### Code No.222102			-2-			Set No.4

7.a)	Write in detail about the following:

- Dials and scales

- Integrators and linkages

- Pivots and bearings

b)	Write in detail about the Geneva mechanism.

8.	Write short notes on the following:

- Ratchets and counters

- Belt chain and Friction drives

- Pneumatic operated valves

- Pneumatic cylinder motor.

***     ***     ***