Code No:220501

Set No.

1

# II B.Tech. II Semester Supplementary Examinations January,  2003

# COMPUTER ORGANISATION

(Common to Computer Science & Engineering, Computer Science & Information Technology, Electronics & Computer Engineering, Computer Science & Systems Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	Perform the binary division of (66.25)10 and (50.625)10.

b)	By how many ways the negative numbers in binary can be represented.  Explain them with an example.

2.  a)	Explain the types of operands and operations each with an example.

- List out the characteristics of RISC architectures.

3.  a)	Distinguish between horizontal and vertical micro instructions.

- Explain micro program control unit architecture.

4.  a)	Explain the segmented – page mapping of memory management.

b)	A digital computer has a memory unit of 64 K x 16 and a cache memory of 1 K words.  The cache uses direct mapping with a block size of 4 words. (i) How many bits are there in the tag, index, block, and word fields of the address format.  (ii) How many bits are there in each word of cache, and how are they divided into functions & Include a valid bit.  (iii) How many blocks can the cache accommodate?

5.	Explain with a block diagram,  the salient features of 8085 microprocessor.

6.	Explain with a block diagram DMA controller.

7.	Explain the organization of I/O processor.

8.	Write short notes on any two:

- Von Neuman Modulee

- I/O addressing

- Micro Operations.

@@@@@

## Code No:220501

Set No.

2

# II B.Tech. II Semester Supplementary Examinations January,  2003

# COMPUTER ORGANISATION

(Common to Computer Science & Engineering, Computer Science & Information Technology, Electronics & Computer Engineering, Computer Science & Systems Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	What are the various computer components.  Explain them.

b)	A 36-bit floating-point binary number has 8-bits plus a sign for the exponent.  The mantissa is assumed to be a normalized functions.  Negative numbers in the mantissa and exponent are in signed-magnitude representation.  What are the largest and smallest positive quantities that can be represented, excluding zero?

2.  a)	Classify the instructions based on number. of addresses.  Explain each with an example.

b)	Explain the Pentium operation types.

3.  a)	Explain manoprogrammed control unit with a diagram

- What are the applications of micro programming?

4.  a)	Write notes on memory hierarchy.

b)	Explain the concept of virtual memory.

5.	Explain the architecture of 8085 microprocessor with a block diagram.

6.	Explain programmed I/O and interrupt driver I/O.

7.  a)	Explain the segmented-page mapping of memory management.

- Describe the organization of DMA.

8.	Write short notes on any two:

- Cache memory

- I/O addressing

- Priority interrupts.

@@@@@

## Code No:220501

Set No.

3

# II B.Tech. II Semester Supplementary Examinations January,  2003

# COMPUTER ORGANISATION

(Common to Computer Science & Engineering, Computer Science & Information Technology, Electronics & Computer Engineering, Computer Science & Systems Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	Write notes on Floating-point arithmetic.

- Write an 8085 ALP for reversing the n digit decimal number.

2.  a)	Explain the power PC processor organization.

- Mention few uses of NOOP instruction.

3.	Explain micro program sequencer with a block diagram.

4.  a)	Explain the salient features of associative memory with a block diagram.

- An address space is specified by 16-bits and the corresponding memory space by 12 bits.  (i) How many words are there in the address space and in the memory space. (ii) If a page consists of 256 words, how many pages and blocks are there in the system.

5.	Explain the organization of 8085 microprocessor.

6.  a)	Explain the I/O module structure with a block diagram.

b)	A DMA module is transferring characters to memory using cycle-stealing from a device transmitting at 9600 bps.  The CPU is fetching instructions at the rate of 1 million instructions per second (1MIPS).  By how much will the processor be slowed down due to the DMA module?

7.  a)	Describe the addressing modes of 8085.

b)	Describe various instruction formats.

8.	Write short notes on any two:

- Cache Memory

- Memory Hierarchy.

- Micro Operations.

@@@@@

## Code No:220501

Set No.

4

# II B.Tech. II Semester Supplementary Examinations January,  2003

# COMPUTER ORGANISATION

(Common to Computer Science & Engineering, Computer Science & Information Technology, Electronics & Computer Engineering, Computer Science & Systems Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	Perform the binary multiplication of (66.25)10 and (50.625)10.

b)	By how many ways the negative fixed-point decimal numbers can be represented.  Explain them with an example.

2.  a)	Explain the various addressing modes of an instruction each with an example.

- Explain the instruction cycle with the state diagram.

3.	Explain the operation of processor control unit.

4.  a)	Explain the memory hierarchy.

b)	A set-associative mapping cache has a block size of 4 words and a set size of 2.  The cache can accommodate a total of 2048 words from main memory.  The main memory size is 128 K X 32.

(i) Formulate all pertinent information required to construct the cache memory

(ii) What is the size of the cache memory?

5.	With a neat block diagram explain the various blocks in 8085 microprocessor.

6.	What are he draw backs of programmed I/O and interrupt driven I/O.  Explain the procedure to overcome these.

7.  a)	Explain the implementation of segmentation scheme.

b)	Describe associate memory organization.

8.	Write short notes on any two:

- Semiconductor Memories

- I/O Processor

- Priority Interrupts.

@@@@@