Code No.220103

Set No.

1

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							   Max. Marks:80

Note:- 1) Answer ONE question from Part-A and THREE questions from Part-B.

2) Use of IS:456 and design charts from SP-16 is allowed.

- - -

P A R T – A

1.	A short circular column 5m height is to carry a characteristics load of 350 KN.  Assuming both the ends of column are fully restrained, design the column if the column is to be made as a spirally reinforced column.  Use M25 concrete and Fe 415 grade steel.  Also design suitable footing.  Assume moderate expose condition.  Adopt limit state method.  Draw to scale (i) Sectional plan

(ii) Longitudinal section.

2.	The panel of a slab is 4m x 5m.  One short edge and long edge of the slab are discontinuous and other short edge and long edge are continuous.  The slab is restrained with edge beam.  Super imposed load is 4 kN/m2 and floor finish being 1.5 kN/m2.  Design the slab.  Use M25 concrete and Fe 415 steel.  Adopt limit state method.  Assume mild exposed condition.  Draw to scale the cross section of the slab showing the reinforcement details.

P A R T – B

3.a)	What is workability of concrete.

b)	What are the factors affecting the workability of concrete.

c)	What is Abram’s law?  What are its limitations.

4.a)	Find the ultimate moment of resistance of a  T-section.  The materials are M20 grade concrete and HYSD reinforcement of grade Fe 415.  Breadth of flange = 1500 mm, Depth of slab = 100 mm,  Breadth of rib = 300 mm.  5 bars of 25 mm are provided as tension reinforcement.  Assume moderate exposure condition.

b)	Explain the terms (i) Bond and Anchorage  (ii) Development length.

5.a)	Design an uniaxially loaded rectangular short column section.  Consider concrete of grade M25 and steel of grade Fe 415.  One dimension of the column shall be 300 mm.  Sketch  the reinforcement details (Not to scale).

b)	Why is it necessary to provide lateral ties in a column.

Contd……….2

Code No. 220103				-2-			               Set No.1

6.	Determine the maximum short term deflections and deflection due to shrinkage for cartilever beam having 3 bars of 25 mm dia in tension and 3 bars of 16 mm dia in compression zone.  Assume moderate exposure condition.  The total dead and live load is 18 kN/m.  The effective span of the beam is 3.5 m.  Use M20 concrete and Fe 415 grade steel.  Adopt limit state method.  The cross section of the beam is 230 mm x 450 mm overall depth.  The materials are M20 grade concrete and Fe415 grade steel.

7.	A rectangular beam section of 300 mm width and 500 mm effective depth is reinforced with 6 bars of 20 mm dia.  Out of which 3 bars have been bent at 45o.  Determine the shear resistance of the bent up bars and additional shear reinforcement required if it is subjected to ultimate shear force of 400 KN.  Consider concrete of grade M20 and steel of grade Fe 415.

- - -

Code No.220103

Set No.

2

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							   Max. Marks:80

Note:- 1) Answer ONE question from Part-A and THREE questions from Part-B.

2) Use of IS:456 and design charts from SP-16 is allowed.

- - -

P A R T – A

1.	A hall in a building is to be provided a floor  consisting of a continuous slab cast monolithically with simply supported beams spaced 3.5 m apart.  The clear span of the beam is 9.5 m.  The floor is to support live and partition loads of 3 kN/m2 and 1.25 kN/m2 respectively at the service state.  The load due to finishes may be considered as 0.75 kN/m2.  Design the slab using M20 grade concrete and Fe415 grade steel.  Use Limit state method.

Draw to a scale: (a) Longitudinal section of the slab showing the reinforcement details.  (b) The cross section at mid span showing the reinforcement details.

2.	A short column carries a service load of 1200 kN.  Using  M20 concrete and Fe415 steel design a circular column with helical reinforcement using limit state method.  Design a suitable footing for the column.  Draw to scale: (a) Plan of the footing showing reinforcement details.  (b) Sectional elevation of the column and footing showing the reinforcement details.

P A R T – B

3.	Design a simply supported concrete beam to support a uniformly distributed factored load of intensity 35.5 kN/m (inclusive of all weights) on an effective span of 4.5 m.  Use M 15 grade concrete and Fe 250 grade steel.

4.	Determine the ultimate moment of resistance of a T-beam section having flange width and depth of 1200 mm and 110 mm respectively.  The effective depth of the beam is 500 mm with a web width of 250 mm.  It is reinforced with 6 no.s of 25 mm dia tension bars.  Use M 20 grade concrete and Fe 250 grade steel.

5.	Design a circular column to carry an axial working load of 1200 kN.  Assume emin<0.05 D.  Use helical reinforcement.  The materials are M 20 grade concrete and Fe 415 grade steel.  For the lateral reinforcement mild steel may be used.

6.	Design a simply supported one-way slab of span 3 m to carry a live-load of

3.5 kN/m2.  Use M-20 grade of concrete and TOR steel reinforcement.  Sketch the reinforcement details.

Contd………2

Code No. 220103				    -2-				       Set No.2

7.	A simply supported T-beam section with a flange width of 1250 mm, web width of 250 mm and flange thickness of 100 mm is reinforced with 3 bars of 20 mm dia at an effective depth of 450 mm.  The beam carries a uniformly distributed service load of 12 kN/m and a concentrated load of 25 kN at midpoint over an effective span of 6m.  Using M 15 grade concrete and Fe 415 grade steel, calculate the short term deflection at the midpoint.

- - -

Code No.220103

Set No.

3

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							   Max. Marks:80

Note:- 1) Answer ONE question from Part-A and THREE questions from Part-B.

2) Use of IS:456 and design charts from SP-16 is allowed.

- - -

P A R T – A

1.	Design an isolated sloped footing for 450 mm x 300 mm column to transit a load of  1000 kN and a moment of 20 kNm.  Use M 20 concrete and TOR 40 steel.  Assume clear cover to reinforcement of footing as 60 mm.  Draw to scale

(a) Sectional plan  (b) Sectional elevation.

2.	The floor of a hall is 9 m x 12 m to the centre of supports.  Beams are spaced at

3 m centre to centre.  The thickness of the floor slab is 120 mm.  Live load on the floor is 4.5 kN/m2 and load due to floor finish and plaster being 1.5 kN/m2.  Design one intermediate beam for flange and shear.  Assume moderate exposure condition.  Use M 25 concrete and Fe 415 grade steel.  Use limit state method.  Draw to scale (a) Longitudinal section of the beam showing reinforcement details.  (b) The cross section of the beam at mid span showing reinforcement details.

P A R T – B

3.a)	Why is the cube strength different from cylinder strength for the same grade of concrete.

b)	What is the effect of water-cement ratio on strength of concrete?

c)	Draw a typical stress-strain curve of concrete and indicate the various modulii.

4.a)	Explain the term ‘limiting depth of neutral axis’ in R.C beam design.  Derive its value for a rectangular section using Fe 250 grade steel and M 20 concrete.

b)	A doubly reinforced rectangular beam is 300 mm inside and the effective depth of the beam section is 600 mm and the effective cover to both tension and compression reinforcement from outer surfaces is 50 mm.  The tension reinforcement consists of 4 bars of 16 mm dia.  Assume M 25 concrete and HYSD bars of Fe 415 grade.  Determine the moment of resistance of the section using limit state method.

5.	Design a beam 400 mm x 600 mm subjected to a bending moment of 160 kNm, twisting moment of 20 kNm and a shear force of 100 kN at collapse.  Use M 20 concrete and Fe 415 grade steel.  Assume moderate exposure condition.  Adopt limit state method.

Contd……2

Code No.220103				-2-				   Set No.3

6.a)	What is the necessity for specifying minimum percentage of steel to be provided in columns?

b)	A short circular column 6 m long is to carry a characteristic load of 270 kN.  Assuming both the ends of the column are fully restrained, design the column if the column is to be made as a spirally reinforced column.  Use M 20 concrete and Fe 415 grade steel.  Assume moderate exposure condition.  Show the reinforcement details (Not to scale).

7.	Design a slab for a room of size 4m x 5m.  The edges of the slab are simply supported and the corners are not held down.  The live load on the slab is

3 kN/m2 and load due to floor finish being 1.5 k N/m2.  Take bearing of slab on walls as 230 mm.  Use M 20 concrete and HYSD bars of grade Fe 415.

- - -

Code No.220103

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							   Max. Marks:80

Note:- 1) Answer ONE question from Part-A and THREE questions from Part-B.

2) Use of IS:456 and design charts from SP-16 is allowed.

- - -

P A R T – A

1.	The floor of a hall is 9m x 12m to the centre of supports.  Beams are spaced at 3m centre to centre.  The thickness of the floor slab is 120 mm.  Live load on the floor is 4 kN/m2.  Load due to floor finish and plaster being 1.5 kN/m2.  Design one intermediate beam for flange and shear.  Assume moderate exposure condition.  Use M 20 concrete and Fe 415 grade steel.  Adopt limit state method.  Draw to scale: (a) longitudinal sectional of the beam showing reinforcement details.

(b) The cross section at mid span.

2.	Design using limit state method the interior panel of a slab measuring 4.5m x 6m cross section of supporting beams.  If the live load is 3.5 kN/m2 and the floor finish being 1.5 kN/m2.  Assume mild exposure condition.  Adopt M 20 concrete and Fe 415 grade steel.  Draw to scale: (a) Sectional plan  (b) Longitudinal section parallel to both shorter direction and longer direction of the slab panel.

P A R T – B

3.a)	Briefly explain how the strength of concrete varies with age.

b)	What is modulus of rupture of concrete?  How can the direct tensile strength of concrete be obtained?

4.a)	Discuss the stress block parameters and hence obtain the formula for depth of neutral axis from first principles in the limit state method.

b)	Design a rectangular beam at mid span having an effective span of 4.5 m.  The superimposed load is 50 kN/m and the beam is limited to 230mm x 400mm (overall depth).  Use M 20 concrete and Fe 415 grade steel.

5.	A rectangular beam of size 230mm x 600mm overall depth is subjected to a bending moment of 135 kNm, shear force of 25 kN and a twisting moment of

20 kNm.  Design the section using limit state method.  Use M 20 concrete and

Fe 415 grade steel.  Assume moderate exposure condition.

Contd…2.

Code No.220103				-2-				       Set No.4

6.a)	What is effective length of a column?  How does the behaviour of a column is affected by its effective length.

b)	A column of unsupported length 4.6m is subjected to an axial load of 750 kN.  Assume both ends ad hinges.  Design a rectangular column if one of its dimensions is 230 mm.  Use M 20 concrete and Fe 415 grade steel.  Assume moderate exposure condition.

7.	A simply supported rectangular effective beam of 10m span is having breadth

300 mm and effective depth 700 mm.  Tension steel provided is 3082 mm2 and 2 number of 16 mm bars are provided as compression steel.  Estimate the short deflection and deflection due to shrinkage.  Assume moderate exposure condition.  Use M 25 concrete and TOR 40 steel.

- - -