Code No:220153

OR

II.B.Tech II-Semester. Supplementary Examinations. January,2003

SURVEYING-I

(Civil Engineering)

Time: 3 hours.							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain the following terms in a chain survey.

(i) Base line (ii) Check line (iii) Tie-line  (iv) Random line (v) Oblique off-set

(vi) Swing off-set (vii) Long off-set.

b)	A survey line ABC cuts the banks of a river at B and C and to determine the distance BC, a line BE, 60 m long was set out roughly parallel to the river.  A point D was then found in CE produced and middle point F of DB determined.  EF was then produced to G, making FG equal to EF, and DG produced to cut the survey line in H.  GH and HB were found to be 40 and 80 metres respectively.  Find the distance from B to C.

2.a)	Explain clearly the points of difference between the prismatic compass and surveyors compass.

b)	The following bearings were taken in running an open traverse with a compass in a place where local attraction  was suspected.

Line		observed bearing	Line 		Observed bearing

AB		440 401			CD			300 401

BA		2250 201		DC			2120 21

BC		960 201			DE			3200 121

CB		274018	1		ED			1400 121

At what stations do you suspect local attraction.  Find the corrected bearings of the lines.

3.a)	What is meant by plane Tabling?  When do you recommend it?  State the  advantages and disadvantages  of plane tabling?

b)	Describe with a neat sketch the method of Resection.  For what purpose it is chiefly used.

4.a)	What are the sources of error in “Levelling”.  What precautions should be taken to guard against them.

b)	In running fly-levels from a benchmark of R.L.384.705, the following readings were obtained.

Back sight :	3.215,		1.030,		1.295,		1.855

Fore sight  :	1.225,		3.295,		2.085.

From the last position of the instrument, six pegs at 25 metres interval are to be set out on a uniformly falling gradient of 1 in 100, the first peg is to  have R.L of 384.500.  Work out the staff readings required for setting the tops of the pegs on the given gradient.

Contd…2.

Code No:220153				-2-			OR

5.a)	Describe various methods of contouring.  Discuss the merits and demerits of each.

b)	Explain with sketches the uses of contour maps.

6.	The levels were taken at every 30m along the centre of a proposed road.

They are as follows:

Chainage (metres)			Reduced level (metres)

- 525.55

- 528.30

- 531.85

- 531.50

- 531.55

A cutting is to be made for a line of uniform gradient passing through the first and last points.

Find (a) The gradient of the line and

(b) The volume of cutting.

Take width of cutting at formation Level as 8 m.

The side slopes are 1:1

The surface of the ground has a uniform side slope of 1 in 8.

7.	Explain the construction, working and use of the following equipment

a) Box sextant (b) Pantograph

8.	It is impossible to observe the length and bearing of a line AB directly and the following are the observation made from structures C and D.

Line		Length (m)		Bearing

CA		129.0			S  680 241 W

CD		294.0			N  200 361 E

DB		108.0			N  600 181 W

Compute the length and bearing of AB and also the angles CAB and DBA.

********