Code No.221002 ## Set No. 1

II-B.Tech. II-Semester Supplementary Examination January - 2003

TRANSDUCERS IN INSTRUMENTATION

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	What are transducers? Give a general classification of various types of transducers based on the conversion principle.

- Distinguish between static and dynamic characteristics of an instrument.

2.	Obtain the transient output response of first and second order instrumentation systems to unit step input using differential equation solution method. Give the necessary equations and draw the related graphs.

3.a)	Define gauge factor and derive an expression for the same.

b)	What is a thermistor? Explain the principle used. Plot a graph between resistance and temperature of thermistor and indicate the maximum safe continuous temperature condition.

4.a)	what are magnetostrictive transducers? Explain their operation and discuss one application in detail.

b)	Explain the principle of operation of inductive transducer working on principle of variation of mutual inductance with linear displacement.

5.a)	Explain about applications of capacitive transducers. How they are used for liquid level measurement, explain in detail.

b)	Explain the principle of working of capacitive transducer based on variable gap type.

6.a)	What is Piezoelectricity? List materials exhibiting Piezoelectric property. Derive expressions for voltage sensitivity and charge sensitivity of Piezoelectric material.

b)	What is Piezo resistive effect? Briefly explain. Also list materials exhibiting this property.

7.a)	With schematic diagram, explain about Electrodynamic Force-balance transducer for the measurement of linear acceleration.

b)	Explain the static performance of an Electrodynamic Force-balance acceleration transducer with respect to sensitivity and linearity.

8.	Discuss various optical pyrometers briefly and bring out their usefulness in non-contact temperature measurement.

***     ***     ***

Code No.221002 ## Set No. 2

II-B.Tech. II-Semester Supplementary Examination January - 2003

TRANSDUCERS IN INSTRUMENTATION

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Define a transducer. What are the points to be considered while selecting a transducer.

b)	Define ‘drift’, ‘accuracy’, ‘precision’ relevant to a measurement system with example.

2.a)	Explain the procedure to plot the frequency response of higher order systems to a periodic input harmonic signal. Give necessary equations.

b)	A first order instrument is to measure signals with frequency content upto 100Hz with amplitude inaccuracy of 5%. What is the maximum allowable time constant. What will be the phase shift at 50Hz?

3.a)	Distinguish between bonded and unbonded strain gauges. List the main advantages of semiconductor strain gauges.

b)	Describe the principle of operation of Hotwire anemometer. Explain about constant-temperature method.

4.a)	Explain in detail about the core position of LVDT and the variation of output voltage respectively and clearly mention the phase and core position.

b)	What are electromagnetic type inductive transducers? Explain any one of their operation in detail.

5.	Explain the principle of operation of capacitive transducers based on variable dielectric and variable area type. How do you measure liquid level using capacitive transducer, explain.

6.	What is Piezoelectricity? Explain the materials and their desirable properties for use in Piezoelectric transducer. Explain about one application of this transducer.

7.a)	Discuss about the transient response of Force-balance transducers with respect to feedback damping ratio.

b)	Explain the generalized transient response of Force-balance transducers with different degrees of feedback damping to a unit impulse function input.

8.a)	Contrast the principle of operation of thermistor and thermocouple.

- What are Photosensors? Explain the principle of operation of Photo transistor.

***     ***     ***

## Set No. 3

## Code No.221002

II-B.Tech. II-Semester Supplementary Examination January - 2003

TRANSDUCERS IN INSTRUMENTATION

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Draw the block diagram of Instrumentation system and explain its each block.

- Define and briefly explain about ‘Static Sensitivity’, ‘Linearity’ and ‘Hysteresis’ relevant to a measurement system.

2.	Obtain the frequency response of first and second order instrumentation systems to a periodic input harmonic signal. Give the necessary equations and draw the related graphs.

3.a)	Mention various types of Potentiometers with sketches.

b)	Explain the principle and construction of semiconductor strain gauges and explain their advantages and disadvantages.

4.a)	Explain the principle of operation of Magnetostrictive Transducer with a neat sketch.

b)	Explain the construction and principle of working of a LVDT with a neat sketch. List two applications of LVDT.

5.	Explain the functioning of variable gap and variable dielectric type capacitive Transducers. How do you measure liquid level using capacitive transducer, explain in detail.

6.a)	What is Piezo electricity? Explain the materials and their desirable properties for use in transducers.

- Explain the modes of operation of  Piezo electric crystals.

7.a)	Draw the block diagram of the generalized feedback system of a Force-balance transducer and explain its each block.

b)	Explain the dynamic response of Force-balance transducers to sinusoidal excitation with respect to sensitivity and phase angle.

8.	Explain the thermoelectric effect. Give the laws of thermocouple and their applications. Suggest the merits of a Thermocouple system for measurement of temperature when compared to resistance thermometer.

***     ***     ***

Code No.221002 ## Set No. 4

II-B.Tech. II-Semester Supplementary Examination January - 2003

TRANSDUCERS IN INSTRUMENTATION

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Define a transducer. Give a brief description about classification of transducers with suitable example.

b)	Classify the different types of errors that occur in overall instrumentation system and briefly explain about them.

2.a)	Explain the types of test signals used for determination of dynamic characteristics of an instrumentation system.

b)	Draw the block diagrams of a first and second order instruments and explain about determination of their dynamic characteristics experimentally.

3.a)	What is a strain gauge? Mention the types of strain gauges and explain in detail.

b)	Describe the working and construction of resistance thermometers. Describe the materials used for RTDS.

4.	Discuss how transformer-type transducers are used for measurement of linear and angular displacements.

5.a)	Explain the principle of working of capacitive transducer based on variable area type.

b)	What is principle of operation of capacitive transducer. List out the applications of this transducer.

6.a)	Explain the principle of Piezoelectric transducer and explain with a neat diagram the operation of a thermocouple transducer.

b)	Discuss about ‘Loading effect’ and ‘Frequency response’ in Piezoelectric transducers.

7.a)	Explain about ‘The Force-balance Principle’, related to force-balance transducers.

b)	List out the advantages and the disadvantages of Force-balance transducers and briefly explain about them.

8.a)	With a neat sketch explain the operation of total radiation Pyrometer.

- Explain the principle of Photoelectric transducer with a neat diagram.

***     ***     ***