Code No.220106

Set No.

1

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

SURVEYING - II

(Civil Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain permanent adjustments of a theodolite and write method of testing any two adjustments.

b)	The coordinates of A and B are 150N, 200E and 1500N, 1300E respectively.  A third point C has been chosen such that the bearing of AC is29o30' and that of CB is 45o45'.  Calculate lengths AC and AB.

2.a)	What is meant by closing error in a traverse?  Explain various methods for adjusting it.

b)	The following are the particulars of a traverse run in the counter clockwise direction.

Line		Length(m)		Bearing

AB		145.8			342o24'

BC		517.2			14o35'

CD		315.9			137o20'

Calculate the length and bearing of the line DA and angle CDA.

3.a)	Explain the field procedure of conducting a Tacheometric survey.  How are the field observations recorded.

b)	The following are the observations in a Tacheometric survey.

Instrument		Staff		Vertical	Hair Readings

Station		Station	Angle

O			BM		-5o30'		1.75, 1.95, 2.15

O			P		+9o30'		1.50, 1.65, 1.80

The reduced level of the B.M. was 500m.  The Tacheometer was fitted with an anallatic lens with constant 100.  Find the R.L. of station P.

4.	Find difference of levels of the points P and Q from the following data:

Distance between P and Q = 7118 m.

Angle of depression to P at Q = 1o32'12''

Height of signal at P = 3.87m.

Height of instrument at Q = 1.27 m.

Coefficient of refraction = 0.07

R sin 1'' = 30.88 m

R.L. of Q = 417.860 m.

Contd……..2

Code No.220106				     -2-				         Set No.1

5.a)	Explain types of horizontal circular curves with neat sketches and write formulae for finding parameters of simple circular curves.

b)	The alignment of a road is as follows:

Line		W.C.B.		Length(m)

AB		30o			250

BC		90o			150

CD		140o			325

These lines are to be connected by a simple circular curve.  Find its radius and tangent lengths.

6.a)	What is a transition curve?  What are the characteristics of it and explain various advantages of it.

b)	Two parallel lines 200m.  Apart are to be joined by a reverse curve with a deflection angle of 30o.  If the radius of first arc is 400m. and the chainage of the starting point of the curve is 1500m.  Calculate radius of second arc, the chainage of the point of reverse curvature and the finishing point of the reverse curve.

7.a)	Two triangular stations A and B are 60 Km. apart and have elevations 240m and 280m respectively.  Find the height of signal required at B so that the line of sight may not pass near the ground than 2m.  The intervening ground may be assumed to have a uniform elevation of 200m.

b)	Define most probable value of an observation and explain how it is determined.

8.a)	State various tape corrections applied to a base line measurement.

b)	Define normal equation and explain various rules to obtain normal equation for unknown quantity.

- - -

Code No.220106

Set No.

2

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

SURVEYING - II

(Civil Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define fundamental lines of a theodolite and state their relationship.

b)	Explain how you would measure horizontal angles in the field by (i) Method of repetition (ii) Method of reiteration.

2.a)	Explain the following terms in theodolite survey.  (i) changing face (ii) closing error (iii) gales traverse table.

b)	Below are tabulated the measured lengths and bearings of the sides of a closed traverse ABCDE.

Line		Length(m)		Bearing

AB		217.50			S 59o45' E

BC		    --			N 62o30' E

CD		    --			N 37o36' W

DE		283.50			S 55o18' W

EA		173.15			S 2o40' W

Calculate the omitted measurements of BC and CD.

3.a)	Determine the gradient from a point P to a point Q from the following observations.  The constants of the tacheometer were 100 and zero.

Instrument at		Staff	Bearing	Vertical	Hair

Point			Angle		Reading

A		P	140o		+10o45'	1.35, 1.92, 2.49

Q	230o		+5o30'		1.08, 1.90, 2.72

b)	Explain various errors in tacheometric survey.

4.	Two stations A and B are situated at a distance apart of 3541.776m.  The following observations were recorded.

Height of signal at A = 1.517 m

Height of instrument at A = 1.463 m

Angle of elevation from A to B = 2o12'20''

R.L. of A = 1420.5m

Height of signal at B = 3.938 m

Height of instrument at B = 1.512 m

Angle of depression from B to A = 2o2'30''

Determine the R.L. of B and the refraction correction.  Take R sin 1''=30.88 m.

Contd………….2

Code No. 220106					-2-			     Set No.2

5.a)	Draw a neat sketch of a simple circular curve showing various parts and write formulae for parameters of the curve.

b)	Two tangents intersect at chainage of 1190 m., Deflection angle being 36o.  Calculate all the data necessary for setting the curve in the field by Rankine’s deflection angle method.  Peg interval being 30m.

6.a)	What is a transition curve?  State different formulae to find length of it.

b)	Two straights AB and BC intersect at a chainage of 2635.22 m, deflection angle being 48o24'.  It is proposed to insert a circular curve of 300 m radius with two transition curves 80 m. long at each end.  Calculate the shift of the main curve, spiral angle of the transition curve and chainages of all salient points.

7.a)	Explain briefly the following terms in triangulation.  (i) Satellite station  (ii) Phase of signals.

b)	An angle A was measured by different persons and the following are the values.

Angle			No. of measurements

65o30'10''			2

65o29'50''			3

65o30'00''			3

65o30'20''			4

65o30'10''			3

Find most probable value of the angle.

8.a)	Explain measurement of base line by Wheeler’s method.

b)	The angles of a triangle ABC were recorded as follows:

A = 77o14'20''		Weight 4

B = 49o40'35''		Weight 3

C = 53o04'52''		Weight 2

Give the corrected values of the angles.

- - -

Code No.220106

Set No.

3

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

SURVEYING - II

(Civil Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are the different errors that can be eliminated by recording both faces observations?  Explain briefly.

b)	Define Latitude and departure.

c)	Explain Transit Rule of adjustment of closed Traverse.

2.a)	Two Tangents intersect at a chainage of 1050 m with a external deflection angle of 90o.  Compute the data required to set the curve of radius 325 m by offsets from chords produced.  Assume a peg interval of 20m.

b)	Define Degree of Curvature and derive an expression for finding the same in MKS system.

c)	Write an expression for finding the length of vertical curve.

3.a)	Derive an expression for finding RL of a object whose base is inaccessible by single plane method.

b)	The following observations were taken with a Theodolite.

Angle of elevation from P to Q = 2o45'30''

Height of Instrument at P = 1.20 m.

Height of signal at Q = 4.40 m.

Horizontal distance between P and Q = 9800 m.

Take R Sin 1'' = 30.88

Coefficient of refraction :0.07.

Find the RL of Q if the RL of P = 1000 m.

4.a)	Two triangular stations A and B, 75 km apart have elevations of 260 m and 395 m respectively.  The intervening ground may be assumed to have an uniform elevation of 215 m.  Find the minimum height of signal at B so that the line of sight may not pass nearer to the ground than 4 m.

b)	Observations are recorded from the instrument station B to the station A.  The distance AB was 9 km and the diameter of signal was 20 cm.  If the sun rays make an angle of 50o with the line AB, compute the phase correction when the observations were made on (i) on the bright portion (ii) on the bright line.

5.a)	The three angles of A, B and C of a triangle ABC are 49o 20' 14'', 50o29'13'' and 80o10'24'' with weights of 1, 2 and 4 respectively.  Determine the most probable values of A, B and C.

b)	What do you understand by True error and Residual errors?

Contd…………..2

Code No.220106				-2-				Set No.3

6.a)	What are the different functions of Transition Curve?

b)	Two straights AT1 and CT2 met at V.  It is proposed to introduce a reverse curve of  equal radius having T1 and T2 as tangential points.  The angles AT1T2 and VT2T1 are 45o and 25o respectively.  The distance T1T2 is 750m.  Determine the common radius, chainage of PRC and T2 if the chainage of V is 2000 m.

7.a)	State the principle of stadia Tacheometry.

b)	The following observations are recorded by using a Tacheometer fitted with Analectic lens.

| Inst At | Height Of Instrument | Total coordinates |  | Sight To | R.B | Vertical Angle | Staff Readings | Staff Position |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | E |  |  |  |  |  |
| A | 1.55 | 1000 | 500 | C | N30oE | 5o | 1.5, 1.65, 1.8 | Vertical |
| B | 1.59 | 800 | 400 | D | N40oW | 3o | 2.5, 2.95, 3.35 | Vertical |

Given RL of A = 104 m and RL of B = 108 m.  Compute the gradient of line CD.

8.a)	What are the different ways for allocating weights to different observations?  Explain briefly.

b)	Explain various checks to be considered in Theodolite Traversing.

c)	How do you overcome the obstacle when the point of Intersection is inaccessible.

- - -

Code No.220106

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

SURVEYING - II

(Civil Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	State the fundamental axes of a theodolite.  What are the desired relationships between them.  Explain the collimation adjustment of a transit theodolite with the help of a neat sketches.

b)	Following are the lengths and bearings of a traverse ABCD, shown in clockwise direction

Line		Length(m)		Bearing

AB		248			30o

BC		325			145o

CD		185			216o

Calculate the length and bearings of line DA.

2.a)	Explain with the help of illustrations, a method of balancing a traverse.

b)	A and B are two stations whose co-ordinates are as given below.

Station		North co-ordinate(m)		East co-ordinate(m)

A			1056.8					585.2

B			1455.5					995.7

From the station A, a line AC is given whose length is 455m on a bearing of 135o18' and from the station C a line CD of length 1305m parallel to AB.  Find the length and bearing of line BD.

3.a)	Distinguish between fixed hair method and movable hair method of tacheometric survey.

b)	What are the sources of errors in tacheometric survey.

c)	The station intercept read by means of a fixed hair tacheometer on a vertically held staff is 1.050m, the angle of  elevation being 5o14'.  The instrument constants are 100cms 0.4.  What would be the total number of turns registered on a movable hair instrument at the same section for a 1.82m staff intercept on a staff held on the same points the vertical angle in this case being 5o15' and the constants are 1000 and 0.45.

4.a)	What is meant by degree of a curve?  Derive its relationship with the radius of the curve.

b)	Two straights AB and BC intersect at a chainage of 1500m.  The deflection angle is 30o.  It is required to set out a 6ocurve to connect the two straights.  Calculate the necessary data to set out the curve by the method of offsets from the chords produced with a peg internal of 30m.

Contd……..2

Code No.220106				      -2-				       Set No.4

5.a)	Draw a neat sketch of a compound curve showing the salient points and give the relationship of the parts.

b)	Two straights deviate by 45o.  These are connected by a compound curve such that the point of compound curvature is 35m from the point of intersection and the line joining these two points make 36o9' with the first tangent.  The radii of the two branches are 300m and 200m respectively.  Calculate the chainages of the two tangent points and the point of compound curvature if the chainage of intersection point is 7862.6m.

6.a)	Explain the terms (i) coefficient of Refraction and (ii) Axis signal correction in the trigonometric leveling.

b)	The following reciprocal observations were made from two points A and B.

Horizontal distance between A and B = 4032 m

Angle of depression of B at A = 6'10''

Angle of depression of A at B = 9'20''

Height of signal at A = 4.24m

Height of signal at B = 4.-05m

Height of instrument at A = 1.15m

Height of instrument at B = 1.60m

Calculate the reduced level of point B if that of the point A is 1359.6m and the average coefficient of refraction at the time of observations.

7.a)	What is meant by a satellite station and reduction to centre.

b)	The altitudes of two proposed stations P and Q 125km apart are respectively 450m and 725m.  The intervening obstruction situated at the point R, 80km from P has a elevation of 525m.  Ascertain whether P and Q are intervisible and if  necessary, find by how much the point Q be raised so that the line of right must no where be less than 4m above the surface of the ground.

8.a)	What is meant by ‘Bace net’?  Explain the procedure of extending a bace line.

b)	A normal distance of 30m was set out with a 30m steel tape from a mark on the top of one peg to a mark on the top of another, the tape being is catenary under a pole of 150N at mean temperature of 20oC.  The top of one peg was 0.25m below the top of the other.  The top of height peg was 450m above mean sea level.  Calculate the exact horizontal distance between the marks on the two pegs and reduce it to mean sea level if the tape was standardized at temperature of 15oC in catenary under a pole of 100N.  Take radius of earth=6370km.  Mass of tape=0.03kg per m.  Cross sectional area of tape = 2.2mm2.  Coefficient of expansion = 6 x 10-7 per oC.

- - -