Code No.220454

OR

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

PROBABILITY AND RANDOM VARIABLES

( Common to Electronics & Communication Engineering and Electronics & Telematics)

Time: 3 hours							   Max. Marks:70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the significance of the probability density function of a continuous random variable and state the properties of the probability density function.

b)	Consider the probability density function fX(x)=ae-b|x|, where X is a random variable whose allowable values range from x=-µ to x=+µ.  Find the cumulative distribution function F(x) and the relationship between a and b.  What is the probability that the outcome X lies between 1 and 2?

2.a)	Define the mean value and the variance of a continuous random variable.  Calculate these results for a random variable X, whose probability density function is given below:

fX(x)=1/(b-a)		a≤x≤b

=0		elsewhere

- The independent random variables X and Y have the probability density functions fX=1a[4(x)-4(x-a)] and fy(y)= 1b[4(y)-4(y-b)]  with o<a<b.  Find and plot the probability density function of the variable Z=X+Y.

3.a)	Prove the random process X(t)=cos (wct+q) is wide-sense stationary if it is assumed that wc is a constant and q is a uniformly distributed variable in the interval [O, 2P].

b)	What are the properties of the power spectral density of a random process?

4.a)	A random variable x has the probability density function

fX(x)=332 (-x2+8x-12)		2≤x≤6

=0					elsewhere

Find the mean and variance of the random variable X.

- The autocorrelation function of a stationary random process X(t) is given by RXX(t)=(25t2+36)/(6.25 t2+4).  Find the mean and variance of the random process.

5.a)	Explain the following:

(i) Noise sources,  (ii) noise temperature  (iii) Available noise bandwidth.

b)	Derive the noise figure of a two stage cascaded amplifier whose available power gains of the first stage and the second stage are GA and GB, respectively.  The individual noise figures of the two stages are FA and FB.

Contd……….2

Code No.221101				-2-				Set No.1

6.	Consider five messages given by the probabilities 12,14,18,116,116.

(a) Calculate the average information of the messages.

(b) Use the Shannon-Fano algorithm to develop an efficient code and for that code, calculate the average number of bits/message.

7.a)	Consider a binary symmetric channel characterized by the transmission probability of 0.6.  Calculate the mutual information of the channel when the a-priori probability of symbol 1 at the channel input is 0.5.

b)	State and prove the Shannon-Hartely law of information theory.

8.	Write short notes on any THREE of the following:

a) Rayleigh probability density function.

b) Central limit theorem

c) Huffmen source encoding

d) Properties of auto correlation of random process.

- - -