Code No. 220405

Set No.

1

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

CONTROL SYSTEMS

(Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics & Telematics, Electronics and Control Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Explain the Rules of Block diagram Algebra.

- Obtain block diagram representation for the network shown in figure.1

R1		         R2

O						      O

Ei	          C1		   	 C2   Eo

O						      O

Fig.1

c)	Obtain transfer function for network shown in fig.1 using Mason’s gain formula.

2. a)	Explain the classification of control systems with relevant examples.

b)	Obtain the closed loop transfer function C(s)/R(s) for block  diagram shown in fig.2.

3. a)	For a unity feed back system with

G(s)	=	    20 (s +2)

s(s+3) (s + 4)

- Find the static error constants   (ii) Find the  steady state error for

r(t) = 3u(t) + 5 t u(t)

b)	The open loop transfer function of a servo system with unity feed back is

G(s)	= 	10

s(0.1s + 1)

Evaluate the dynamic error using the dynamic error coefficients when the system is subjected to the input.

### Contd…2

#### Code No. 220405				.2.		            	Set No. 1

4. a)	Explain transient and steady state response in time domain.

b)	Give the expression for natural frequency of oscillations of electrical and mechanical system.

- Find the value  of K for the system given below.

(i)	s4 + s3 + Ks2 + s + 1	= 0

(ii)	G(s)	=	      K(s + 13)

s(s+3) (s +7

5. a)	Describe root countours.

- The open loop transfer function of a system is given by.

G(s) H(s)	=	K(s +12)

s2(s +20)

sketch the root locus for the system.

6.	Plot the Bode diagram for the following transfer function and obtain the gain and phase cross over frequencies

GH(s)	=	10

s(1+2s) (1 + s)

7. a)	What is Relative stability?

b)	By applying Nyquist stability criterion determine the stability of closed loop system, whose open loop transfer function is given by

G(s) H(s)	=	s+2

(s+1) (s-1)

comment on the stability of open loop and closed loop system.

8. a)	Obtain the state transition matrix  (t) given the system matrix

A	=	1	0

1	1

- Obtain the time response of the system given by

x1		1	0	       x1		    1

=				     +	   	4

x2		1	1	       x2	                1

where u(t)  is a unit step occurring at t = 0 and xT (0) = (1  0).

###

Set No.

2

# Code No. 220405

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

CONTROL SYSTEMS

(Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics & Telematics, Electronics and Control Engineering)

# Time: 3 Hours						       Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Obtain the transfer functions of the translational mechanical system shown in figure .1

FIG.1

b)	Explain Mason’s gain formula and construct the signal flow graph for the following set of simultaneous equations.

x2	=	A21 x1 + A23 x3

x3	=	A31 x1 + A32 x2 + A33 x3

x4	=	A42 x2 + A43 x3

2. a)	(i)	What is servomotor?

(ii)	What are the characteristic of servo motors?

b)	Derive transfer function of a DC motor when it is operated in field control mode.  Assume necessary data.

3. a)	(i)	Derive an expression for position, velocity and acceleration error constants.

(ii)	Derive an expression for steady state error with step, ramp and parabolic inputs.

b)	The open loop transfer function of a control system with unity feed back is given by	G(s) H(s)	=	150

s(1 + 0.25 s)

- Evaluate the generalized error series for the system.

- Determine the steady-state error for an input r(t) = (1 + t2) u(t).

Contd…2

###### Code No. 220405				.2.				Set No. 2

4. a)	Explain the following time Response specifications.

(i) Delay time	(ii) Rise time	(iii) Settling time

(iv) Maximum (Peak) over shoot.

b)	The system shown in fig.2., find the value of “a” such that the damping ratio is 0.5.  Determine the rise time, peak time, maximum over shoot and settling time, in the unit step response.

Fig.2

5. a)	A unity feed back control system has an open-loop transfer function.

G(s)	=	K

s(s2 +4s + 13)

Sketch the root locus of the system by determing the following.

- Certroid, number of Asymptotes.

- Angle of departure of roots from the poles. (iii) Breakaway points if any.

- The value of K and the frequency at which the root loci cross the

jw-axis.

6. 	Draw the Bode plot for the transfer function

G(s) H(s)	=    	  200(s+z)

s(s2 + 10s + 100)

and determine gain margin and phase margin.

7.	Sketch the Bode plot for the transfer function

G(s)	=   	Ke-01s

s(1+s) (1+0.1s)

“K” for the cross over frequency Wo to be 5 rad/sec.

8.	A system is characterized by the following state space equations.

·

x1	  -3	1	x1	0

·      =			       +		u;	t>0

x2	 -2	0	x2	1

x1

y= {1 0}  	x2

- Find the Transfer Function

- Compute the state transition matrix

- Solve the state equation for a unit step input under zero initial condition.

###

Code No. 220405

Set No.

3

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

CONTROL SYSTEMS

(Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics & Telematics, Electronics and Control Engineering)

# Time: 3 Hours						       Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Explain the following terms with suitable examples.

- Open loop

- Feed back

- Signal flow graphs.

b)	Obtain the transfer function x(s)/(s) for the electromechanical system shown in figure. 1.  Assuming that the coil has a back emf ev = k1 dx and the coil

dt 			current i2 produces a force Fc = k2 i2 on the mass M.

Fig.1

2. a)	(i)	What is the difference between ac servomotor and two phase induction  motor?

(ii)	What is tachogenerator?

b)	An ac servo motor has both windings excited with 115 V a.c.  It has a stall torque of 2 lb-ft.  Its coefficient of viscous friction is 0.2 lb ft. sec.

- Find its no load speed.

- It is connected to a constant load of 0.9 lb ft and coefficient of viscous friction of 0.05 lb-ft.sec. through a gear pass with a ratio of 4.  Find the speed at which the motor will run.

### Contd..2

Code No. 220405				.2.				Set No. 3

3. a)	Explain with the help of graph and mathematical expression of standard test signals as applicable to control systems.

b)	Derive an expression for generalized ERROR coefficients.

4. a)	A unity feed back control system is characteristed by the following open loop transfer function  G(s)		=	0.4s +1

s(s+0.6)

Determine its transient response for unit step input and sketch the response.  Evaluate maximum overshoot and the corresponding peak time.

b)	The error response of a servomechanism is given as e(t) = 1.66e-8t sin (6t+37o) where the input is given a sudden unit displacement.  Determine the natural frequency, damping ratio and damped angular frequency.

5	Sketch the root locus for 0< K< , for the system with open-loop transfer function.

G(s) H(s)	=		 	K

s(s+3) (s2+2s+2)

6. a)	Explain Nyquist stability criterion.

b)	Determine the resonant frequency r, resonant Peak Mr, and Bandwidth for the system whose transfer function is given by

C(s)		=		5

R(s)			    s2 + 2s  + 5

7. a)	Define gain margin and phase margin.

b)	Sketch the Bode plot of the transfer function given by

GH(s)	=	    200 (s + 2)

s(s2 + 10s + 100)

8.	Write a short on the following:

- State vector

- State transition matrix

- LEAD compensation.

- LEAD-LAG NET WORK.

# ###

Code No. 220405

Set No.

4

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

CONTROL SYSTEMS

(Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics & Telematics, Electronics and Control Engineering)

# Time: 3 Hours						       Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Obtain the mathematical model for the mechanical system shown in figure.1

Fig. 1

b)	Draw the force-voltage and force-current electrical analogous circuits for the system show in fig.1

- Verify the result by writing mesh and node equations.

2. a)	With aid of neat sketch, explain the working principle of synchro transmitter.

- Derive the transfer function of synchro receiver.

3. a)	Identify the type, order and characteristic equation of the system shown in figure.2

Fig.2

b)	The open loop transfer function of a servo system with unity feed back is

10

s( 0.1s + 1)

evaluate the static error constants and obtain the steady state error of the system when subjected to an input of r(t) = Ao + A1t + A2 t2

2

Derive the expressions used for error constants.

Contd…2

Code No. 220405				.2.				Set No. 4

4. a)	Derive an expression for Rise time (tr) of a second order system.

- Ascertain the stability of the system with characteristic equation.

s5 + 2s4 + 24s3 + 48s2 – 25s – 50=0

5. 	Show that the loci of complex roots are part of a circle with (-1,0) as center and radius =2 .    For a unity feed back system with open-loop transfer function.	G(s)	=	k(s+1)

s(s-1)

6. a)	Ascertain the stability of the system with loop transfer function

G(s) H(s)	=     	    4s +1

s2	(1 + s) (1 + 2s)

by applying Nyquist stability criterion.

b)	Write short note on constant M and N circles.

7.	The forward path transfer function of a certain unity negative feed back control system is given by.		G(s)	=	k

s(s +2) (s +30)

The system has to satisfy the following specifications.

a)	Phase margin  35o           (b)	Gain margin  20 db

b)	Steady state error for unit ramp input  25.  Design a suitable lead

Compensator.

8. a)	Find a state model for the system with transfer function.

C(s)		=	  	s+4

U(s)			s3 + 6s2 +11s + 6

- Obtain the state space representation of the electrical network shown.

###