Code No. 220404

Set No.

1

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

ANALOG COMMUNICATION

(Electronics and Communications Engineering and Electronics & Telematics)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Show that for an AM wave

Pt	=	Pc   1 + m2

2

Pt	=	Total Power

Pc	=	Carrier Power

m	=	modulation index.

b)	When the modulating frequencies in an FM system is 400Hz and the modulating voltage is 2.4v the modulation index is 60.  Calculate the maximum deviation.  What is the modulation index when the modulating frequency is reduced to 250 Hz and the modulating voltage is simultaneously raised to 3.2v.

- Differentiate between SSB and VSB systems

2. a)	Draw the circuit of practical diode detector for AM signals and explain its operation.

- Explain pre-emphasis and De emphasis with necessary circuits

- What is the necessity of limiter in FM receivers?

3. a)	Describe Foster seeley  Discriminator with a neat circuit diagram and explain its principle with necessary Equations.  What are its merits and Demerits.

- Differentiate between Foster seelay discriminator and ratio detector.

4. a)	Draw the block diagram of FM Transmitter and explain each block’s function.

- Compare AM and FM Transmitters.

- Classify AM Transmitters.

5. a)	What are the advantages of having an RF amplifier in Receivers.

b)	Explain the principle of separately excited and self excited mixers with circuit diagram.

6. a)	Draw the block diagram of TRF receiver.  Explain the function of each block.  What are its Demerits.

- Define the terms sensitivity and selectivity image frequency, double spotting.

7. a)	What are the advantages of Electronic telephone exchange.  Draw the block diagram of stored program control Electronic exchange and explain its working principle.

- Write short notes on telegraphsignalling.

8.	Write short notes on

- Communication receivers, (b)	 NBFM, WBFM

- Fading and diversity receptors.

###

Code No. 220404

Set No.

1

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

ANALOG COMMUNICATION

(Electronics and Communications Engineering and Electronics & Telematics)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Explain third method of SSB generators with a neat block diagram.

b)	Define and describe VSB Transmission.  What is its application.  What are its merits?

2. a)	Derive the formula for instantaneous value of an FM voltage.

b)	The antenna current of an AM Transmitter is 8A when only the carrier is sent, but it increases to 8.93A when the carrier is modulated by a single sine wave. Find the percentage modulation.  Determine the antenna current when the Depth of modulation changes to 0.8.

3. a)	What are the types of distortions in diode detectors and explain them.  How to reduce theses distortions.

b)	Explain envelope detection of AM signals.

4. a)	Explain one method to generate FM signal with circuit diagram.

b)	Explain the principle of ratio detector with the help of neat circuit diagram and necessary sketches and Equations.  Compare foster seeley and ratio detectors.

5. a)	Draw the block diagram of AM Transmitter and explain the function of each block.

b)	Compare AM and FM Transmitters.

c)	Classify AM Transmitters.

6. a)	Draw the block diagram of superheterodyne AM receiver and explain the function of each block.

b)	What are the limitations of TRF Receivers

7. a)	Explain the necessity of  De-emphasis and pre-emphasis in FM.

b)	Draw the block diagram of communication receiver and explain the function of each block.

8. a)	Explain the principle of fascimile telegraphy.

b)	Describe the differentiate duplex and bridge duplex type of telegraph systems.

###

Code No. 220404

Set No.

3

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

ANALOG COMMUNICATION

(Electronics and Communications Engineering and Electronics & Telematics)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Derive the formula for instantaneous value of AM voltage.

b)	Explain the filter method and phase shift method to generate SSB signals.

2. a)	Draw the practical diode detector circuit and explain the function of each component in it.

b)	List different FM signal demodulation methods.  Describe one method of Fm signal demodulation with neat block diagram.

3. a)	What are the different types of distortions in diode detectors.  How to reduce them.

b)	Distinguish between AM, FM and PM.

4.	Draw the block diagrams of FM and Am Transmitters and explain the function of each block.

5. a)	Draw the block diagram of Fm receiver and explain the function of each block.

b)	When a superheterodyne Receiver is tuned to 555KHz, its local oscillator provides the mixer with an input 1010 KHz.  What is the image frequency?  The antenna of the receiver is connected to the mixer via a tuned circuit whose loaded Q is 40.  What will be the rejection ratio for the calculated image frequency?

6. a)	Draw the block diagram of communication receiver and explain the function of each block.

b)	Define sensitivity, selectivity, image frequency.

7.	Write short notes on the following.

- NBFM, WBFM

- Significance of signal to Noise ratio in communication systems.

- AGC.

- De-emphasis and pre-emphasis.

8. a)	What are limitations of non electronic telephone exchanges.

b)	Write short notes on telegraph signaling.

c)	Explain the principle of fascimile telegraphy.

###

Code No. 220404

Set No.

4

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

ANALOG COMMUNICATION

(Electronics and Communications Engineering and Electronics & Telematics)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Derive the formula for instantaneous value of FM voltage.

b)	A 360 w carrier is simultaneously Amplitude modulated by two audio waves with modulation percentages of 55 and 65 respectively.  What is the total sideband power radiated.

2. a)	 Explain the principle of ratio detector with the help of neat circuit diagram, necessary sketches and equations.

b)	Compare ratio detector and Foster seeley discriminator.

c)	Compare AM and FM.

3. a)	Differentiate between high level and low level AM modulations.

b)	Explain one method to generate FM wave.

c)	Draw the block diagram of FM Transmitter.

4. a)	Why the local oscillator frequency should be higher than the incoming RF signal frequency.

b)	Draw the circuits of self excited and separately excited mixers and explain their principle in detail.

5. a)	Draw the block diagram of communication receiver.

b)	Explain the procedure for measurement of selectivity and sensitivity of a receiver.

6. a)	What is the need for modulation

b)	Explain phase shift method of SSB signal generation with neat block diagram.

c)	What is the significance of image frequency and its rejection in radio receivers.

7. a)	Draw the block diagram of SPC stored program control  electronic exchange and explain its principle.

b)	Explain the principle of fascimile telegraphy.

8.	Write short notes on the following:

- Pre-emphasis and De-emphasis	b)	AGC

- Fading and diversity reception.	d)	VSB modulation system.

###