Code No.221152

OR

II-B.Tech. II-Semester Supplementary Examination January - 2003

THERMODYNAMICS

(Bio-Medical Engineering)

Time:3 hours								Max.Marks:70

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain about thermodynamic system, intensive property of a system, internal energy and enthalpy.

b)	A fluid in a vessel is stirred by a paddle wheel. If the power input to the wheel is 800 watts and the heat transferred to the system is 1250 k.cal/hr, determine the change in internal energy per hour.

2.a)	What is steady flow energy equation? Explain each term involved in it.

b)	The efficiency of a Carnot cycle is 60%. The minimum temperature of the cycle is 10o. Determine the maximum temperature of the cycle. The adiabatic expansion ratio for the gas.  = 1.4.

3.a)	Which non-flow compression process needs maximum work to be done?

b)	During a certain flow process the internal energy per kg. of gas decreases by 160 kJ/kg and the flow work increases by 5000 kNm for 10 kg. of the gas. Determine the change in total enthalpy and specific enthalpy.

4.a)	How mechanical reversibility differ from thermodynamic reversibility. Give examples to explain the same.

b)	Air is throttled from 1000kpa at 27oC to 100kpa in a steady flow adiabatic process. Changes in kinetic and potential energies are negligible. Determine the change in entropy per kg. of air flow. Cp = 1.004 kJ/kg.ok.

R = 0.287 kJ/kg.ok.

5.a)	Show that the heat transferred during constant pressure process is equal to change of enthalpy.

b)	A mass of gas has an initial pressure and temperature of 0.11 MN/m2 and 15oC respectively. It is compressed according to the law pv1.2 = C until the temperature becomes 90oC. Determine  (i) Final pressure of gas  	(ii) Work done.

6.a)	Explain the influence of pressure on the following properties of steam with the help of pr-diagram.

- Enthalpy of evaporation.

- Temperature of saturation.

- Enthalpy of dry saturated steam.

b)	How the enthalpy, entropy and internal energy of superheated steam will be found out?

(contd…2)

### Code No.221152				-2-			OR

7.a)	Derive an equation to find the air-standard efficiency of Otto-cycle.

b)	In an Otto-cycle the compression ratio is increased from 5 to 7. Find the percentage change in efficiency.

8.	Answer the following:

- Bell-Coleman cycle.

- Psychrometric charts.

- Throttling and free-expansion.

***     ***     ***