Code No.221201

Set No.

1

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	A signal with a PSD of Px(f)=214Π2+f2 is applied to the network shown (a) Find the PSD for y(t)   (b) Find the average normalized power for y(t).

2Ω

4Ω      y(t)

x(t)

0.5F

b)	Evaluate the Fourier Transform of

w(t)= e-t	t1

= 0		t<0

2.a)	Explain (i) probability.  (ii) Rayleigh probability density.

b)	A random experiment consists of drawing two cards from a deck in succession (without replacing the first card drawn).  Assign a value to the probability of obtaining two red aces in two draws.

3.a)	Describe the operation of a balanced ring Modulator.

b)	Write down the advantages and disadvantages of single sideband Transmission.

4.a)	Determine the peak frequency deviation and modulation index for an FM modulator with a deviation sensitivity 5 KHz/V and a modulating signal υm(t)=2cos(2Π.2000t).  Determine the peak phase deviation for a PM modulator with a deviation sensitivity 2.5 rad/V and a modulating signal m(t).

b)	Explain the Armstrong indirect FM transmitter.

5.a)	Describe the transmitter and receiver of Delta Modulation.

b)	A message signal at a frequency 2 KHz sampled at 50 KHz in the Delta Modulation.  Find the Data rate and Bandwidth required.

6.a)	Distinguish between synchronous and asynchronous time Division Multiplexing techniques.

b)	Distinguish between natural and flat top sampling.

Contd………..2

Code No.221201				-2-				Set No.1

7.a)	What is Source Coding., Channel Coding.  What is Entropy, code efficiency.

b)	Describe about Convolutional Code.

8.a)	Two binary symmetric channels are connected in cascade, Find the overall channel capacity of the cascaded connection, assuming that both channels have the same transition probability.

b)	A voice channel of the telephone network has a bandwidth of 3.4 KHz.  Calculate the information capacity of the telephone channel for a signal to noise ratio of 30 dB.  Also calculate the minimum signal to noise ratio required to support information at the rate of 9600 bits/sce.

- - -

Code No.221201

Set No.

2

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Using the frequency shifting property., find the inverse Fourier Transform of the given spectra

G(f)

1

-5	-3	0	   3	5

f

- Find the fourier series of given waveform below

g(t)

2

-2	0	2	4	6	t

2.	Explain (i) Cummulative distribution function  (ii) Central limit theorem

(iii) Probability Density function  (iv) Random process.

3.a)	For an AM DSBFC Modulator with a carrier frequency fc=100 KHz and a Maximum modulating signal frequency fmax=5 KHz, determine (i) frequency limits for the upper and lower sidebands.  (ii) Bandwidth  (iii) Draw the output frequency spectrum.

b)	What is Envelope Detection.  Explain.

4.a)	Design a wideband FM Tx that uses the indirect method for generating a WBFM signal.  Assume that the carrier frequency of the WBFM signal is 96.9 MHz and that the TX is capable of producing a high quality FM signal with a peak deviation of 75 KHz when modulated by a  1V(rms) sinusoid of frequency 20 Hz.

b)	An FM signal has sinusoidal with a frequency of fm=15 KHz and modulation index of β=2.0.  (i) Find the transmission bandwidth by using Carson’s rule.

(ii) What percentage of the total FM signal power lies within the Carson rule bandwidth.

Contd……2

Code No.221201				-2-				Set No.2

5.a)	What is uniform and Nonuniform quantization?  Explain.  Calculate the quantization noise power for uniform quantization.

b)	Compare PCM and DM.

6.a)	Describe about the generation of pulse width Modulation with neat diagrams.

b)	State and prove the Sampling theorem for low pass signals.

7.a)	Derive the expression for the channel capacity of a binary symmetric channel.

b)	Write about Convolutional Codes.

8.a)	Define information, average information channel capacity, Data Rate.

b)	If a source is delivering 4 different messages with probabilities ½, ¼, 1/8, 1/8 what is the entropy of the source?  If it is sending the messages at the rate of 2000 symbols/sec, calculate the average rate of information.

- - -

Code No.221201

Set No.

3

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Using the convolution property, find the spectrum for W(t) = Sin(2Πf1t). cos (2Πf2t).

b)	Determine the autocorrelation of the signal shown

g(t)   1

-4	           4	t

2.a)	Explain about (i) Central limit theorem  (ii) Cumulative Distribution function.

b)	In a random expt.  a trial consists of four successive tosses of a coin.  If we define an random variable x as a the number of heads appearing in a trial, determine PX(x) and FX(x).

3.a)	For a modulation index m=0.2 in AM and an unmodulated carrier power Pc=1000w, determine (i) Total sideband power (ii) upper and lower sideband power  (iii) Modulated carrier power (iv) Total Transmitted power.

b)	Explain the generation of SSB-SC using phase shift method.

4.a)	Explain the NBFM.  Generation of FM using reactance Modulator.

b)	Describe about the Frequency Division Multiplexing.

5.a)	State and prove Sampling theorem for low pass signals.

b)	What is PAM?  Describe how it is modulated?

6.a)	Describe the Transmitter and Receiver of PCM.

b)	A message signal at a frequency 3 KHz sampled at Nyquist rate and encoded at the rate of 8 bit/sample.  Find the Datarate and Bandwidth required.

7.a)	Derive expression for channel capacity for infinite Bandwidth.

b)	A message source generates one of four messages randomly every microsecond.  The probabilities of these message are 0.4, 0.3, 0.2, 0.1.  Each message is independent of the other message in the sequence.  (i) What is the Source entropy.  (ii) What is rate of information generated by this source in bits /sec.

Contd………….2

Code No.221201				-2-				Set No.3

8.a)	Describe about parity check and algebric coding.

b)	A convolutional encoder has a rate r=1/2, constraint length K=2.  The code is systematic.  Find the encoder output produced by the message sequence 10111.

FF

i/p

n1

o/p	n2

- - -

Code No.221201

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Find the Fourier Transform of signal shown

1	g(t)

-1           0               1         t

b)	Prove the (i) time shifting property.  (ii) frequency shifting property.

2.a)	Explain (i) Gaussian probability Density  (ii) probability Density function.

b)	The PDF of a Gaussian variable x is given by Px(x)=13|2Πe−(x−4)218  Determine (i) P(x4)  (ii) P(x0).

3.a)	Describe the Vestigial sideband (VSB).  What is the bandwidth required in VSB system?

b)	Compare the AM and FM transmission system.

4.a)	Describe about synchronous Detection.  How the carrier is acquired at Receiver.

b)	For an AM DSBFC transmitter with an unmodulated carrier power Pc=100w that is modulated simultaneously by the three modulating signals with coefficient of Modulation m1=0.2, m2=0.4 and m3=0.5, determine the (i) Total coefficient of Modulation.  (ii) Upper and lower sideband power (iii) Total transmitted power (iv) Power saving when one sideband is suppressed.

5.a)	What is inter symbol interference(ISI).  How to reduce the ISI.  What are causes to get ISI?

b)	Derive the quantization noise power and signal to noise ratio in the case uniform quantization.

6.a)	What is slope over load distortion and granular noise in DM.  Explain.  How to reduce the above distortions.

b)	Describe about Adaptive Delta Modulation.

Contd…….2

Code No.221201				    -2-				Set No.4

7.a)	What is Shannon’s theorem?  Explain trade off between the Bandwidth and signal to noise ratio of a channel capacity.

b)	What is source coding?  Explain about algebric coding.

8.a)	A source is delivering 3 messages with probabilities ½, ¼, ¼.  What is the entropy of second order source extension.

b)	Describe about Convolutional Code.

- - -