Code No. 220301

Set No.

1

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

MECHANICS OF FLUIDS

(Common to Mechanical Engineering, Metallurgical Engineering and Metallurgy & Material Technology)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Define the following properties of fluids.

- Specific weight (ii) Density (iii) Compressibility (iv) Vapor pressure.

Write the units of above properties in “SI system”.

b)	What is meant by “Compressible fluid”?  How is it different from incompressible fluid?

c)	Describe any one type of Viscometer used in industry, with a neat schematic diagram.

2. a)	A barometric pressure of the atmosphere at sea level is equivalent to 760 mm Hg and its temperature is 288 k.  The temperature decreases with increasing altitude at the rate of 6.5 K per 1000 m until the stratosphere is reached in which the temperature remains constant at 216.5 K.  Calculate the pressure in mm of mercury and the density in Kg/m3 at an attitude of 14,500 m. Assume, R = 287 KJ/Kg K.

b)	The top of the inverted U-tube manometer is filled with of specific gravity 0.98 and the remainder of the tube with water of specific gravity.  1.01.  Find the pressure difference in Pascals between two points at the same level at the base of the legs when the difference of water level is 75 mm.

3. a)	Derive an equation for law of conservation of mass in cylindrical coordinates.

b)	In a 2-D flow (& incompressible) the fluid velocity components are given by     u= x-4y and v= -y-4x.  Show that the flow satisfies the continuity equation and obtain expressions for velocity potential and stream function.

4. a)	What is meant by hydrodynamic boundary layer?  Discuss the development of a boundary layer over a flat plate from leading edge.

b)	An oil having a viscosity of 0.048 Kg/ms flows through a 50 mm diameter tube at an average velocity of 0.12 m/s calculate the pressure drop in 65 m of tube and the velocity, 10 mm from the tube wall.

5. a)	Explain formation of Mach cone and define the terms Mach angle and Mach number.

b)	Calculate the maximum mass flow possible through a frictionless, heat insulated, convergent nozzle if the entry or stagnation condition are 5 bar and 15oc and the throat area is 6.5 cm2.  Also calculate the temperature of the air at the throat.  Take Cp= 1.005 kJ/kg K, = 1.4.

Contd..2

#### Code No. 220301				.2.				    Set No. 1

6. a)	Define the terms (i) Hydraulic grade line and (ii) Energy Grade line.

Deduce an expression for head loss due to friction, assume usual notations.

b)	An air duct, carrying a volume Q of air per second, is abruptly changed in section.  Deduce the diameter ratio for the two duct sections if the pressure loss is to be independent of flow direction.  Assume a value of 0.6 for the contraction coefficient.

7. a)	Obtain a relation for maximum power through a given pipe line.  Follow the usual notations.

b)	Water is supplied from a reservoir through a 300 m diameter pipe 600 m long to a nozzle which is situated 108 m below the free surface of the greatest possible power of the issuing jet.

8.	Write a short notes on the following:

- Pressure gauges

- Hot wire anemometer.

###

Code No. 220301

Set No.

2

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

MECHANICS OF FLUIDS

(Common to Mechanical Engineering, Metallurgical Engineering and Metallurgy & Material Technology)

# Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Explain what do you mean by capilarity.

b)	An open cylindrical bucket 30 cm in diameter and 50 cm long whose wall thickness and weight can be considered negligible is forced open and first in to water until its lower edge is 4 m below in surface.  What force will be required to maintain this position assuming the trapped air to maintain at constant temperature during the entire operation?

2. a)	How is Bernoulis equation related to the first law of Thermodynamics?

b)	A blower having an efficiency of 75% is to supply 14 cubic metres of air per minute to a 15 cm pipe under pressure equivalent of 5 cm of water.  If 30 cm intake pipe draws directly from the atmosphere, what horse power motor should be provided?  What will be the pressure intensity in the intake pipe?

air = 1.284 Kg/m3.

3. a)	What is a laminar sub-layer?

*

b)	For a steady poisenille flow in a pipe of Radius R obtain an expression for .

4. a)	For compressible adiabatic flow, will the entropy increase, decrease N constant?  Explain.

b)	At a certain section of duct in which air is flowing at a temperature of 32oc and pressure of 0.815 kgf/cm2 with a velocity of 365 m/sec.  Assume isentropic flow, determine.

- the velocity and temperature at a section where the pressure is 1.25 kgf/m2 and

- the Mach number at both the sections.

5. a)	Enumarate distinguishing characteristics of Laminar flow.

b)	Find the horse power required per 1000 m of pipe line to overcome the viscous resistance to flow of oil, density = 0.9 gm / cc,  N= 1.9 poise, through a horizontal 8 cm diameter pipe at the rate of 19 m3/hr.

Contd…2

#### Code No: 220301				.2.				Set No. 2

6. a)	What are the causes of loss of energy in pipe bends?

b)	Two pipes each of length L and diameters D1 and D2 are arranged in parallel; the loss of head, when a total quantity of Q flows through them, being h1 of the pipes are arranged in series, and the same quantity of water Q, flows through them, the loss of head is h2.  If D1 = 2D2 find the ration of h1 and h2.  Neglect minor loss and friction factor, f, to be constant.

7. a)	Under what circumstances will an orifice function as a large orifice?

b)	A Reservoir has an area of 90,000 m2 and is provided with Francis Weir, 5 m long with end conditions.

- If the initial reservoir level is 1 m above the sill of the weir, how long will it take for the water-level to fall to 50 cm above the still?

- What would be the time required for the water level to reach the sill level?

8.	Write short notes an any TWO of the following.

  - Drag and Lift forces.

  - Equation of energy.

  - Hydraulic gradiant.

###

Code No. 220301

Set No.

3

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

MECHANICS OF FLUIDS

(Common to Mechanical Engineering, Metallurgical Engineering and Metallurgy and Material Technology)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Explain the phenomenon of surface tension and also explain why does it occur.  Derive expression for excess pressure in a droplet.

b)	A rectangular surface 2m wide and 3m high is subjected to water pressure upto the top.  A layer of oil of specific gravity 0.8 and 2m thick acts on top of water.  Determine the total pressure on the surface and its point of application.

2. a)	Derive the continuity equation in a steady incompressible three dimensional flow.

b)	The velocity components in a two dimensional flow are u= y3/3 + 2x – x2y and     = xy2-2y-x3/3 check whether the flow is possible.  Find the stream functions and potential functions.

3. a)	Derive an expression for kinetic energy correction factor and write the Bernoulli’s equation for real fluids with this factor.

b)	Find the force on a nozzle 25 mm in diameter filled to a 75 mm diameter pipe line through which the discharge is 15 lit/sec.

4.	A smooth flat plate of 2.0 m wide and 2.5 m long is towed in oil of relative density 0.8 at a velocity of 1.5 m/s along its length.  Find the thickness of Boundary layer and shear stress at

- the centre and

- the trailing edge of the plate.  Also find the power required for towing the plate. Kinematic viscosity of oil is 10-2 m2/s.

5. a)	Draw shear stress and velocity distribution curves at a section in a laminar flow in a circular pipe.  Derive an expression for velocity in laminar flow.

b)	Oil of relative density 0.92 and dynamic viscosity 1.05 poise flows between two fixed parallel plates kept 1.2 cm apart.  If the mean velocity is 1.4 m/s.  Calculate

- The maximum velocity

- Velocity and shear stress at a distance of 0.2 cm from one of the plates and

- Head loss in a distance of 25 m.

### Contd…2

#### Code No. 220301				..2..				Set No. 3

6. a)	Derive Darcy-Weisbach equation and mention the factors effecting the friction factor.

b)	A 6 cm diameter horizontal pipe has a discharge of 450 l/min.  At a section the pipe has a sudden expansion to a size of 9 cm diameter.  If the pressure just on the upstream side of the expansion is 29 KN/m2,  calculate the pressure just after the expansion.

7. a)	Explain clearly the construction and working of pitot-static tube.

b)	A vertical venturimeter 40cm x 20cm is provided in a vertical pipe to measure the flow of oil of relative density 0.8.  The difference in elevation of throat section and the entrance section is 1m, the direction of flow of oil being vertically upwards.  The oil mercury differential gauge shows deflection of mercury equal to 40 cm.  Neglecting the losses, determine the quantity of oil flowing in the pipe.

8.	Write notes on any two of the following:

- Energy equation for compressible flows.

- Triangular notch

- Pressure gauges.

###

Set No.

4

# Code No. 220301

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

MECHANICS OF FLUIDS

(Common to Mechanical Engineering, Metallurgical Engineering and Metallurgy and Material Technology)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	The viscosity of liquids decreases with increase in temperature while that of gases increases.  Explain.

b)	A vertical gate of width 2.0 m and height 2.5 m controls a sluice opening in a dam.  The top of the gate is 10m below the water surface.  If the weight of the gate is  80 KN, find the vertical force required to raise the gate. The coefficient of friction between the gate and the guides can be assumed to be 0.25.

2. a)	Differentiate between steady flow and uniform flow.

b)	The flow field is given by

u= xy	and = 0.5 (x2-y2).

Verify whether the flow is irrotational.  Determine the stream function and potential function of this flow.

3. 	The velocity distribution in a circular pipe of radius R is given by

u 	=	1 – r   n

um		     R

where u is the velocity at any radial distance r from the axis of the pipe, um is the maximum velocity at the axis of the pipe and n is a constant.  Determine the energy correction and momentum correction factors for this flow.

4. a)	Explain what do you understand by sonic flow and shock  wave.

b)	If the boundary layer over a flat plate, kept parallel to the flow, is laminar find the ratios of the skin friction drag on the front half of the plate to the rear half.

c)	Bring out the difference between friction drag and form drag.

5. a)	Show from fundamentals that the velocity distribution in a laminar flow through a circular pipe is parabolic.

b)	An oil of relative density 0.92 and viscosity 0.9 poise flows through a 10 cm diameter pipe 30 m long.  Determine the largest flow that can be passed through this pipe while maintaining laminar regime.  What is the head loss between the two ends of the pipe under this flow?

Contd…2

#### Code No. 220301				..2..				    Set No. 4

6. a)	How does the Darey’s friction factors f vary with the Reynold’s number of the flow.  What is the relation between f and Rc for laminar flow in circular pipes?

b)	When a sudden contractions from 50 cm diameter to 25 cm diameter is introduced in a horizontal pipe line, the pressure changes from 105 kPa to 69 kPa.  Assuming Cc = 0.65.  Calculate the discharge in the pipe.

7. a)	Explain the principle of working of a hot wire anemometer to measure the velocity of gases.

b)	A venturi  meter 300 mm x 150 mm is used to measure the discharge of water in a 300 mm dia horizontal pipe.  If the pressure difference between entrance and throat is 3 m of water and if the long head between the same sections is 1/8 of velocity head at the throat, calculate the discharge in the pipe and the coefficient  of the meter.

8. 	Write short notes on any TWO of the following.

- Streamline, patterline and streakline.

- Orifice meter

- Boundary layer separation.

###