Code No:220553

OR

II.B.Tech II-Semester. Supplementary Examinations. January,2003

SWITCHING AND LOGIC THEORY THROUGH DIGITAL ICs

(Common to Computer Science and Engineering, Computer Science and Information Technology)

Time: 3 hours.							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain why NAND  and NOR gates are called universal gates.

b)	Compare the relative merits and demerits of CMOS logic gates.

c)	Draw the circuit diagram of a Medium speed TTL NAND gate and explain its operation.  Explain why TTL IC’s are unsuitable in noisy environments.

2.a)	Simplify the following expression and implement it with two level NOR Gate circuit.

F(A,B,C,D) =  (0,2,4,5,6,7,8,10,13,15).

- Prove that the sum of all min terms of three Boolean variables is equal to one.

- Show that the dual of EX-OR is equal to its compliment.

3.a)	Explain the working of a 4-bit serial adder with a neat diagram.

b)	Draw the logic diagram of a 1 bit digital comparator and explain its operation.

4.a)	Design a 4 line to 2 line parity encoder.  Using this realize a 12 line to 4 line priority encoder.

b)	Design and realize a 4 bit shift register using J-k Flip-Flop.  If the initial state  in 1001,  What is the final state after 4 clock pulses.

5.a)	Design a MOD 6 counter where 001 and 100 are unused states.  The counter should be free of lock out.

b)	Show the realization of 4kx8 RAM using 1kx4 RAM chips.

6.	Design a two-input, two-output synchronous sequential circuit which produces an output z=1 whenever any of the following sequences occur: 1100,1010 OR 1001.  The circuit resets to its initial state after a 1 output has been generated.

(i)	Form the state table and state diagram.

(ii)	Choose an assignment, and show the excitation functions for JK flip-flops.

7.	Write short notes on any THREE:

(a) Interfacing CMOS with TTL (b) SOP and POS forms

(c) Multiplexer (d) T and D Flip-Flops.

8.	Write short notes on any THREE:

a)     Mater slove J-k flip-flop   (b)	Excess - 3 code   (c)	PLA and PAL

d)    ROM as code converter.

**********