Code No.221003 Set No. 1

II-B.Tech. II-Semester Supplementary Examination January - 2003

PULSE AND DIGITAL CIRCUITS

(Common to  Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	A symmetrical square wave of peak to peak amplitude ‘V’ and frequency ‘f’ is applied to a High Pass RC circuit. Show that the percentage tilt is given by:

1-e-1/2fRC

P =    ---------------

1+e-1/2fRC

b)	A pulse of amplitude 5V and duration 10sec is applied to a Low Pass RC circuit having R = 10k and C = 1000pf. Calculate the output o(t) and sketch the waveform. What is the value of upper 3dB frequency to minimize distortion. Derive the necessary expressions.

2.a)	Sketch the double-ended clipper circuit using two Zener diodes and explain the working with sine wave input.

b)	For the figure 1 shown below, assume that the diodes are ideal. Make a plot of  Vo against Vi for the range of Vi from oV to 150V. Indicate all slopes and voltage levels.

Figure.1

3.a)	State and prove Clamping theorem.

b)	In the restorer circuit shown in figure.2, Rf = 100, Rh = , Vr = 0, for the diode D. The input Vs = 0 for t<0 and t>0 is a square wave of frequency 5kHz and makes excursions between 0 and 10V. The capacitor is initially less charged. Draw the output waveform for the first three cycles labeling all voltage levels and time constants.

Figure.2

(contd…2)

Code No.221003				-2-			Set No.1

4.a)	For a transistor switch, sketch the waveforms of the input voltage, base current, collector current, and collector voltage. Show the various components of transistor turn on time and turn-off time and discuss their origins.

- For a common emitter circuit, Vcc = +10V, Rc = 1k, IB = 0.2mA. Determine

  - The value of hFE(min) for saturation to occur.

  - If Rc is changed to 220, will the transistor be saturated?

5.a)	Explain the different triggering methods of bistable circuits.

b)	Design an Astable multivibrator to have a 5kHz square wave with 40% duty cycle and amplitude 10V. Use NPN transistors having hfe = 100, VCE(sat) = 0.3V, VBE(sat) = 0.7V and Ic(max) = 100 mA. Draw the circuit and sketch the waveforms seen at both the collectors and bases.

6.a)	Draw the circuit of a Bootstrap voltage sweep generator and explain its operation. Derive the expressions for its sweep period, retrace period and recovery time.

b)	Describe the working of a Linear current sweep circuit. Sketch the relevant waveforms.

7.a)	Explain the necessity and principle of synchronization for waveform generators.

b)	Explain the method of synchronization applied to Astable Multivibrator. Draw the circuit diagram and the necessary waveforms.

8.	Write short notes on any THREE of the following:

- Blocking Oscillators

- Unidirectional sampling gates

- Frequency division in sweep circuits

- RL and RLC circuits.

***     ***     ***

Code No.221003

Set No.

2

II-B.Tech. II-Semester Supplementary Examination January - 2003

PULSE AND DIGITAL CIRCUITS

(Common to  Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Obtain the response of a low pass RC circuit for an exponential input. Sketch the waveforms.

b)	A pulse of amplitude 5v and duration 20µ sec is applied to a High pass RC circuit having R=10k and C=1000pf. Calculate the output Vo(t). Sketch the output waveform. Calculate the tilt and undershoot. Derive the necessary expressions used.

2.a)	Draw a typical circuit for clipping at two independent levels. Explain in detail. Sketch its transfer characteristics.

b)	In the figure.1 shown below, assume that the diodes are ideal. Make a plot of Vo against Vi for the range of Vi from 0V to 50V. Indicate all slopes and voltage levels.

Figure.1

3.a)	Explain the operation of a diode comparator. Draw the necessary circuit and waveforms.

b)	A symmetrical 10kHz square wave whose peak to peak excursions are ±10V with respect to ground is impressed upon the diode clamping circuit shown in figure.2. The diode has rf = 100, rr =  and Vn = 0. Sketch the steady state output waveform indicating clearly the voltage levels.

Figure.2

Contd…2.

Code No.221003				-2-			Set No.2

4.a)	Sketch typical transistor common emitters characteristics. Identify the various regions of the characteristics, and show how VCE(sat) differs with different load resistances.

b)	Draw the circuit diagram of a collector coupled monostable multivibrator and derive the expression for gate width (T).

5.a)	Explain the operation of a schmitt Trigger circuit. Derive the expressions for UTP and LTP. Sketch its transfer characteristics. Discuss one application of the circuit.

b)	For a self biased Binary (symmetrical), Vcc = 20V, Rc = 1.5k, Re = 0.3k, R1 = 15k, R2 = 100k and C = 100pf. Determine the biasing conditions of the two transistors in ON and OFF states and the maximum operating frequency of the Binary.

6.a)	Explain different methods of linearisation in sweep circuits. Draw the relevant waveforms.

- Draw the circuit diagram of a Miller time base generator and explain its operation.

7.	Draw the circuit of an Astable blocking oscillator with resistor and capacitor in the emitter and in the base. Explain its operation, draw the waveforms across RC combination and derive the necessary expressions.

8.	Write short notes on any THREE of the following:

- Unidirectional sampling gates

- Phase delay and pulse filters

- Ringing circuits

- Voltage comparators.

***     ***     ***

Code No.221003

Set No.

3

II-B.Tech. II-Semester Supplementary Examination January – 2003

PULSE AND DIGITAL CIRCUITS

(Common to  Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	A square wave of time period ‘T’ is applied to an ‘RC’ High Pass circuit whose time constant ‘T’ is variable. If ‘V’ is the peak to peak voltage of square waveform, draw and theoretically justify the shapes of the waveform that result in the output for   (i) T<<	(ii) T = 	(iii) T>>

Draw the waveforms of input and output to the same time scale.

b)	Prove that an RC circuit behaves as a reasonably good integrator if RC>>15T, where ‘T’ is the period of an input sinusoid Emsinwt.

2.a)	Define the difference between clipping and clamping circuits. A +  10V square wave is applied to the input of a negative voltage clamping circuit and to the input of a negative shunt clipper. Sketch the output waveforms that will result in each case.

- Explain the operation of Transistor Clippers in detail. Sketch the waveforms.

3.a)	Explain the effect of diode characteristics on clamping voltage, with necessary output waveforms.

b)	An unsymmetrical square wave with T1 = 1msec and T2 = 1µsec has an amplitude of 10V. This signal is applied to the restorer circuit shown in figure.1 in which Rf = 50, R = 50k, RS = 0. Assume that the capacitor C is arbitrarily large, so that the output is a square wave without tilt. Find

i)   Where, on the waveform zero level is located

ii)  If T1 = 1 µ sec and T2 = 1 msec, find the location of the Zero level

iii) If the diode is inverted, locate the Zero level.

Figure.1

(contd…2)

Code No.221003				-2-			Set No.3

4.a)	Show how a speed-up capacitor may be employed to improve the turn-on and turn-off time of a  transistor. Sketch the waveforms of the base current with and without speed-up capacitor.

b)	A common emitter circuit has Vcc = 25V and a collector resistor, which can be either 22k or 2.2k. Calculate the minimum level of base current to achieve saturation in each case.

5.a)	Draw the circuit of Miller sweep generator. Sketch the timing diagrams and explain its operation.

b)	A fixed biased binary circuit has Rc = 1k, R1 = 3.9k, R2 = 39k, Vcc = +10V, VEE = -10V. Assume VCE(sat) = 0.2V, VBE(sat) = 0.7V. Find the stable state voltages and currents. What is the minimum value of ‘hfe’ to satisfy the ON-OFF conditions of the Binary?

6.a)	Draw the circuit diagram of a Miller voltage sweep circuit and explain its operation with waveforms.

b)	Draw a circuit to generate linear current sweep. Explain the operation with waveforms.

7.	Explain the operation of a triggered transistor blocking oscillator with emitter timing. Draw the circuit diagram. Sketch the current and voltage waveforms. Derive the expressions for the pulsewidth at the output. Explain Recovery and loading considerations.

8.	Write short notes on any THREE  of the following:

- Bidirectional sampling gates

- Diode as a switch

- Schmit trigger

- Attenuators.

***     ***     ***

Code No.221003

Set No.

4

II-B.Tech. II-Semester Supplementary Examination January - 2003

PULSE AND DIGITAL CIRCUITS

(Common to  Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Draw the circuits of Integrator and differentiator using RC elements. Derive the necessary conditions for satisfactory performance.

b)	A square wave with peak to peak value 1V, ON time 0.1sec and OFF time 0.2sec is impressed upon an RC differentiating circuit with time constant 0.2sec. sketch the steady state output waveform showing all the details.

2.a)	Sketch the circuit of a positive series clipper, showing the input and output waveforms. Explain its operation.

b)	For the diode wave shaping cir4cuit shown in figure.1, calculate and plot the transmission characteristics. Assume non-ideal diode with Rr = 100k, Vr = 0.6V.

figure.1

3.a)	Sketch a negative voltage clamping circuit showing input and output waveforms. Explain the operation of the circuit.

b)	A symmetrical 10kHz square wave whose peak to peak excursions are ±10V with respect to ground is impressed upon the diode clamping circuit shown in figure.2. Diode has rf = 100, rr =  and Vr = 0V. Sketch the steady state output waveform indicating clearly the voltage levels.

Figure	-2

(contd…2)

Code No.221003				-2-			Set No.4

4.a)	Sketch ideal diode characteristics and approximate characteristics for silicon and germanion diodes. Discuss the parameters that should be considered when selecting a diode. Explain the origin of reverse recovery time, by means of sketches.

b)	For a CE transistor circuit with Vcc = 10V, Rc = 1k, calculate the transistor power dissipation	(i) At cutoff 	(ii) At saturation.

5.a)	Derive the expression for output time period of monostable multivibrator that uses BJTs.

b)	Design a self biased symmetrical binary with the following specifications.         Vcc = 10V, Rc = 1k, VBE(sat) = 0.7V, VCE(sat) = 0.3V, BON = 20. Operating frequency upto 80 kHz, impedances of the triggering source = 250.

6.a)	An exponential sweep is produced by charging the capacitor. Explain how such a sweep is linearized using the Bootstrap and Miller technique. Derive the expression for the sweep speed error in each case.

b)	Draw a circuit to generate a linear current sweep. What is the necessity of generating a trapezoidal waveform?

7.a)	Explain the synchronization of sweep circuits with symmetrical signals, in detail.

- Explain the method of Frequency division in the sweep circuits.

8.	Write short notes on any THREE of the following:

- Bidirectional sampling gates

- Astable blocking oscillators

- Breakdown voltage consideration of transistor

- Blocking oscillators.

***     ***     ***