Code No.220206 Set No. 1

II-B.Tech. II-Semester Supplementary Examination January - 2003

CONTROL SYSTEMS

(Electrical and Electronics  Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain the differences between open loop and closed loop control systems with examples.

- Obtain the transfer function for the electrical system shown below.

2.a)	Explain the armature voltage controlled DC servomotor and obtain its transfer function.

- Draw the connection diagram of a synchro pair used as a position indicator.

3.a)	Find the step, ramp and parabolic error coefficients and their corresponding steady state errors for unity feedback control system having tad transfer function:

14 (s+3)

G(s) = ---------------------

s(s+5)(s2+2s+5)

- A unity feedback control system has the forward transfer function:

25

G(s) = ---------

S(s+6)

Find the rise time, peak time and the maximum over shoot for unit step input.

4.a)	Define BIBO stability of a feedback control system.

b)	A unity feedback control system is characterized by the open-loop transfer function

k(s+13)

G(s) = ----------------

s(s+3)(s+7)

i)   Using the Routh criterion, calculate the range of values of K for the system   to          be stable.

ii) Check of for k=1, all the roots of the characteristic equation of the above    system have damping factor greater than 0.5.

(contd…2)

Code No.220206			-2-			Set No.1

5.	A unity feedback system has an open-loop transfer function G(s) = K(s+1)/s(s-1).

Sketch the root locus plot with K has a variable parameter and show that the loci of complex roots are part of a circle with (-1,0) as center and radius 2

Is the system stable for all values of K? If not, determine the range of K for stable system operation. Find also the marginal value of K which causes sustained oscillations and the frequency of these oscillations.

6.	Draw the Bode magnitude and phase plots for the following transfer function:

10

G(s) =  -----------------

S(s2+0.4s+4)

Also obtain the gain margin and phase margin from the plots.

7.	Sketch the complete Nyquist plot for the following loop transfer function

10

G(s) H(s) = ---------------------------

S2(1+0.25s)(1+0.5s)

And test its stability under closed-loop condition.

8.a)	For the system shown below, obtain state variable model:

- Find the response of the system

x1	 	1   0           x1			  1

=   	+	u(t)

x2 	         1   1	   x2		       1

where u(t) is unit step function.

x1(0) = 1, x2(0) = 0.

***     ***     ***

Code No.220206 Set No. 2

II-B.Tech. II-Semester Supplementary Examination January - 2003

CONTROL SYSTEMS

(Electrical and Electronics  Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Find the transfer function of a field controlled d.c.motor.

- What are the various types of control systems? Given an example for each. What are the merits and demerits of open loop and closed loop systems.

2.a)	Explain what is a closed loop system? Explain the effects of feedback on control system.

- Explain Mason’s gain formula for signal flow graph analysis.

k

3.	In a closed loop control system, the open loop transfer function G(s) = ---

s2

and feedback transfer function H(s) = as+b. If k = 20, find the values of a,b so that overshoot is 16% and the time constant is 0.1sec. Also determine the steady state error, if the input to the system is a unit ramp.

4.a)	The open-loop transfer function of a Servo system with a unity feedback is

400

G(s) = ------------

s(1+0.1s)

Determine the dynamic error for a input r(t) = 1+2t+t2 by using the dynamic error coefficients.

- The open-loop transfer function of a unity feedback control system is given by

k

G(s) =  --------------------------

(s+2)(s+4)(s2+6s+25)

by applying the Routh criterion, discuss the stability of the closed-loop system as a function of k.

(contd…2)

Code No.220206			-2-			Set No.2

5.a)	Discuss the rules of construction of root loci.

- A unity feedback control system has the following open loop transfer function:

K(s+3)

G(s) = ---------------------------

S(s2+2s+2)(s+5)(s+6)

Sketch the root locus diagram. Calculate the value of K corresponding to the damping ratio of 0.42.

6.	Sketch the Bode magnitude and phase plots of a closed-loop system which has the open-loop transfer function

2e-ST

G(s) = --------------------

S(1+s)(1+0.5s)

Determine the maximum value of T for the system to be stable.

7.a)	Explain the Nyquist criterion for accessing the stability of a closed-loop control system.

- Sketch the Nyquist plot of the transfer function:

100

G(s) H(s) =    ----------------------

S(s+1)(s2+2s+2)

Discuss its stability.

8.a)	Find the solution of the following state model

1   0	          1			1

x = 		X +	     u(t) 	with X(0) =

1   1	          1			1

y =     1   0   X

- Obtain the state model of the system whose transfer function is given on

Y(s)		10

-----   =  -----------------

U(s) 	   s3+4s2+2s+1

***     ***     ***

Code No.220206 Set No. 3

II-B.Tech. II-Semester Supplementary Examination January - 2003

CONTROL SYSTEMS

(Electrical and Electronics  Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Explain, with examples, the use of control system concepts to engineering and non engineering fields.

b)	Obtain the transfer function X(s)/F(s) for the mechanical system shown below in figure (p1b)

Figure(p1.b)

2.a)	Explain what is a closed loop system? Explain the effects of feedback on control system.

b)	Obtain the overall transfer function C/R by using block diagram reduction technique for following system shown in figure (2pb)

Figure(2pb)

3.a)	What are the various standard test signals? Explain the importance of a unit impulse and unit step function input for a second order system.

b)	Derive the relations for specifications for the time domain response of a second order system, subjected to a step input.

(contd…2)

Code No.220206			-2-			Set No.3

4.a)	Explain the concepts of stability of a control system.

b)	A system oscillates with frequency w, if it has poles at s = ±j'w and no poles in the right-half s-plane. Determine values of k and a, so that the system shown in figure (4pb) oscillates at a frequency 2 rad/sec.

Figure(4pb)

5.	The characteristic equation of a feedback control system is

s4+3s3+12s2+(k-16)s+k = 0

Sketch the root locus plot for 0  k < ∝ and so that the system is conditionally stable (stable for only a range of gain k). Determine the range of gain for which the system is stable.

6.	Sketch the Bode plot for a unity feedback system, characterized by the open-loop transfer function

K(1+0.2s)(1+0.025s)

G(s) = ------------------------------

s3(1+0.001s)(1+0.005s)

Show that the system is conditionally stable. Find the range of values of K for which the system is stable.

7.a)	For the given unity feedback system with transfer function

k(1+0.5s)(s+1)

G(s) H(s) =   --------------------

(1+10s)(s-1)

Sketch the Nyquist plot and determine the range of k for which the system stable.

- Explain the phase margin and gain margin with respect to Nyquist criteria.

8.a)	Write short notes on compensators.

- Obtain the solution for y(t) of the following system

̣

x1		0   -1	x1		1

̣	=  				+		u(t)

x2	           -2   -1		   x2		1

y(t) =    1    2	x1

x2

Where u(t)  is a unit step function and x1(0) = 1 and x2(0) = 1.

***     ***     ***

Code No.220206 Set No. 4

II-B.Tech. II-Semester Supplementary Examination January - 2003

CONTROL SYSTEMS

(Electrical and Electronics  Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	What are various effects of feedback control?

- Obtain the block diagram model of an armature controlled D.C.Servomotor.

2.	For the mechanical system given below in figure (2p).

- Write the mathematical model.

- Obtain the transfer functions

X1(s)		 X2(s)

--------   and 	-------

F(s) 		  F(s)

figure (p2)

3.a)	What are the time domain specification? Derive their relations for the time response of a second order system, subjected to a unit step input.

b)	Measurements conducted on a Servomechanism show the system response to be

c(t) = 1+0.2e-60t – 1.2e-10t

When subjected to a unit step-input.

- Obtain the expression for the closed-loop transfer function.

- Determine the undamped natural frequency and damping ratio of the system.

4.a)	i)   Explain error constants Kp, Kr, Ka for type I system.

ii)  Given the open-loop transfer function of a unity feedback system as

100

G(s) = ------------

S(0.1s+1)

Find Kp, Kv and Ka.

b)	The characteristic equation of a servo system is given by

a0s4+a1s3+a2s2+a3s+a4 = 0

Determine the conditions which must be satisfied by the coefficients of the characteristic equation for the system to be stable.

(contd…2)

Code No.220206			-2-			Set No.4

5.	Sketch the root locus plot of a unity feedback system with an open-loop transfer function

K

G(s) =  ---------------

S(s+2)(s+4)

Find the range of values of K for which the system has damped oscillatory response. What is the greatest value of K which can be used before continuous oscillations occur? Also determine the frequency of continuous oscillations.

Determine the value of K so that the dominant pair of complex poles of the system has a damping ratio of 0.5.

6.	Draw the Bode magnitude and phase diagrams for the system with the transfer function

200 (s+0.5)

G(s) =  ------------------

S(s+10) (s+50)

Also obtain the phase margin and gain margin from the plots.

7.	Draw the complete Nyquist plot for:

3(s+2)

G(s) H(s) =    ----------------

s3+3s+1

and discuss stability of the system.

8.a)	Consider the state-space model

̣

X = AX+Bυ

Y = CX+Dυ

Show that the above state-space model is not unique.

b)	Determine the state transition matrix for the following system

0     -1                      1

X =                         X  +                 U

-2     -1		          1

Y =    1     2      X

***     ***     ***