Code No. 220453

OR

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

E M WAVES AND TRANSIMISSION LINES

( Common to Electronics and Communications Engineering and Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 70

## Answer any Five questions

All questions carry equal marks

---

1. a)	What do you understand by the term ‘Polarization’ in dielectrics?  Explain.

b)	A charged wire with linear charge density of l c/m is in the form of a semi circle.  Find an expression for the electric field at the center of the circle.

2. a)	Define the term ‘Electric Potential’ in an electro static field.  What do you understand by ‘Absolute Potential’? Show that the potential difference between any  two points is independent of the path of integration?

- Prove that the electric field intensity in the negative gradient of potential.

3. a)	What is Bist Savart Law?  Using it find magnetic field on the axis of a circular loop carrying a current ‘I’.

- Explain the concepts of scalar and vector magnetic potentials.

4. a)	Derive Poisson’s equation for the vector magnetic potential of steady current.

- Explain the terms ‘self inductance’ and ‘mutual inductance’.

__     __    __

5. a)	Show that the vector product   P  =  E  x H represents the rate of energy flow per unit area at a point.

b)	A copper wire carries a conduction current of 1 ampere. Determine the displacement current in the wire at 100 MHz.  Take Î = Î0 ad conductivity        5.8x 107mho/m.

6. a)	Two infinitely parallel perfect conducting planes are separated by ‘a’ cm apart.  Justfy the existence of TMno node in this guide. Derive expression for its filed components and justify their nature.

b)	Deduce the two conditions for which a wave traveling from one meduim to the other will be total internal reflected.

7. a)	Derive the expression for input impedance of an uniform transmission line terminated in load ZL.  Hence discuss the properties of a quarter wave legth and half wavelength lines assuming the line to be loss less.

- Describe the consternation of Smith chart and give its application.

8.	Write short notes on the following:

- Energy stored in electrostatic field.

- Boundary condition of a EM field

- Circular wave guide.

- Stub matching.

###