Code No. 220102

Set No.

1

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

STRENGTH OF MATERIALS - II

(Civil Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1.	A rectangular block of material is subjected to a tensile stress of 30 N/mm2 on one plane and a tensile stress of 10 N/mm2 on a plane at right angles together with the shear stress of 10 N/mm2 on the same planes.  Find

- The magnitude of the principal stresses

- The direction of the principal planes.

- The magnitude of the greatest shear stress.

2.	Determine the diameter of a solid shaft which will transmit 300 kN at 250 r.p.m.  The maximum shear stress should not exceed 30 N/mm2 and twist should not be more than 1o in a shaft length of 2m.  Take modulus of rigidity = 1 x 105 N/mm2.

3. a)	A laminated Spring 1 m long is made up of plates each 5 cm wide and 1 cm thick.  If the bending stress in the plate is limited to 100 N/mm2, how many plates would be required to enable the spring to carry a central point load of 2 k N?                  If E = 2.1 x 105 N/mm2, what is the deflection under the load?

b)	A closely coiled helical spring is to carry a load of 500 N.  Its mean coil diameter is to be 10 times that of the wire diameter.  Calculate these diameters if the maximum shear stress in the material of the spring is to be 80 N/mm2.

4. a)	Derive the expression for critical load when one end of the column is fixed and the other end is free.

b)	A solid round bar 3.5m long and 5 cm in diameter is used as a struet with one end fixed and other end is free.  Determine crippling load when E = 2.0 x 105 N/mm2.

5.	A short column of rectangular cross section 80 mm  x  60 mm carries a load of 40kN at a point 20 mm from the longer side and 35 mm from the shorter side.  Determine the maximum compressive and tensile stresses in the section.

6.	A trapezoidal masonry dam having top width 1m and height 8 m, is retaining water upto a height of 7.5 m.  The water face of the dam is vertical.  The density of masonry is 2240 kg/m3 and co-efficient of friction between the dam and soil is 0.6.  Find the minimum bottom width of the dam required.

##### Contd.. 2

### Code No. 220102				.2.				Set No. 1

7.	A steel tube of 200 mm external diameter is to be shrunk on to another steel tube of 60 mm internal diameter. After shrinking the diameter at the junction is 120mm.  Before shrinking on the difference of diameter at the junction is 0.08mm.  Find the hoop stresses developed in the two tubes after shrinking on and the radial pressure at the junction.  Take E=2 x 105 N/mm2.

8.	A cast iron beam is of T-section as shown in Fig. 1.  The beam is simply supported on a span of 8m.  The beam carries a UDL of 1.5 kN/m length on the entire span.  Determine the maximum tensile and maximum compressive stresses.

100 mm																																			           20 mm																																																																	      80 mm																																								       20 mm

###

Code No. 220102

Set No.

2

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

STRENGTH OF MATERIALS - II

(Civil Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1.	A rectangular block of material is subjected to a tensile stress of 40 N/mm2 on one plane and a zero stress on a plane at right angles together with shear stress of 80N/mm2 on the same planes.

Find

- The magnitude of the principal stresses

- The direction of the principal planes.

- The magnitude of the greatest shear stress.

2. a)	A solid shaft in a rolling mill transmits 20 kW at 2 Hz.  Determine the diameter of the shaft if the shearing stress is not to exceed 40 MPa and the angle of twist is limited to 6o in a length of 3m.  Shear modulus is 83 GPa.

b)	Derive torsional equation T/J = q/r = N/L.

3.	The stiffness of a close-coiled helical spring is 1.5 N/mm of compression under a maximum load of 60 N.  The maximum shearing stress produced in the wire of the spring is 125 N/mm2.  The solid length of the spring (when the coils are touching) is given as 5cm.

Find:	i.	Diameter of wire

- Mean diameter of the coils and

- Number of coils required.

Take C= 4.5 x 104 N/mm2.

4. a)	Derive expression for critical load when one end of the column is fixed and the other end is hinged.

b)	A solid round bar 3 m long and 5 cm in diameter is used as a strut with both ends hinged.  Determine the crippling (or collapsing) load, (Take E= 2.0 x 105 N/mm2).

Contd..2

###### Code No. 220102					.2.			Set No. 2

5.	A rectangular pier of 1.5 m x 1.0 m is subjected to a compressive load of 450 kN as shown in Fig.1.  Find the stresses on all four corners of the pier.

Fig. 1

6.	A masonry trapezoidal dam 4m high, I m wide at its top and 3m width at its bottom retains water on its and 3 m width at its bottom retains water on its vertical face.  Determine the maximum and minimum stresses at the base:

- when the reservoir is full and

- when the reservoir is empty.  Take the weight density of masonry as 19.62kN/m3.

7.	A short column of external diameter 40cm and internal diameter 20 cm carries an eccentric load of 80kN.  Find the greatest eccentricity which the load can have without producing tension on the cross section.

8. a)	A pipe of 400mm internal diameter and 100 mm thickness contains a fluid at a pressure of 8 N/m2.  Find the maximum and minimum hoop stresses across the section.

b)	A circular ring is simply supported on 6 equally spaced columns.  Draw the shear force, bending and twisting moment diagram due to uniformly distributed load of intensity, w over the entire beam.

###

Code No. 220102

Set No.

3

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

STRENGTH OF MATERIALS - II

(Civil Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1.	A rectangular block of material is subjected to a compressive stress of 120 N/mm2 on one plane and a tensile stress of 50 N/mm2 on a plane which is perpendicular to it with the shear stress for 100 N/mm2 on the same planes.

Find	(i)	the magnitude of the principal stresses

- the direction of the principal planes

- the magnitude of  the greatest shear stresses.

2.	A solid cylindrical shaft is to transmit 300 kW power at 100 r.p.m

a)	If the shear stress is not to exceed 80 N/mm2, find its diameter.

b)	What percentage saving in weight would be obtained if this shaft is replaced by a hollow one whose internal diameter equals to 0.6 of the external diameter, the length, the material and ‘maximum’ shear stress being the same?

3. a)	A closely coiled helical spring of round steel wire 10 mm in diameter having 10 complete turns with a mean diameter of 12 cm is subjected to an axial load of 200N.  Determine (i) the deflection of the spring	(ii) maximum shear stress in the wire	(iii) Stiffness of the spring.  Take  C	=	8 x 104 N.mm2.

b)	A leaf spring carries a central load of 3000N.  The leaf spring is to be made of 10 steel plates  5 cm wide and 6mm thick.  If the bearing stress is limited to 150N/mm2 determine:

- length of the spring and

- deflection at the centre  of the spring.

Take E = 2 x 105 N/mm2.

4. a)	Derive the expression for critical load when both the ends of the column are hinged.

b)	A solid round bar 4 m long and 6 cm in diameter is used as a strut with both ends hinged.  Determine the crippling load.  Take E = 2.0 x105 N/mm2.

5.	A rectangular column of width 120mm and of thickness 100mm carries a point load of 120kN at an eccentricity of 10mm.  Determine the maximum and minimum stresses at the base of the column.

6.	A trapezoidal masonry dam is of 18 m height. The dam is having water upto a depth of 15m on its vertical side.  The top and bottom width of the dam are 4m and 8m respectively.  The weight density of the masonry is given as 19.62 kN/m3. Determine the maximum and minimum stress intensities at the base.

##### Contd…2

### Code No. 220102				.2.				Set No. 3

7.	A compound tube is composed of a tube 250mm internal diameter and 25mm thick shrunk on a tube of 250mm external diameter and 25 mm thick.  The radial pressure at the junction is 8 N/mm2. The compound tube is subjected to an internal fluid pressure of 84.5 N/mm2.  Find the variation of the hoop stress over the wall of the compound tube.

8.	A cast iron beam is of I-section as shown in Fig.1.  The beam is simply supported on a span of 5 metres.  If the tensile stress is not to exceed 20 N/mm2, find the safe uniformly load which the beam can carry.  Find also the maximum compressive stress.

80 mm

20 mm

200 mm

20 mm

40 mm

160 mm

###

Code No. 220102

Set No.

4

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

STRENGTH OF MATERIALS - II

(Civil Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1.	A rectangular block of material is subjected to a tensile stress of 140 N/mm2 on one plane and a tensile stress of 20 N/mm2 on plane at right angles together with the shear stress of 60 N/mm2 on the same planes.

Find

- The magnitude of the Principal stresses

- The direction of the principal planes

- The magnitude of the greatest shear stresses.

2.	Two shafts of the same material and of same lengths are subjected to the same torque.  If the first shaft is of a solid circular section and the second shaft is of hollow circular section, whose internal diameter is 2/3 of the outside diameter.  If the maximum shear stress developed in each shaft is the same then compare the weights of the shafts.

3.	A close-coiled helical spring has a stiffness of 10 N/mm.  Its length when fully compressed, with adjacent coils touching each other is 40cm.  The modulus of rigidity of the material of the spring is 0.8 x 105 N/mm2.

- Determine the wire diameter and mean coil diameter if their ratio is 1/10

- If the gap between any two adjacent coil is 0.2 cm, what maximum load can be applied before the spring becomes solid, i.e., adjacent coils touch?

- What is the corresponding maximum shear stress in the spring?

4. a)	Derive expression for critical load when both ends are fixed.

b)	A solid round bar 3m long and 5 cm in diameter is used as a strut with both ends are fixed.  Determine the crippling load.  Take E = 2.0 x 105 N/mm2.

#### Contd…2

### Code No: 220102				.2.				Set No. 4

5. 	A masonry pier of 3m x4m supports a vertical load of 80 kN as shown in Fig.1

- Find the stresses developed at each corner of the pier.

- What additional load should be placed at the center of the pier, so that there is no tension anywhere in the pier section?

- What are the stresses at the corners with the additional load in the center?

Fig-1.

6.	A trapezoidal masonry dam having 4 m top width, 8 m bottom width and 12 m high, is retaining water upto a height of 10 m.  The density of masonry is 2000 kg/m3 and  co-efficient of friction between the dam and soil is 0.55.  The allowable compressive stress is 343.35 kN/m2. Check the stability of dam.

7. a)	A thick spherical shell of 100 mm internal diameter is subjected to an internal fluid pressure of 30 N/mm2.  If the permissible tensile stress is 80 N/mm2.  Find the thickness of the shell.

b)	A pipe of 200 mm internal diameter and 100 mm thickness contains a fluid at a pressure of 6 N/mm2.  Find the maximum and minimum hoop stress across the section.

8.	A cast iron bracket subject to bending has the cross-section of I-form with unequal flanges.  The dimensions of the section are as shown in fig. 2.  Find the position of the neutral axis and the moment of inertia of the section about the neutral axis.  If the maximum bending moment on the section is 40 MN-mm, determine the maximum bending stress.  What is the nature of the stress.

200 mm

50 mm

50 mm			200 mm

50 mm

130 mm

###