Code No:220205

Set No.

1

# II B.Tech. II Semester Supplementary Examinations January,  2003

# ELECTRO MECHANICS - II

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	In a 1- transformer, the cOre flux remains substantially constant and in independent of load current.  Explain?  In a transformer, find the load current at which maximum efficiency occurs.

b)	A 100 KVA, 2400/240V, 50 Hz 1- transformer has an exciting current of 0.64A and core loss of 700 watts, when its high voltage side is energized at rated voltage and frequency.  Calculate two components of exciting current.

2.  a)	Define voltage regulation of a transformer.  Deduce the expression for voltage regulation?

b)	A 100 KVA transformer has 400 turns as primary and 80 turns as secondary.  The primary and secondary resistances are 0.3 ohms and 0.01 ohms respectively and the corresponding leakage reactances are 1.1 and 0.035 ohms respectively.  The supply voltage is 2200 V Calculate (i) equivalent Impedance referred to primary (ii) voltage regulation and secondary terminal voltage for full load having a p.f. of 0.8 leading.

3.  a)	Mention different types of starters used for starting of Induction motors.  Explain any one of them with a neat sketch and necessary derivation?

b)	A 3- Induction Motor has a 4-pole, star connected stator winding and runs as 220V, 50 Hz supply.  The rotor resistance perphase is 0.1 and reactance 0.9.  The Ratio of stator to rotor turns is 1.75.  The full load slip is 5% calculate         (i) load torque in Kg-m  (ii) speed at maximum torque  (iii)  Rotor emf at maximum torque.

4.  a)	Distinguish between 3- slip ring and squirrel cage induction motors in all aspects?  Why Induction motor operates always at lagging p.f.?

b)	Calculate Torque exerted by an 8 pole, 50 Hz, 3- IT operating with a 4% slip which develops a maximum torque of 150 kg-mt at a speed of 600 rpm.  The resistance per phase of the rotor is 0.5.

Contd……2.

Code No: 220205				:: 2 ::				Set No:1

5.  a)	Derive the condition for maximum torque and maximum power output for a three phase Induction motors?

b)	The Power Input to Rotor of a 440 V, 50 Hz, 6 pole 3- Induction motor is       100 kw.  The Rotor electromotive force is observed to make 120 cycles per minute.  Calculate (i) slip  (ii) Rotor speed  (iii) Mechanical Power developed  (iv) speed of stator field with respect to rotor.

6.  a)	Derive the exact equivalent circuit of 3 Induction motor.  What is difference between exact and approximate equivalents?  Discuss from approximate equivalent,  find Rotor output and output torque?

b) 	A 3 Y connected 400V, 50 Hz, 4-pole Induction Motor has R1=0.15, X1 = 0.45, R12 = 0.12 l, X21 = 0.45, Xm = 28.5.  Compute stator current and Power factor?  When motor is operated at rated voltage and frequency with S = 0.04?

7.  a)	Explain the load sharing phenomena between two transformers operating in parallel?

b)	Two transformer A and B are connected in Parallel to a load of (2+j1.5) ohms.  The Impedance in secondary are ZA = (0.15 + j0.5) ohms ZB = (0.1 + j0.6) ohms.  Their no load terminal voltages are EA = 207 o V, EB = 205o V.  Find power output and Power factor of each transformer.

8.	Write short notes on the following:

- Torque-slip characteristics of an Induction Motors.

- No load phasor diagram of an ideal transformer

- Buchholz relay

@@@@@

Code No:220205

Set No.

2

# II B.Tech. II Semester Supplementary Examinations January,  2003

# ELECTRO MECHANICS - II

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	Draw phasor diagram of loaded 1- Transformer and derive from this equivalent circuit of transformer?

b)	A transformer has a Primary winding of 800 turns and a secondary winding of 200 turns.  When load current as secondary is 80 A at 0.8 p.f. lagging.  The primary current is 25 A at 0.707 p. f. lagging. Determine graphically or otherwise the no load current of transformer and its phase with respect to voltage.

2.  a)	Name the suitable test for separation of losses in two identical transformers and explain it?

- The maximum efficiency of a 100 KVA, 1- Transformer is 98% and occurs at 80% of full load at 0.8 p.f.  If the leakage impedance of transformer is 5%.  find the voltage regulation at rated load of 0.8 p.f. lagging?

3.  a)	Explain the working principle and construction of an auto transformer ?  State its merits and demerits over the two winding transformer.

b)	A 3- Transformer has its primary connected in  and secondary in Y.  It has equivalent resistance of 1% and equivalent reactance of 6%.  The primary applied voltage is 6600V.  What must be ratio of transformation in order that it will deliver 4800V at full load current and 0.8 p.f. (lag).

4.  a)	Why double squirrel cage induction motor has high starting torque and high operating efficiency.  Explain it with necessary diagrams?

b)	In a double cage induction motor, If outer cage has an Impedance at stand still of (2+j1.2) ohm.  Determine the slip at which two cages develop equal torques if inner cage has an impedance of (0.5 + j3.5) ohm at stand still?

5.	Draw a circle diagram for a 5.6 kW, 400 kW, 3, 4 pole, 50 Hz slip ring Induction from following data

No load readings :  400 V, 6A, Cos0 = 0.087

Short circuit Test : 100V, 12 A, 720 W

Ratio of primary to secondary turns in 2.62 stator resistance/ph 0.67 ohm and of rotor is 0.185 ohms calculate full load current, full load slip, full load power factor, maximum power and efficiency?

Contd……2.

Code No: 220205				:: 2 ::				Set No:2

6.  a)	Explain the no load and blocked rotor test of an induction motor.

b)	An induction motor has an efficiency of 0.9 when delivering an output of 37 kW.  At this load, the stator Cu loss and Rotor Cu loss each equals to stator iron loss.  The mechanical losses are ⅓ rd of no load loss.  Calculate the slip?

7.  a)	Discuss the Procedure for conducting OC and SC tests of a single phase transformer?

b)	Obtain the equivalent circuit of a 200/400 V, 50 Hz, 1- Transformer from following data.

O.C. Test: 200V, 0.7A, 70W – on L.V. side

S.C. Test : 15V, 10A, 85W – on  H.V. side

Calculate secondary voltage when delivering 5 kW at 0.8 p.f. lagging, primary voltage being 200 V.

8.	Write short notes on the following.

- Advantages and disadvantages of poly phase induction motor.

- Y -  starter of an Induction motor.

- Scott connection.

@@@@@

## Code No:220205

II B.Tech. II Semester Supplementary Examinations January,  2003

Set No.

3

# ELECTRO MECHANICS - II

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.   a)	State various losses that takes place in transformer. On what factors do they depend?  Explain the steps taken to minimize these losses?

b)	A 200 KVA, 1-, 50 Hz, 2300/230V Transformer has a core loss of 300W.  At full load, I2 R loss is 800 W and leakage reactance is 0.032 P.U. calculate the total loss and voltage regulation for

i)  1.2 P.U. load at p.f. 0.8 lagging

ii) 0.70 P.U. load at p.f. 0.6 leading

2.  a)	Suggest a suitable test to predetermine the regulation of a transformer and explain it?

b)	A transformer has a primary winding of 800 turns and a secondary winding of 200 turns.  When load current on secondary winding of 200 turns.  When load current on secondary is 80 A at 0.8 p.f. lagging.  The primary current is 25A at 0.707 p.f. lagging.  Determine graphically (or) otherwise the no load current of transformer and its phase with respect to voltage.

3.  a)	State the similarities between transformer and induction motor and explain why induction machine is called a generalized transformer?

b)	A 4-pole, 50 Hz, 3- Induction motor develops a maximum torque of 162.8N– m at 1365 rpm.  The resistance of Y connected rotor is 0.2  / ph.  Calculate value of resistance that must be inserted in series with each rotor phase to produce a starting torque equal to half the maximum torque?

4.  a)	Mention different methods for speed control of induction motors.  
Explain any two with suitable diagrams.

b)	Two 50 Hz, 3- Induction motors having six and four poles respectively are cumulatively cascaded, the 6-pole motor being connected to main supply.  Determine frequencies of rotor currents and slip referred to each stator field if set has a slip of 2%.

5.  a)	Why are tap changing transformers required?  Explain the operation of no load tap changing transformers?

b)	A 3- transformer has its primary connected in  and its secondary connected in Y.  It has equivalent Resistance and reactance of 1% and 6% respectively.  Primary applied voltage is 6600V.  What must be ratio of transformation in order that it will deliver 4800V at full load current and 0.8 p.f. lag?

Contd……2.

Code No: 220205				:: 2 ::				Set No:3

6.  a)	Explain the factors that govern the operating characteristics of poly phase induction motors?  Why rotor speed falls as load torque in increased?

b)	A 10 kW, 6 pole, 50 Hz 3 induction motor has linear torque slip characteristics between zero torque and maximum torque.  The slip at which maximum torque of 520 Nm occurs is 0.2.  For mechanical losses of 600 W.  Find the speed at which motor would run when delivering rated shaft power?

7.  a)	Explain the operation of Double cage induction motor?

b)	At stand still, the equivalent impedance of inner and outer cages of a double cage rotor are (0.4 + j0.2) ohms and (2+j0.4) l respectively.  Calculate ratio of torques produced by two cages (i) at stand still (ii) at 5% slip.

8.  	Write short notes on the following

- Draw the equivalent circuit of Induction motor and explain it.

- Distribution transformers are designed for low iron losses, why?

- Plugging of an induction motors.

@@@@@

## Code No:220205

Set No.

4

# II B.Tech. II Semester Supplementary Examinations January,  2003

# ELECTRO MECHANICS - II

(Electrical and Electronics Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.  a)	Give the concept of single phase Ideal transformer.  Describe its performance with half of emf equations. Its phasor diagram at no load and under load.  Discuss how core flux in an ideal transformer is

independent of load current

b)	A 1- ideal transformer has 400 primary turns and 600 secondary turns.  The primary is connected to 800 V source.  The full secondary has a resistive load of 24 kW.  The secondary is tapped at 500 turns across which there is a pure inductive load of 20 KVA.  Find primary current, its power factor and power input to primary?

2.  a)	Derive an expression for computing per unit voltage regulation of a transformer both for lagging and leading power factors?

b)	The maximum efficiency of a 500 KVA, 3300/500V, 50 Hz 1- Transformer is 97% and occurs at ¾ full load, unity power factor.  If the impedance is 10%.  Calculate regulation at full load, power factor 0.8 lagging.

3.  a)	Explain the terms air gap power Pg, internal mechanical power developed and shaft power Psh.  How are these terms related to each other.  Hence show that    Pg: Rotor ohmic loss: Pm = 1:S: (1-S).

b)	A 18.65 kW, 4-pole 50 Hz, 3 I.M. has friction and windage losses of 2.5% of out put.  The full load slip is 4% compute for full load (i) rotor cu losses (ii) Rotor input  (iii) shaft torque (iv) gross electro magnetic torque.

4. a)	Derive the condition for maximum torque of a 3 induction motor under running conditions?

b)	A 400 V, 4-pole, 3, 50Hz, Induction motor has rotor resistance per phase of 0.01 and 0.1  Determine (i) Max torque in N-m and corresponding slip (ii) Full load slip and power o/p in watts, if maximum torque is twice full load torque.  The ratio of stator to rotor turns is 4.

5.	A 415V, 29.84 kW, 50Hz, delta connected motor gave following test data.

No load test		: 	415V, 21A, 1250 W

Locked rotor test	:  	100V, 45A, 2730 W

Construct circle diagram and determine (i) line current and power factor for rated out put (ii)  maximum torque (assume stator and Rotor cu losses equal at stand still)

Contd……2.

Code No: 220205				:: 2 ::				Set No:4

6.  a)	Explain torque slip characteristics of an induction motor working at rated voltage and frequency with a neat sketch.

b)	A 6-pole, 50Hz, 3- induction motor running as full load with 4% slip develops a torque of 149.3 N-m at its pulley rim.  The friction and windage losses are 200 W and stator Cu and iron losses are equal 1620W Calculate  (i) output power

(ii) Rotor cu losses  (iii) efficiency at full load.

7.  a)	State (i) essential (ii) desirable conditions to be satisfied so that two 3- transformers may operate successfully in parallel.  Explain.

b)	A 2000 KVA Transformer A is connected in parallel with a 4000 KVA transformer B to supply a 3- load of 5000 KVA at 0.8 p.f. lagging.  Determine KVA supplied by each transformer assuming equal on load voltages.  The percent voltage drops in windings at their rated loads are as follows:

Transformer A Resistance 2% Reactance 8%

Transformer B Resistance 1.6% Reactance 3%.

8.	Write short notes on the following:

- Crawling and cogging of induction motors

- Auto transformer

- Differences between current transformer and potential transformer.

@@@@@